PEP-01/04/08

PARALLER/PRINTER PORT E/EEPROM PROGRAMMER

PEP-01/04 ARE THE FIRST THREE OF A COMPACT AND VERSATILE E/EEPROM

PROGRAMMER SERIES. LINKING AND COMMUNICATING WITH PC'S VIA

PRINTER PORTS, THEY COULD PROGRAM IC'S INCLUDING EPROM, EEPROM

AND FLASH EEPROM. EASY CARRYING, INSTALLING AND OPERATING

MAKES IT SUITABLE FOR R&D AND MASS-PRODUCTION PROGRAMMING.

MAIN FUNCTION:

1. USING PARALLEL/PRINTER PORT TO COMMUNICATE WITH IBM PC

INCLUDING DESTOPS AND NOTEBOOKS.

2. SELECTING IC VENDOR AND TYPE TO SETUP PROGRAMMING ALGORITHM

SO HIGH PASS RATE AND RELIABILITY ARE EXPECTED.

3. SUPPORTING COMPLETE FUNCTION INCLUDING BLANK CHECK/READ/

VERIFY/PROGRAM AND AUTO BLANK CHECK+PROGRAM+VERIFY.

PROGRAMABLE IC INCLUDING:

1. EPROM  : 27C16 - 27C080, 27C513, 27C011, 87C257.........etc

2. EEPROM : 28C16 - 28C040............etc

3. FLASH EEPROM : 28F256 - 28F020, 29EE010 NOT ABOVE 4M BITS

SPECIFICATION:

POWER CONSUMPTION:  12VDC  500MA FOR PEP-01/04

12VDC  800MA FOR PEP-08

POWER SOURCE: DC-12 CARD CONNECTED TO PC ISA SLOT OR DC 12V ADAPTER

MODULE SIZE: 185mm (L) X 105mm (W) X 25mm (H)

WEIGHT: ROUGHLY 1.2KG

ORDERING INFORMATION:

1. PROGRAMMER

2. 1 DC-12 CARD WITH POWER CABLE

3. 1 D-25 CABLE

4. INSTALLATION DISC

5. USER'S MANUAL

<<<<< REMARK >>>>>

When user selects EPROM size larger than software allocated memory

buffer, software opens a file (EDBIN.MEM) with same EPROM size

automatically in the "Work path" to use file buffer when software

access EPROM's data.

Example:

Assumption: software allocated memory buffer space is 256K bytes

Work path: D:\PEP01

When programming 27C040 (512 Kbytes), allocated memory buffer becomes

insufficient. Software automatically opens a 512 Kbytes file

(C:\PEP01\EDBIN.MEM). If file opening is successful, "EDBIN.MEM" will

appear after "Buffer address :" inside the status field.

If it is not work that the user may change  directory of work path on config

item. that will be no problem.