| 3 | Design Examples |
| --- | --- |

Contents

µMultiplexer	50

Comparator	51

Left/Right Shifter	52

Barrel Shifter	53

Simple 3-Bit Counter	54

Decoder	56

Up-Down Counter and Up-Counter with Parallel Load	57

Data Acquisition System	58

Moore State Machine	60

The following design examples show how several commonly used design elements are implemented using the MACHXL language.  For your convenience, the MACHXL design files for these design examples are installed on your hard disk when you install the MACHXL software, in the \MACHXL\EXAMPLES directory.

# MultiplexerXE "design examples:multiplexer"§

This 8:1 multiplexer uses three select bits to route one of eight busses.  Unlike the multiplexer presented in the "Data Acquisition System" section of this chapter, this multiplexer has been implemented with Boolean equations rather than CASE statements.

To view the MACHXL implementation of this design, open the design file MUX.PDS.

µ §

## Features of Interest

r	Uses vectors

r	Implements a multiplexer using Boolean logic

# ComparatorXE "design examples:comparator"§

A fast comparator that compares two ten-bit busses.

The logic diagram for this design appears below.  To view the MACHXL implementation of this design, open the design file COMPARA.PDS.

µ §

## Features of Interest

r	Uses XOR statements in equations

r	Uses FOR..TO..DO in simulation

# Left/Right ShifterXE "design examples:left/right shifter"§

Shift registers are widely used in communications and computer systems.  Shift registers can serialize data, which allows designers to minimize the number of output pins and bus bits.  This particular shift register performs three operations:

r	Loads a new eight-bit byte of data (when the control line LOAD is high and the device is clocked)

r	Shifts data left (when the control line LOAD is low, the control line LR is high, and the device is clocked)

r	Shifts data right (when the control line LOAD is low, the control line LR is low, and the device is clocked)

µ §

To view the MACHXL implementation of this design, open the design file LRSHIFT.PDS.

## Features of Interest

r	Uses CASE statements

r	Uses vectors on both sides of a Boolean equation (that is, transfers data from one bank of signals to another ng a single statement)

r	Uses vectors of signals in both the Equations and Simulation segments, including CHECK statements using vectors

r	Uses FOR..TO..DO statements for simulation

# Barrel ShifterXE "design examples:barrel shifter"§

This design is discussed at length in Chapter 2.

µ §

To view the MACHXL implementation of this design, open the design file BARREL.PDS.

## Features of Interest

r	All behavior specified using sum-of-products Boolean equations

r	Uses vectors of signals in both the Equations and Simulation segments

# Simple 3-Bit CounterXE "design examples:counter, 3-bit"§

A counter that increments on each clock cycle when the control line COUNT is high.  This counter is similar to the counter modules in the two designs that follow, except that the logic for loading an initial count into the counter is omitted for the sake of simplicity.

µ §

| ã | Note: T flip-flops are recommended for most counter designs since these require fewer product-terms than do D flip-flops. Parallel-loaded counters using D flip-flops require less logic for loading, but these still need more product-terms than an equivalent T flip-flop counter. For large counter designs the user should design one lookahead carry bit for every 15 counter bits. The carry bit will be the only input to the next PAL block from the present PAL block, and input array resources will not be wasted. Ripple counters may be a good choice for very large counter designs. However, take into consideration the following caveats: Ripple counters are slow and may require extra logic to avoid glitches. Also, functional simulation of these counters will not show timing delays and glitches. |
| --- | --- |
| ã | Note: If you set the Optimize registers for D/T-type field of the Logic Synthesis Options form (File:Set up:Compilation options) to "Best for device," the Fitter will automatically change D-type flip-flops to T-type or vice versa to produce the most efficient implementation for the target MACH device. |

To view the MACHXL implementation of this design using D-type flip-flops, open the design file 3COUNT_D.PDS.  To view the MACHXL implementation of this design using T-type flip-flops, open the design file 3COUNT_T.PDS.

## Features of Interest

r	For the purpose of comparison, uses individual pin names in places where other examples in this chapter typically use vector notation

# DecoderXE "design examples:decoder"§

Decoders, such as the one shown here, can be used for memory addressing, where they select one of several memory devices.  They can also be used to demultiplex data and reroute clocks.

µ §

To view the MACHXL implementation of this design, open the design file DECODE.PDS.

## Features of Interest

r	In simulation, sets a vector of pins to a hexadecimal value

# Up-Down Counter and Up-Counter with Parallel LoadXE "design examples:counter,up-down"§XE "design examples:up with parallel load"§

This design is presented in four design files, each of which implements the design in a different way:

r	The up-down counter with parallel load is implemented using Boolean equations in the design file UDCNT.PDS.

r	The up-counter with parallel load is implemented using IF..THEN..ELSE constructs in the design file CNTIF.PDS.

r	The up-counter with parallel load is implemented using CASE constructs in the design file CNTCASE.PDS.

r	The up-counter with parallel load is implemented using Boolean equations in the design file COUNTER.PDS.

In addition to incrementing the current count by one when counting is enabled, the counter can be loaded with a new current count from a bank of input pins or reset to zero at any time.

µ §

## Features of Interest

r	Shows several ways to implement equivalent logic

# Data Acquisition SystemXE "design examples:data acquisition system"§

This design (shown on the next page) contains the following subsystems:

r	A demultiplexer

r	A 4-to-1 multiplexer

r	A synchronous counter

r	A bidirectional multiplexer.

These subsystems are used in many data acquisition applications as well as in computer applications.  In this design:

r	Inputs are in the form of multiplexed data, which are demultiplexed inside.

r	The demultiplexer consists of two sets of registers controlled by different clock-enable logic.  Therefore, each bank of registers will register a different set of data.  Such demultiplexing techniques are useful to save device pins.

r	The adder and the multiplexer provide several addressing choices for this real-time CPU-controlled system.

r	The counter loads data from external RAM, increments it, and writes it back to the RAM.

r	The Bidirectional multiplexer provides multiplexing and parallel communication between external buses and memory devices.

The following block diagram shows the overall design in pictorial form.

The design file is DATA_AQ.PDS.

## Features of Interest

r	A realistic example of a moderately complex design that combines several subsystems on the same device

µ §

# Moore State MachineXE "design examples:Moore state machine"§

State machines are used in many different applications.  There are many state machine models available, the two most common (and the two supported by MACHXL software) being the Moore and Mealy models.

Moore machines have a single set of outputs for each state; that is, outputs are determined solely by the machine's current state.  Mealy machines can have different outputs in the same state.  Mealy  machine outputs are determined by evaluating inputs as well as by the machine's current state.

The design presented here is not meant to perform any particular function, but rather to illustrate Moore state machine design principles.  To increase its usefulness as a learning tool, this design is implemented in three different ways:

r	Using CASE statements as described in Chapter 6, "Equations Segment In Depth" (MOORE_C.PDS)

r	Using the MACHXL state machine language as described in Appendix A, "State Segment In Depth" (MOORE_S.PDS)

r	Using Boolean equations (MOORE_B.PDS)

You can implement only one state machine in the STATE segment of any design file.  If your design contains multiple state machines, a CASE implementation is usually the best choice.

µ §

For other examples of Moore and Mealy machines, see Appendix A, "State Segment In Depth."

## Features of Interest

r	Identical behavior generated using three different models

r	Full initialization and illegal state recovery logic in each design

r	State syntax example (MOORE_S.PDS) shows how to check state namesXE "state names:checking in simulation"§ in the Simulation segment

To view these or any other designs in Boolean form: compile the design, then choose Run:Other operations:Disassemble from:Intermediate file from the MACHXL menu.  The resulting file shows the Boolean equivalents of all high-level constructs used in the design, including STATE, CASE, and IF..THEN..ELSE constructs.