| B | Glossary |
| --- | --- |

ACTIVE EDGEXE "ACTIVE EDGE"§	A low-to-high or high-to-low signal transition that initiates an action.

ACTIVE HIGHXE "ACTIVE HIGH"§	One of the two possible polarity attributes for input, output, and I/O pins.  An active-high output is high when the corresponding Boolean equation is true.  An output pin is active high when the polarity of the pin's logic equation agrees with the polarity of the pin's PIN declaration statement.

ACTIVE LOWXE "ACTIVE LOW"§	An active-low output is high when the corresponding Boolean equation is false.  An output pin is active low when the polarity of the pin's logic equation is opposite to the polarity of the pin's PIN declaration statement.

ASSEMBLYXE "ASSEMBLY"§	The procedure of creating a JEDEC file to implement a design (specified in a PDS file) on the target device.

ASYNC_LISTXE "MACH_SEG_x"§	A keyword used in a GROUP statement to list all signals that are to be configured as asynchronous macrocells.

ASYNCHRONOUS REGISTERXE "ASYNCHRONOUS REGISTER"§	On devices that support the asynchronous mode, each macrocell register can be configured by the Fitter as synchronous or asynchronous, depending on several factors.  The principal factor is the register's clock input: if a) the clock definition includes more than one literal or b) specifies any input other than a global clock pin, the register is always asynchronous.  A register that uses a single-literal clock definition may nevertheless be implemented as asynchronous by the Fitter, if a) the clock pin specified in the design is not a global clock pin, or b) the clock pin is floated but the number of single-literal clocks in the design exceeds the number of available global clock pins.  (Refer to SYNCHRONOUS REGISTER for comparison.)

AUXILIARY SIMULATION FILEXE "AUXILIARY SIMULATION FILE"§	A file, separate from the design file, that contains simulation commands used to simulate the design.  This file must have the same name as the design file, with the extension .SIM.  (Refer to SIMULATION SEGMENT for comparison.)

BACK ANNOTATIONXE "BACK ANNOTATION"§	The MACHXL software allows designers to assign logical names and behavior to signals without assigning those signals to specific locations in the target device.  Back annotation is a software function that copies to the appropriate PIN or NODE statement in the original design file the actual pin or node location assigned by the Fitter to the corresponding signal.

BANKXE "BANK"§	A collection of pins or nodes within a PAL block.

BLOCKXE "BLOCK"§	Refer to PAL BLOCK.

BLOCK CLOCK MECHANISMXE "BLOCK CLOCK MECHANISM"§	Each MACH 3xx/4xx PAL block has its own clock generator that can provide up to four different clock signals to the block.  The process whereby up to four global clock signals are made available to all macrocells in the block is commonly referred to througout this user's guide as "the block clock mechanism.

BLOCK FANINXE "BLOCK FANIN"§	The collection of pins or nodes that is routed to a PAL block through the central switch matrix.

BLOCK FANOUTXE "BLOCK FANOUT"§	The collection of blocks to which a pin or node is routed through the central switch matrix.

BLOCK PARTITIONINGXE "BLOCK PARTITIONING"§	Refer to PARTITIONING.

BLOCK-RESTRICTEDXE "BLOCK-RESTRICTED"§	A pin or node is block-restricted if it appears in the list of a GROUP  MACH_SEG_x statement.  A block-restricted pin or node can be placed only in the user-specified PAL block.

BOOLEAN POST-PROCESSORXE "BOOLEAN POST-PROCESSOR"§	A MACHXL program that runs after the Parser and again after the STATE Syntax Expander (if needed).  The Boolean Post-Processor substitutes logical names for vectors and groups, converts CASE and IF-THEN-ELSE statements into Boolean equations, and merges multiple equations written for the same signal.

BURIED MACROCELLXE "BURIED MACROCELL"§	A macrocell the output of which is not routed to an I/O pin.  Buried macrocells are useful for implementing internal logic, and are commonly used to store state bits.

CASEXE "CASE"§	A construct used to express logical operations in natural language, as an alternative to writing out the equivalent Boolean equations.  The preferred way to express state machine designs.

CENTRAL SWITCH MATRIXXE "CENTRAL SWITCH MATRIX"§	PAL blocks in a MACH device communicate with each other through the central switch matrix.  Feedback signals from a PAL block that only go to the same PAL block must still pass through the central switch matrix.  The inputs to the central switch matrix are called array or switch-matrix-to-block inputs.

CHECK COMMANDXE "CHECK COMMAND"§	A simulation command that compares the pin's simulated value against a user-defined expected value.

CHECKQ COMMANDXE "CHECKQ COMMAND"§	A simulation command that compares the simulated value of a register's Q output against a user-defined expected value.

CLKFXE "CLKF"§	Defines the rising-edge clock used to synchronize a state machine defined in the STATE segment.

CLOCKF COMMANDXE "CLOCKF COMMAND"§	A simulation command that generates a pulse on a global clock pin during simulation.

CLUSTERXE "CLUSTER"§	The group of product terms that is physically aligned with a MACH macrocell.  The product term cluster can be steered to an adjacent macrocell to increase that macrocell's capacity to implement large equations.  In MACH 3xx/4xx devices, the XOR product term can be separated from the cluster and used by the original macrocell to implement a single product term equation, while the remaining product terms in the cluster are steered to an adjacent macrocell.  In addition, in asynchrnous mode, two product terms are used for Set and Reset and are consequently unavailable for logic equations.

COMBINATORIAL EQUATIONSXE "COMBINATORIAL EQUATIONS"§	Equations that combine signals for immediate output instead of storing the resulting value in a register or latch.

CONDITIONXE "CONDITION"§	a) The set of signals that is evaluated to determine a state machine's next state and/or its outputs.  b) A STATE syntax keyword that precedes the equations used to define conditions.  c) Any set of signals that is evaluated before performing some action.

CONDITION EQUATIONSXE "CONDITION EQUATIONS"§	The equations that define conditions in a design that uses STATE syntax.

CONDITIONAL BRANCHXE "CONDITIONAL BRANCH"§	A state branch that can only occur in the presence of certain input conditions.

CONTROLLABILITYXE "CONTROLLABILITY"§	The degree to which signals in a part of a circuit can be made to take on specific values through manipulation of primary inputs; used in testability analysis.

CRITICAL PATH EVALUATIONXE "CRITICAL PATH EVALUATION"§	The identification and analysis of signal paths, the delays of which could limit the speed of the circuit.

CURRENT DESIGN FILEXE "CURRENT DESIGN FILE"§	The design file that you specified to work on, using the Begin new design or Retrieve existing design commands.

DECLARATION SEGMENTXE "DECLARATION SEGMENT"§	The portion of the design file in which the designer provides design identification, specifies the target device, declares pins and nodes, defines string substitutions, and defines groups of signals.

DEDICATED CLOCK PINXE "DEDICATED CLOCK PIN"§	Refer to GLOBAL CLOCK PIN.

DEDICATED INPUT PINXE "DEDICATED INPUT PIN"§	A pin that can be used only as a signal input to the device's sum-of-products logic array(s).

DEFAULT BRANCHXE "DEFAULT BRANCH"§	The state branch that occurs when the inputs do not match any of the transition conditions specified for the present state.

DEFAULT VALUEXE "DEFAULT VALUE"§	The value used unless you specify a different one.

DESIGN FILEXE "DESIGN FILE"§	A text file that contains the designer's instructions for producing specific behavior in the target device.  The MACHXL software processes the design file to create a JEDEC file that is used to program the target device.  The design file's name takes the general form Design_name.PDS.

DISASSEMBLEXE "DISASSEMBLE"§	The process of producing Boolean equations equivalent to the original design file from a) the intermediate file or b) the JEDEC file.

DO LOOPXE "DO LOOP"§	A set of instructions that is performed repeatedly until some condition occurs.  (See also IF-THEN-ELSE and WHILE-DO.)

EDITORXE "EDITOR"§	The program used to create and modify design files and other text files.

EQUATIONS SEGMENTXE "EQUATIONS SEGMENT"§	The portion of the design file in which the designer defines the behavior of pins and nodes declared in the DECLARATION segment using Boolean equations and/or CASE and IF-THEN-ELSE statements.

EXPANDXE "EXPAND"§	Refer to STATE SYNTAX EXPANDER.

FIELDSXE "FIELDS"§	Areas in the MACHXL menu forms where you enter data.

FITTERXE "FITTER"§	The compilation module that automatically manages the internal arrangement of resources.  The Fitter software automatically distributes product terms to the macrocells and adjusts the distribution as required by the design.

FLIP-FLOPXE "FLIP-FLOP"§	A clocked memory device that stores a binary value.  The flip-flop's stored value is a function of the input value(s) present when the clock pulse occurs.

FLOATXE "FLOAT"§	verb  To declare a pin or node without specifying a location, by typing a question mark (?) instead of entering a pin or node number in the PIN or NODE statement.  Floating pins and nodes allows the Fitter maximum flexibility to find a successful fit, and is the best fitting strategy in most cases.

FOR-TO-DO LOOP COMMANDXE "FOR-TO-DO LOOP COMMAND"§	A simulation construct that repeats a set of commands a fixed number of times.

FUNCTIONAL EQUATIONXE "FUNCTIONAL EQUATION"§	A special equation used in the EQUATIONS segment to define clock, set, reset, or output-enable behavior.

GATE SPLITTINGXE "GATE SPLITTING"§	The process of dividing equations that are too large to fit in the number of product terms available (through product-term steering) to a single macrocell. Also, the process of dividing equations that exceed the user-defined threshold value into subsidiary equations as specified by the maximum number of allowable product terms per equation.  These subsidiary equations use smaller groups of product terms, the results of which are routed, as feedback signals, to a single macrocell.  Gate-splitting buys equation size at the expense of speed, since each pass through the sum-of-products array entails an additional propagation delay (or clock cycle).

GLOBAL CLOCK PINXE "GLOBAL CLOCK PIN"§	A pin that can be used to clock synchronous registers.  Some MACH devices contain multiple global pins.  In some devices, inputs to the global pins can be routed, in the same design, to the clock inputs of synchronous registers as well as to the sum-or-products array(s).  Refer to the device data sheet for details.

GLOBAL NODEXE "GLOBAL NODE"§	A logical node, which may does not correspond to an architectural feature, for which functional equations can be written.  In the MACH 3xx/4xx family of devices, the global node can be used to control the set and reset behavior of synchronous registers.  If no functional equations are written for the global node, the behavior of the synchronous registers is controlled by either a) the functional equations written for one or more of the synchronous registers in each PAL block or b) the default behavior of the device.

GNDXE "GND"§	a) The reserved word used in MACHXL designs to denote an unconditionally false condition.  b) A permissible logical name for the Ground pin(s) of a MACH device.

GROUPXE "GROUP"§	A logical name assigned to any number of pins or nodes.  Any time the group name appears on the left side of an equation, the MACHXL software performs the operation described in the equation on all pins or nodes in the group.  (Refer also to MACH_SEG_x.)

GRP FILE	The partitioner creates the file Design.GRP, which contains as many GROUP MACH_SEG_x statements as are required to define the partitioning used to fit the design Design.PDS.

HISTORY FILEXE "HISTORY FILE"§	An output file generated by the Simulator that shows the values of every declared pin and node at each stage of the user-defined simulation sequence.  The history file is stored under the name Design_name.HST.

HST FILEXE "HST FILE"§	Refer to HISTORY FILE.

IF-THEN-ELSE COMMANDXE "IF-THEN-ELSE COMMAND"§	A conditional branching construct used in the SIMULATION segment to control the simulation sequence.

IF-THEN-ELSE STATEMENTXE "IF-THEN-ELSE STATEMENT"§	A construct used to express logical operations in natural language, as an alternative to writing out the equivalent Boolean equations.

INITIALIZEXE "INITIALIZE"§	The process of establishing an initial condition or starting state.  For example, setting logic elements in a digital circuit, or the contents of a storage location, to known states so that subsequent application of digital test patterns drive the logic elements to another known state.  Initialization sets counters, switches, and addresses to zero or other starting values at the beginning of, or at prescribed points in, a computer or state machine routine.

INPUT-PAIREDXE "INPUT-PAIRED"§	A pin and node that are associated to produce a dedicated, registered input.  Input-paired pins and nodes are always defined by using the PAIR keyword in the PIN statement of the paired pin.  (Refer to OUTPUT-PAIRED for comparison.)

INPUT SWITCH MATRIXXE "INPUT SWITCH MATRIX"§	MACH 3xx/4xx devices have input switch matrices through which I/O pins, logic macrocells, and input macrocells are routed to different feedback paths.  These feedback paths are equivalent to inputs to the central switch matrix.  Each I/O pin of a MACH 3xx/4xx device has one input switch matrix, consisting of three two-to-one muxes, where a) the pin is an input to each of the muxes and b) two logic macrocells and an input macrocell comprise the remainder of the mux inputs.

INTERMEDIATE FILEXE "INTERMEDIATE FILE"§	A file created or modified by programs invoked by the Compile command, prior to assembling the JEDEC file.

INVERTERXE "INVERTER"§	An architectural feature that reverses the logical state of a signal.

JDC FILEXE "JDC FILE"§	A file that contains JEDEC fuse data and JEDEC test vectors.  The JDC file is generated by the Simulator.

JDM FILEXE "JDM FILE"§	A file that contains JEDEC fuse data and the recalculated JEDEC checksum.  The JDM file is generated by the Recalculate JEDEC Checksum command.

JED FILEXE "JED FILE"§	A file that contains JEDEC fuse data.  The JED file is generated by the Fitter.

JEDECXE "JEDEC"§	An acronym for Joint Electrical Device Engineering Council.

JEDEC FILEXE "JEDEC FILE"§	Contains the fuse-programming information used by the device programmer to program a device.

KEYWORDXE "KEYWORD"§	An instruction that tells the MACHXL software how to interpret the information that follows the keyword.

LATCHED EQUATIONXE "LATCHED EQUATION"§	A logic equation that defines the behavior of a pin or node declared as LATCHED in the corresponding PIN or NODE statement.

LOCAL DEFAULTXE "LOCAL DEFAULT"§	Used in the STATE segment only.  A default branch that applies only to the state in which it is defined.

LOG FILEXE "LOG FILE"§	A file containing all processes and messages generated during a software processing session.

LOGIC EQUATIONXE "LOGIC EQUATION"§	Defines the output of one sum-of-products as a function of external inputs and/or internal feedback signals.  A logic equation is always associated with a pin or a node.  The actual behavior of the pin or node is a function of a) the output of the sum-of-products and b) the functional equations expressed or implied for the same pin or node.

LOGIC MINIMIZERXE "LOGIC MINIMIZER"§	A MACHXL program that performs logic reduction on the intermediate file.

LOW_POWER_LISTXE "MACH_SEG_x"§	A keyword used in a GROUP statement to list all I/Os used in the design that are to be configured in the power-down mode.

MACH_SEG_xXE "MACH_SEG_x"§	A keyword used in a GROUP statement to partition all signals specified in the GROUP statement into a single PAL block.  The x represents the PAL block into which the signals will be partitioned (replace x with A, B, C, ... , etc., the specify the desired PAL block).

MACHXLXE "MACHXL"§	A menu-driven program that controls design entry and compilation for MACH3xx/4xx devices.  The MACHXL software provides backward compatibility with designs developed using PALASM 4 software.

MACROCELLXE "MACROCELL"§	An architectural feature of MACH devices that controls signal routing through or around its internal flip-flop, inverters, and feedback lines using fuse-programmable muxes (refer to the device data sheet for information on the macrocells of specific devices).

MEALY STATE MACHINEXE "MEALY STATE MACHINE"§	A state machine in which the outputs are dependent on a) the present state and b) other inputs or feedback signals.

MOORE STATE MACHINEXE "MOORE STATE MACHINE"§	A state machine in which the outputs are dependent on the present state only.

MUXXE "MUX"§	Abbreviation for "multiplexer."  In MACH devices, the mux selects one signal from a set of two or more available signals.  The selected signal continues through the mux, while the deselected signals terminate at the mux.

NCLKFXE "NCLKF"§	Defines the falling-edge clock used to synchronize a state machine defined in the STATE segment.

NEXT STATEXE "NEXT STATE"§	The state to which a state machine will branch on the next clock pulse, depending on the inputs present when the clock pulse occurs.

NOMINAL DELAYXE "NOMINAL DELAY"§	The mean time signals take to propagate through a logic element or wire.  The effect of an input change to an element on the output does not occur until after the nominal delay.

OPTION LISTXE "OPTION LIST"§	A list that appears when you press the F2 key in an option field of a software form.  You select an option from the list or change the specification in the selected field.

OUTPUT ENABLE EQUATIONXE "OUTPUT ENABLE EQUATION"§	A functional equation (in the form Pin_name.TRST) that enables output from the pin when true and disables output from the pin when false.

OUTPUT-PAIREDXE "OUTPUT-PAIRED"§	A pin and node that are associated to produce registered output at an I/O pin.  Output-paired pins and nodes are defined in one of two ways: a) explicitly, by using the PAIR keyword in the NODE statement of the paired node, or b) implicitly, by declaring a pin as REGISTERED and allowing the MACHXL software to pair it with a macrocell register.  After disassembly, all output pairs become explicit regardless of how they were implemented in the original design file.  (Refer to INPUT-PAIRED for comparison.)

OUTPUT SWITCH MATRIXXE "OUTPUT SWITCH MATRIX"§	MACH4xx devices have an output switch matrix through which logic macrocells are routed to I/O cells.  The output switch matrix can route a logic macrocell to at most one pin, but can choose from among several pins.

PAIRXE "PAIR"§	Refer to INPUT-PAIRED and OUTPUT-PAIRED.

PAL BLOCKXE "PAL BLOCK"§	A collection of PAL-like structures that function as independent PAL devices on a single chip.  The PAL blocks communicate with each other through the central switch matrix.

PALASM 4XE "PALASM 4"§	A menu-driven program that controls design entry and compilation for PAL and MACH 1xx/2xx devices.  The MACHXL software provides backward compatibility with designs developed using PALASM 4 software.

PARSERXE "PARSER"§	The MACHXL compilation program that checks the syntax of the design file and creates the intermediate file that is processed by subsequent compilation programs.

PARTITIONXE "PARTITION"§	a) An individual PAL block, or the set of signals placed therein.  b) verb  To place signals into specific PAL blocks.  (This is usually done automatically by the Fitter, but can be done manually.  Refer to MACH_SEG_x.)

PDS FILEXE "PDS FILE"§	Refer to DESIGN FILE.

PINXE "PIN"§	a) A keyword used to declare a pin's logical name, placement (or float status), storage type, and other attributes.  b) A physical I/O pin to which a logical name can be assigned and for which behavior can be defined in the design file.

PL2 FILEXE "PL2 FILE"§	A design file disassembled from an intermediate or a JEDEC file.

PLA FILEXE "PLA FILE"§	The intermediate file created by the Logic Minimizer and processed by the Fitter.

PLACEMENTXE "PLACEMENT"§	The process of associating signals with specific pins and nodes.

PLC FILEXE "PLC FILE"§	A file, generated by the Fitter following a successful fit, that contains the placement information for each signal used in the design.

PRD FILE	A file, generated by the Ditter following a successful fit, that contains the placement/routing information for the design.

PRELOADXE "PRELOAD"§	A simulation command that sets a register's Q output to the specified value (0 or 1).

PRESENT STATEXE "PRESENT STATE"§	The current state of a state machine.

PRESETXE "PRESET"§	A hardware feature that forces the flip-flop's Q register high regardless of the sum-of-products logic present at the flip-flop's D (or T) input.  (Also called "Set.")

PRIMARY EQUATIONXE "PRIMARY EQUATION"§	Refer to LOGIC EQUATION.

PRODUCT TERMXE "PRODUCT TERM"§	A set of signals that is ANDed to produce a resulting value.

PRODUCT-TERM STEERINGXE "PRODUCT-TERM STEERING"§	An architectural feature of MACH devices that allows the Fitter to fit, without gate-splitting, an equation of up to 20 product terms on a single macrocell, by "borrowing" the product terms of adjacent macrocells.

PROGRAMMABLE POLARITYXE "PROGRAMMABLE POLARITY"§	The ability to provide as output either a) the sum-of-products result of a logic equation or b) the complement of the sum-of-products result, depending on the polarity of the equation as expressed in the design file.  (Refer to ACTIVE HIGH and ACTIVE LOW for additional details.)

PROGRAMMED FUSEXE "PROGRAMMED FUSE"§	Equivalent to a "1" in the JEDEC file.  Sometimes referred to as a "blown" fuse.  (Refer to UNPROGRAMMED FUSE for comparison.)

Q OUTPUTXE "Q OUTPUT"§	The true output of a flip-flop.

/Q OUTPUTXE "/Q OUTPUT"§	The complement output of a flip-flop.

RADIXXE "RADIX"§	The number system according to which a number is to be interpreted.  Radices supported by MACHXL software are binary, octal, decimal, and hexadecimal (base 2, 8, 10, and 16, respectively).

RANGEXE "RANGE"§	A vector consisting of a set of pins or nodes that have the same root name but are differentiated by their subscripts.  Uses the general form Root_name[x..y], where x and y are positive integers.

REGISTERXE "REGISTER"§	a) A bank of flip-flops sharing the same clock signal.  b) An individual flip-flop.  c) verb  To synchronize signals by allowing values to change only in response to a common clock signal.

REGISTERED EQUATIONXE "REGISTERED EQUATION"§	A logic equation the result of which is stored in a register.

RESERVED WORDXE "RESERVED WORD"§	A word used by the MACHXL software to identify design segments and information, device names, commands, functions, and pin defaults.  Some reserved words are keywords that identify the block of information that follows the keyword.  Keywords and operators are listed at the beginning of Chapter 5, "Language Reference."

RESETXE "RESET"§	A hardware feature that forces the flip-flop's Q register low regardless of the sum-of-products logic present at the flip-flop's D (or T) input.

ROUTINGXE "ROUTING"§	The process of finding paths between placed pins and nodes for input, output, feedback, clock, set, and reset signals.

RPT FILEXE "RPT FILE"§	The file containing the Fitting report.

RUN-TIME LOGXE "RUN-TIME LOG"§	A file (Design_name.LOG) that contains the messages generated by the compilation programs.

SETXE "SET"§	A hardware feature that forces the flip-flop's Q register high regardless of the sum-of-products logic present at the flip-flop's D (or T) input.  (Also called "Preset.")

SETF COMMANDXE "SETF COMMAND"§	A simulation command that loads specified inputs with specified values.

SIMULATION PROGRAMXE "SIMULATION PROGRAM"§	Checks the functionality of a compiled design.

SIMULATION SEGMENTXE "SIMULATION SEGMENT"§	The portion of a design file that contains simulation commands.  (Refer to AUXILIARY SIMULATION FILE for comparison.)

STATE ASSIGNMENTXE "STATE ASSIGNMENT"§	An equation that defines a state as a unique combination of register values.

STATE BIT ASSIGNMENTXE "STATE BIT ASSIGNMENT"§	Refer to STATE ASSIGNMENT.

STATE BRANCHXE "STATE BRANCH"§	The transition of a state machine from one state to another.

STATE DIAGRAMXE "STATE DIAGRAM"§	A graphical representation of state machines in which individual states appear as ovals (sometimes called "bubbles") and branches appear as arrows connecting two ovals.

STATE EQUATIONXE "STATE EQUATION"§	An equation in the STATE segment that defines branches from the present state.

STATE MACHINE DESIGNXE "STATE MACHINE DESIGN"§	A design file that includes either a) one or more state machines implemented using CASE and IF-THEN-ELSE statements, or b) a single state machine implemented using the STATE syntax.

STATE OUTPUT EQUATIONXE "STATE OUTPUT EQUATION"§	An equation in the STATE segment that defines the state machine's outputs for a given state.

STATE SEGMENTXE "STATE SEGMENT"§	An optional segment in the design file that contains all STATE syntax.

STATE SYNTAXXE "STATE SYNTAX"§	An alternate method of specifying state machine designs (the preferred method is to use CASE and IF-THEN-ELSE statements).  STATE syntax is supported in MACHXL software for compatibility with existing PALASM designs.

STATE SYNTAX EXPANDERXE "STATE SYNTAX EXPANDER"§	The program that converts STATE syntax (if any) into Boolean equations.

STRINGXE "STRING"§	a) A keyword used to assign a logical name to a character string that will be substituted, wherever the logical name appears, by the Parser.  b) The character string thus defined.

SUM-OF-PRODUCTSXE "SUM-OF-PRODUCTS"§	An OR gate that sums the values of one or more product terms.

SYNC_LISTXE "MACH_SEG_x"§	A keyword used in a GROUP statement to list all signals that are to be configured as synchronous macrocells.

SYNCHRONOUS REGISTERXE "SYNCHRONOUS REGISTER"§	Each macrocell register can be configured by the Fitter as synchronous or asynchronous, depending on several factors.  A register that a) uses a single-literal clock that is placed at a global clock pin, and b) shares the Set and Reset lines common to all synchronous registers in its PAL block, is synchronous.  (Refer to ASYNCHRONOUS REGISTER for comparison.)

TAL FILEXE "TAL FILE"§	The file containing the Timing Analysis report.

TARGET DEVICEXE "TARGET DEVICE"§	a) The device specified in the CHIP statement of a PDS design file.  b) More generally, the device on which a design is to be implemented.

THREE-STATE BUFFERXE "THREE-STATE BUFFER"§	An output buffer that enables or disables the output signal path.  When applied to an I/O pin, the three-state buffer can be used to disable output to a pin temporarily so the pin can be used as an input.  The three-state buffer is controlled by the OUTPUT ENABLE EQUATION, if any, associated with the pin.  If no output enable equation exists, the buffer assumes a default state as follows: always enabled if an output equation is associated with the pin, always disabled if no output equation is associated with the pin.

TNC FILE	Contains a conversion table listing the original version of each signal name that was truncated during compilation and/or simulation.  The TNC file is used, during back-annotation, to generate pin placements for the original signal names rather than overwriting the original names with the truncated names.

TOGGLEXE "TOGGLE"§	To reverse logical state in response to a clock pulse.

TRACEXE "TRACE"§	A tool that allows you to view simulation results for a specified group of signals rather than for all signals declared in the design file.

TRACE FILEXE "TRACE FILE"§	A subset of the history file that shows results for only those signals specified in the TRACE_ON statement.

TRANSITIONXE "TRANSITION"§	A state machine's process of changing from one state to another.

TRE FILEXE "TRE FILE"§	The intermediate file create by the Parser and modified by the Boolean Post-Processor, STATE Syntax Expander, and Logic Minimizer.

TRF FILEXE "TRF FILE"§	Created by the Simulator the design includes a TRACE_ON statement.  The TRV file contains simulation information for each signal listed in the TRACE_ON statement.

TRV FILE	Created by the Simulator if vector signals are included in the TRACE_ON statement.  The TRV file contains vector values expressed as hexadecimal values rather than as individual signal states.

UNCONDITIONAL BRANCHXE "UNCONDITIONAL BRANCH"§	A state branch that occurs whenever a state machine enters a certain state, regardless of the input conditions.

UNPROGRAMMED FUSEXE "UNPROGRAMMED FUSE"§	Equivalent to a "0" in the JEDEC file.  Sometimes referred to as an "intact" fuse.  (Refer to PROGRAMMED FUSE for comparison.)

UTILIZATIONXE "UTILIZATION"§	The percentage of the device's resources occupied by a design for which a successful fit was found.

VCCXE "VCC"§	a) The reserved word used in MACHXL designs to denote an unconditionally true condition.  b) A permissible logical name for the VCC pin(s) of a MACH device.

VECTORXE "VECTOR"§	A set of signals in which the order of signals is always constant.  A vector can consist of a range, a comma-delimited list of signals, or a comma-delimited list of signals and ranges.

WHILE-DO LOOPXE "WHILE-DO LOOP"§	A simulation construct that repeats a set of commands while the specified condition remains true.