| 7 | Simulation Segment In Depth |
| --- | --- |

Contents

µOverview	245

Creating a Simulation File	246

Simulation Command Summary	246

Simulation Segment vs. Auxiliary File	248

Considerations	249

Vectors In Simulation	250

SETF and PRELOAD	250

CHECK and CHECKQ	251

CLOCKF	252

TRACE_ON	252

Flip-Flops	252

Buried Nodes	253

Latches	253

Output Enable	253

Preloaded Registers	254

Verified Signal Values	254

Viewing Simulation Results	254

All Signals	255

Trace Signals Only	257

Text Display, Non-Vectored	258

Text Display, Vectored	259

Waveform Display, Non-Vectored	260

Waveform Display, Vectored	261

Using Simulation Constructs	261

For Loop	261

While Loop	262

If-Then-Else	262

Design Examples	263

Boolean Equation Design	263

State Machine Design	266

Notes On Using the Simulator	267

Modeling of Registers and Latches	268

Programmer Emulation at Power-Up	268

Power-Up Sequence	269

Software Preload Sequence	269

Full Evaluation of Input Pins	270

Clock Polarity	270

Driving Active-Low Clocks	271

Product Term-Driven Clocks	273

Simultaneous Events	274

Power-Up Preload On Floating Pins	274

Output Buffers	274

Input Signal Ordering	275

Preventing Unexpected Simulation Behavior	276

Placement Information Missing	276

Set/Reset Signals Swapped	276

Set/Reset Signals Treated As "Don't Care"	277

Uncontrollable Power-Up Conditions	277

# OverviewXE "simulation:overview"§

This chapter describes the features of the MACHXL simulator and provides a design implemented with Boolean equations and state machine implemented with CASE statements to illustrate simulation concepts.  The chapter is divided into five major discussions.

r	An overview of the MACHXL simulation process

r	A summary of the simulation keywords and considerations for simulating a design

r	General information about viewing simulation results

r	Information about using the FOR, WHILE, and IF-THEN-ELSE simulation constructs

r	Design examples of Boolean equation and a state-machine design simulation

| ã | Note: The simulation files for the design examples provide comments to explain each line. |
| --- | --- |

The MACHXL simulator allows you to perform functional verification of MACH-device designs.  You define simulation statements in either the simulation segment of the PDS file or in an auxiliary simulation file.

After entering the simulation statements, you simulate the design and view the results in either a graphical waveform or text format.

| ã | Note: Because the MACHXL simulator performs functional verification, additional delays induced by looping back through an array are not reflected in the simulation results. Refer to the Timing Report, generated by the Fitter, for information on the timing characteristics of your design. |
| --- | --- |

# Creating a Simulation FileXE "simulation file, creating"§

You create an auxiliary simulation file to specify a sequence of simulator statements.  A simulation statement consists of a keyword alone, or a keyword and a list of signals.  This discussion provides a summary of the simulation keywords, information about the simulation segment and auxiliary file, and considerations for simulating a design.

## Simulation Command SummaryXE "simulation:command summary"§

The following keywords are provided with the MACHXL simulator.  Included here is a brief summary of each command.  In the following examples, O1, O2, O3, and O4 are output pin names;Q0 and Q1 are register output names.

Use the following symbols to check for specific signal values:

r	Check for the state of a signal as follows:

u	If the signal name is uncomplemented in the PIN or NODE statement (for example, OUT2), use the uncomplemented name to check for a 1 at the signal  (for example, CHECK OUT2), or use the complemented name to check for 0 at the signal (for example, CHECK /OUT2).

u	If the signal name is complemented in the PIN or NODE statement (for example, /OUT2), use the uncomplemented name to check for a 0 at the signal  (for example, CHECK OUT2), or use the complemented name to check for 1 at the signal (for example, CHECK /OUT2).

The simulator reports a discrepancy if the actual value differs from the one for which you checked.

r	Precede a pin name with a caret (^) to verify that the corresponding pin's three-state ouput buffer is disabled (for example, CHECK ^OUT2).  The simulator reports a discrepancy if the buffer is not disabled.

r	Precede a pin or node name with a percent sign (%) to verify that the corresponding pin's or node's value is undefined (for example, CHECK %OUT2).  The simulator reports a discrepancy if the pin's or node's value is not undefined.

| ã | Note: Simulation examples throughout this chapter illustrate the use of many of these statements. |
| --- | --- |

| CHECKXE "CHECK"§ | Use this keyword to verify that values at the pin are equal to expected values. For example: CHECK O1 /O2 ^O3 %O4 |
| --- | --- |
| CHECKQXE "CHECKQ"§ | Use this keyword to verify that values at the register outputs are equal to expected values. For example: CHECKQ Q0 /Q1 |
| CLOCKFXE "CLOCKF"§ | Use this keyword to generate a clock cycle for a rising-edge or falling-edge clock. The clock cycle for a rising-edge clock is low-to-high-to-low. The clock cycle for a falling-edge clock is high-to-low-to-high. |
| FOR LOOPXE "FOR loop"§ | Use this construct to perform a task a specified number of times. |
| IF-THEN-ELSEXE "IF-THEN-ELSE"§ | Use this construct to test a condition and then perform one of two tasks, depending on the test results. |
| PRELOADXE "PRELOAD"§ | For software simulation only, use this keyword to load specified values into the register outputs. For example: PRELOAD Q0 /Q1 JEDEC TEST VECTOR OUTPUT IS TURNED OFF AT THE FIRST OCCURRENCE OF THE PRELOAD KEYWORD. |
| SETFXE "SETF"§ | Use this keyword to assign specific values to inputs during simulation. For example: SETF IN1 /OE |
| SIMULATIONXE "SIMULATION"§ | Use this keyword at the beginning of each simulation segment or auxiliary simulation file. |
| TRACE_OFFXE "TRACE_OFF"§ | Use this keyword to end a simulation section being traced by the TRACE_ON keyword. For example: TRACE_OFF |
| TRACE_ONXE "TRACE_ON"§ | Use this keyword to define which signals to record in the trace file during simulation. For example: TRACE_ON IN1 IN2[0..3] O1 CLOCK |
| WHILE LOOPXE "WHILE loop"§ | Use this construct when you cannot predetermine how many times to perform a task. The task will be performed while a condition is true. |

## Simulation Segment vs. Auxiliary FileXE "auxiliary file vs. simulation segment"§XE "simulation segment vs. auxiliary file"§

You define the simulator statements in either the simulation segment of the PDS file or in an auxiliary simulation file.  Auxiliary simulation files allow you to simulate several similar designs using the same simulation file.

| ã | Note: In this chapter, the term simulation file is used to refer to either a simulation segment or an auxiliary simulation file. |
| --- | --- |

The simulation segment looks the same as the auxiliary file, except it is part of the design file, as shown below.

;MACHXL Design Description
...

;------------------ Simulation Segment ---------------------------
SIMULATION
TRACE_ON INPUT CLOCK OUTPUT 	;Specify signals for the 
                            ;trace output file
SETF /CLOCK INPUT           ;Initialize INPUT to logical 
                            ;1, CLOCK to logical 0

CLOCKF CLOCK                ;Apply a full clock cycle 
                            ;to CLOCK.
CHECK OUTPUT                ;Verify that the output pin 
                            ;is at logical 1
CHECKQ /Q0                  ;Verify that the Q0 register 
                            ;is at logical 0
TRACE_OFF                   ;Turn tracing ofF

The auxiliary simulation file is a stand-alone file that must be in the same directory as the design; the file name should match the name of the design file and include a .SIM extension.  An example of an auxiliary simulation file is shown below.

...

SIMULATION

TRACE_ON INPUT CLOCK OUTPUT ;Specify signals for the 
                            ;trace output file
SETF /CLOCK INPUT           ;Initialize INPUT to logical
                            ;1, CLOCK to logical 0
CLOCKF CLOCK                ;Apply a full clock cycle 
                            ;to CLOCK.
CHECK OUTPUT                ;Verify that the output 
                            ;pin is at logical 1
CHECKQ /Q0                  ;Verify that the Q0 register 
                            ;is at logical 0
TRACE_OFF                   ;Turn tracing off

Depending on the working environment you've set up, a message asks if you are using an auxiliary simulation file, either on demand or automatically when you simulate.

## Considerations

The following discussions provide general considerations for simulating a design.

r	Vectors in Simulation

r	Flip-Flops

r	BuriedNodes

r	Latches

r	Output Enable

r	Preloaded Registers

r	Verified Signal Values

### Vectors In SimulationXE "vectors:in simulation"§XE "simulation:vectors"§

You can use vectors in the following simulation statements: SETF, PRELOAD, CHECK, CHECKQ, CLOCKF, and TRACE_ON.

Vectors are used in simulation in four ways:

r	In SETF and PRELOAD statements, a vector of pins or nodes is set equal to a user-specified (or implied) value.

r	In CHECK and CHECKQ statements, the vector of pins or nodes is checked against a user-specified (or implied) value.

r	In CLOCKF statements, a clock pulse is applied to a vector of clock pins.

r	In TRACE_ON statements, you list one or more vectors to be added to the trace display.  Signals specified in vector format in the TRACE_ON statement are displayed, in the trace output, as hexadecimal numbers.  Each hexadecimal number represents the values of four signals in the vector, as shown below.

µ §

#### SETF and PRELOADXE "SETF:using vectors"§XE "vectors:SETF"§XE "PRELOAD:using vectors"§XE "vectors:PRELOAD"§

Valid usage includes the following:

SETF Vector	PRELOAD Vector 	;sets/preloads all bits high

SETF /Vector	PRELOAD /Vector 	;sets/preloads all bits low

SETF Vector = number	PRELOAD Vector = number	;sets/preloads bits to binary  equivalent of number10

;(unless a different radix is 
;specified)

| ã | Note: JEDEC test vector output is turned off after the first occurrence of the PRELOAD keyword. |
| --- | --- |

Examples

The following statement:	Results in the following values for these individual signals:

|  | S[7] | S[6] | S[5] | S[4] | S[3] | S[2] | S[1] | S[0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SETF SAMPLE[7..0] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| SETF /SAMPLE[7..0] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| SETF SAMPLE[7..0] = 5 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| SETF SAMPLE[7..0] = #hEB | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |

#### CHECK and CHECKQXE "CHECK:using vectors"§XE "vectors:CHECK"§XE "CHECKQ:using vectors"§XE "vectors:CHECKQ"§

Valid usage includes the following:

CHECK Vector	CHECKQ Vector 	;checks that all pins/registers corresponding to vector signals are high

CHECK /Vector	CHECKQ /Vector 	;checks that all pins/registers corresponding to vector signals are low

CHECK Vector = number	CHECKQ Vector = number	;checks that pins/registers

;corresponding to vector are

;binary equivalent of

;number10 (unless a

;different radix is specified).

Examples

The following statement:	Checks for the following values:

|  | S[7] | S[6] | S[5] | S[4] | S[3] | S[2] | S[1] | S[0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHECK SAMPLE[7..0] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| CHECK /SAMPLE[7..0] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CHECK SAMPLE[7..0] = 5 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| CHECK SAMPLE[7..0] = #hEB | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |

#### CLOCKFXE "CLOCKF:using vectors"§XE "vectors:CLOCKF"§

Clocks all clock signals specified in the vector.

Example

CLOCKF CLOCKS[0..3]

The vector must include only clock pins.  If you are using product-term clocks, use the SETF keyword instead of the CLOCKF keyword.

#### TRACE_ONXE "TRACE_ON:using vectors"§XE "vectors:TRACE_ON"§

You can include in a TRACE_ON statement some or all of the signals defined in a vector.  Simulation results for signals specified in vector format will be reported and displayed as hexadecimal numbers.

Refer to the "Viewing Simulation Results" section in this chapter for additional details.

### Flip-FlopsXE "flip-flops, in simulation"§XE "simulation:flip-flops"§

You can set the output state of any flip-flop during simulation by using the PRELOAD command.

| ã | Note: JEDEC test vector output is turned off after the first occurrence of the PRELOAD keyword. |
| --- | --- |

Flip-flops can be clocked with the CLOCKF command or with a series of SETF statements.

| ã | Note: If you are not using the default clock, use the appropriate initialization statement to initialize at the beginning of the simulation. SETF /<clock_name> ;to initialize a leading-edge clock SETF <clock_name> ;to initialize a trailing-edge clock Otherwise, the simulator reports a warning. |
| --- | --- |
| ã | Note: Setting the value of inputs to a register in the same SETF statement used to generate a clock pulse to the register can cause simulation errors. Instead, set the inputs in one SETF statement, then use another SETF statement to generate the clock pulse. |

### Buried NodesXE "buried nodes, in simulation"§XE "simulation:buried nodes"§

Buried nodes are treated as any other signal in the history and trace files, but they are not included in the JEDEC output file.  The logic states of buried nodes declared in the pin declarations segment are automatically displayed in the history file.

### LatchesXE "latches, in simulation"§XE "simulation:latches"§

For MACH 1xx/2xx designs, the following illegal latch states will result in simulator error messages for an active-low latch.

Latch Enable	Async. Reset	Async. Preset

1	1	1

0	0	1

0	1	0

0	1	1

For MACH 1xx/2xx designs, the following illegal latch states will result in simulator error messages for an active-high latch.

Latch Enable	Async. Reset	Async. Preset

0	1	1

1	0	1

1	1	0

1	1	1

### Output EnableXE "output enable, in simulation"§XE "simulation:output enable"§

If you do not write a .TRST equation for an output pin, the simulator presumes the output pin to be enabled at all times.  If you do write a .TRST equation for an output pin, the simulator presumes the output to be enabled only when the .TRST equation evaluates as true.

### Preloaded RegistersXE "preloaded registers, in simulation"§XE "simulation:preloaded registers"§

You can preload a value into any register during a simulation session.  In state-machine designs, this allows you to set the state bits as required to access any state directly.

The PRELOAD command sets the Q output of the flip-flop to the specified value.

| ã | Note: You can use PRELOAD statements to load a known state to a pin or register during software simulation. However, test vectors are turned off in the JEDEC file as soon as the first PRELOAD statement occurs. For final verification, replace the PRELOAD statement with appropriate SETF/CLOCKF statements so that you can generate JEDEC test vectors and verify design performance using the device programmer. |
| --- | --- |

### Verified Signal ValuesXE "verified signal values, in simulation"§XE "simulation:verified signal values"§

There are two simulator statements that verify the logic states of signals:  CHECK and CHECKQ.

The CHECK command verifies that the simulation result(s) at the pin correspond to your predictions of the design's behavior.  If a discrepancy is detected, a question mark is inserted in the simulation history and trace files at the corresponding signal and test vector, and a warning is issued in the execution-log file.

The CHECKQ keyword verifies the value of a specified signal at the output of the register.  This command is useful for checking the logical state of buried registers.

# Viewing Simulation ResultsXE "simulation:viewing results"§XE "viewing simulation results"§

Once a simulation completes successfully, the results are stored in a history, and optionally, a trace file.  You can view the results in a text or graphical form using commands that appear in the View menu.

## All SignalsXE "simulation:viewing all signals"§

The history file shows the results for every pin and node defined in the pin list of the design.  The polarity of each pin and node is displayed according to the definition in the pin list.  You can view this file in a graphical or text format.

Use the All signals command (View:Simulation Data:All signals) to view the text version of the history file.  This display shows the status of all signals defined in the pin list using letters to represent various states.

The simulation display shows the status of all signals defined in the design file using letters to represent various states:

r	H  =  high

r	L   =  low

r	Z  =  high impedance

r	X  =  undefined (symbol X takes precedence over Z)

r	?  =  discrepancy (symbol ? takes precedence over X)

r	c  =  CLOCKF statement (appears at the top edge of the display)

r	g  =  SETF statement (appears at the top edge of the display)

An example of a simulation history text display is shown below.  (Signals in a vector are listed individually.)

Use the All signals command (View:Waveform display:All signals) to view the waveform version of the history file.  This command converts the ASCII characters into a graphical display similar to a timing diagram.  An example of a history waveform display is shown below.

| ã | Note: If the simulation file includes CHECK or CHECKQ keywords, discrepancies between a specified value and the simulated value of a signal are flagged with a question mark, ?, at the location of the discrepancy. |
| --- | --- |

## Trace Signals OnlyXE "simulation:viewing trace signals only"§

The trace file is only generated if the TRACE_ON command is included in the simulation file.  This file shows results for the signals specified as parameters in the command.  The polarity of each pin and node is displayed according to the definition in the TRACE_ON command.

| ã | Note: If the polarity of the signal in the TRACE_ON statement matches the polarity of the signal in the PIN or NODE statement, the trace waveform will reflect the physical levels at the pin. |
| --- | --- |
| ã | Note: If the simulation file includes CHECK or CHECKQ keywords, discrepancies between the specified value and the simulated value of a signal are flagged with a question mark, ?, at the location of the discrepancy. If a discrepancy occurs in one of the bits of a vector represented as a hexadecimal value, the hexadecimal numeral representing the nibble that contained the discrepancy (or discrepancies) will be replaced with a question mark, ?. |

The trace file is useful for the following four situations:

r	If you want to display vectors as hexadecimal values rather than as individual signals

r	If you do not want to display certain pins or nodes that are not relevant to a simulation session

r	If you want to group signals by function, so they can be viewed on the same page of the display

r	If you want to view the reverse polarity of output signals

You can view the trace file in a text (choose View:Simulation data:Trace signals only) or graphical format (choose View:Waveform display:Trace signals only).

A submenu offers you two choices:

r	Non-vectored
Choose this option to see the value of each signal in a vector represented separately.

r	Vectored
Choose this option to see the value of the entire vector expressed as a hexadecimal value.

### Text Display, Non-VectoredXE "simulation:text display:non-vectored"§

Choose View:Simulation Data:Trace signals only:Non-vectored to display values for all signals, as shown below.  (Signals in a vector are listed individually.)

The simulation display shows the status of all signals defined in the TRACE_ON command using letters to represent various states:

r	H  =  high

r	L  =  low

r	Z  =  high impedance

r	X  =  undefined (symbol X takes precedence over Z)

r	?  =  discrepancy (symbol ? takes precedence over X)

r	c  =  CLOCKF statement (appears at the top edge of the display)

r	g  =  SETF statement (appears at the top edge of the display)

### Text Display, VectoredXE "simulation:text display:vectored"§

Choose View:Simulation Data:Trace signals only:Vectored to display values of all signals.  Vector signals are displayed collectively, as a hexadecimal value, as shown below.

VecorXE "vector:long names, handling"§XE "long vector names"§ names that exceed 14 characters in length will be aliased to a shorter form and the aliases listed in the trace file as follows:

LONG SIGNAL NAME ALIASES:

BEGIN

Q_X[]_1 ==> Q_X[4,2,..3,7..6,0..1]

ID[]_2 ==> ID[23,15,11,8,7,1..0]

END

The simulation display shows the status of all signals defined in the TRACE_ON command using letters to represent various states:

r	H  =  high

r	L  =  low

r	Z  =  high impedance (Any of the four bits in the hecadecimal number representing four vectored signals is in the high-impedance state)

r	X  =  undefined (Any of the four bits in the hecadecimal number representing four vectored signals is in the undefined state)  (Symbol X takes precedence over Z)

r	?  =  discrepancy in any of the four bits of a hexadecimal number (Symbol ? takes precedence over X)

r	c  =  CLOCKF statement (appears at the top edge of the display)

r	g  =  SETF statement (appears at the top edge of the display)

### Waveform Display, Non-VectoredXE "simulation:waveform display:non-vectored"§

Choose View:Waveform Display:All signals:Non-vectored to display values for vector signals individually as shown below.

### Waveform Display, VectoredXE "simulation:waveform display:vectored"§

Choose View:Waveform Display:All signals:Vectored to display values for all signals.  Vector signals are displayed, collectively, as a hexadecimal value, as shown below.

# Using Simulation ConstructsXE "simulation:constructs"§

The MACHXL simulator provides the following constructs for developing loops and making decisions during a simulation session.

## For LoopXE "FOR Loop"§XE "simulation:FOR loop"§

The FOR loop is the most basic flow-of-control construct.  It is ideal for applications in which you can predetermine how many times you must repeat a set of instructions, as illustrated below.

SIMULATION
SETF /OE /CLOCK COUNT
  FOR X:= 1 TO 9 DO
    BEGIN
      CLOCKF CLOCK
    END

## While LoopXE "WHILE loo["§XE "simulation:WHILE loop"§

When you cannot predetermine how many times to perform a task, you can use the WHILE loop to perform the task while some condition remains true, as illustrated below.

SIMULATION
SETF /OE /CLOCK COUNT
  WHILE ( /(BIT3 * /BIT2 * BIT1 * /BIT0) ) DO
    BEGIN
      CLOCKF CLOCK
    END

## If-Then-ElseXE "If-Then-Else in simulation"§XE "simulation:If-Then-Else"§

The IF-THEN-ELSE construct is for testing a condition and then performing one of two tasks, depending on the test results.  The following example nests two IF-THEN-ELSE loops in a FOR loop.  (The signal RST used in this example refers to an input pin, not to the macrocell's asynchronous reset product term.)

SIMULATION
FOR I := 1 TO 16 DO
    BEGIN
      IF ( I <= 9 ) THEN  ;If I is less than or equal to 9, 
                          ;enable count.
        BEGIN
          SETF CNT /RST
          CLOCKF CLOCK
        END

ELSE

BEGIN
        IF ( I < 16 ) THEN ;If I is greater than 9 but 
                           ;less than 16,
          BEGIN            ;continue with no count.
            SETF /CNT /RST
            CLOCKF CLOCK
          END
        ELSE               ;If I is equal to 16, 
          BEGIN            ;reset the state machine.
            SETF RST
            CLOCKF CLOCK
          END

END
END

# Design ExamplesXE "simulation:design examples"§

The following discussions illustrate how to simulate a Boolean 4-bit counter and a state machine implemented using CASE statements.  The simulation statements are contained in auxiliary simulation files.

## Boolean Equation DesignXE "simulation:boolean equation design"§

This discussion is based on simulating the basic 4-bit counter design shown below.

CHIP     _bcntr	MACH435

PIN  3   QA     REGISTERED 
PIN  4   QB     REGISTERED
PIN  5   QC     REGISTERED
PIN  6   QD     REGISTERED
PIN  20  CLOCK

PIN  ?   RST

NODE 1   GLOBAL

EQUATIONS

GLOBAL.RSTF = RST

QA.T = VCC
QA.CLKF = CLOCK
QB.T = QA
QB.CLKF = CLOCK
QC.T = QA * QB
QC.CLKF = CLOCK
QD.T = QC * QB * QA
QD.CLKF = CLOCK

To simulate this design, you can set up a FOR loop that clocks the counter 16 times, as illustrated in the auxiliary simulation file shown in the following example.

SIMULATION

TRACE_ON CLOCK QA QB QC QD ;Generate a trace file with 
                           ;the specified signals.
SETF /CLOCK RST            ;Initialize the clock signal 
                           ;to a logical 0 and initialize

;registers with global reset

Continued...

...Continued

SETF /RST                  ;Restore global reset line

FOR I:= 1 TO 16  DO        ;Clock the counter 16 times.
	BEGIN               ;This FOR loop takes the 
	  CLOCKF CLOCK	      ;place of 16 individual 
	END                 ;Clockf statements.
TRACE_OFF                  ;Turn tracing off.

The simulation results are recorded in a history and a trace file.  You can view either of these files in a text or a graphical mode.

| ã | Note: In the waveform display, the letters g and c indicate the occurrence of SETF and CLOCKF statements, respectively. |
| --- | --- |

If you know what the simulation results should be during any portion of the simulation session, you can use the CHECK or CHECKQ  statements to flag discrepancies.

The following simulation purposely checks for a wrong value.

SIMULATION
TRACE_ON CLOCK QA QB QC QD  ;Generate a trace file with 
                            ;the specified signals.
SETF /CLOCK                 ;Initialize the clock signal 
                            ;to logical 0.
SETF  RST                   ;Initialize registers with global reset

SETF /RST                   ;Restore global reset line

CLOCKF CLOCK                ;Clock the counter to 0001.
CLOCKF CLOCK                ;Clock the counter to 0010.
CHECK /QA /QB /QC /QD       ;Check for 0000, this flags 
                            ;a discrepancy.
TRACE_OFF                   ;Turn tracing off.

The simulation results mark the location of the discrepancy with a question mark, as shown in the following trace text and waveform figures.

| ã | Note: In both displays, the letters g and c indicate the occurrence of SETF and CLOCKF statements, respectively. |
| --- | --- |

## State Machine DesignXE "simulation:state machine design"§

The following simulation segment corresponds to the file TEST2.PDS referred to in Chapter 2 and discussed under "Building State Machines with CASE Statements" in Chapter 6, "Equations Segment In Depth."  This file is in the \MACHXL\EXAMPLES directory

;------------------------------------------------------------

SIMULATION

;============================================================

;            SIMULATION FOR TRAFFIC CONTROLLER

;============================================================

TRACE_ON CLOCK1 SENSOR ST[1,0] VOUT[5..0]

SETF   /CLOCK1  : INITIALIZE CLOCK

SETF   _RESET1  ; RESET REGISTERS

SETF   /_RESET1

SETF	/SENSOR

CLOCKF	CLOCK1

SETF	SENSOR

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

SETF	/SENSOR

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

CLOCKF	CLOCK1

TRACE_OFF

;============================================================

;            SIMULATION FOR ANSWERING MACHINE

;============================================================

TRACE_ON CLOCK2 RING ENDGREETING DIALTONE ENDMESSAGES ST2[1,0]

ANSWER PLAY RECORD

SETF   /CLOCK2  : INITIALIZE CLOCK

SETF   _RESET2  ; RESET REGISTERS

SETF   /_RESET2

SETF	/RING /ENDGREETING /DIALTONE /ENDMESSAGES

CLOCKF	CLOCK2

Continued...

...Continued

CLOCKF	CLOCK2

SETF	RING

CLOCKF	CLOCK2

CLOCKF	CLOCK2

SETF	/RING

CLOCKF	CLOCK2

SETF	DIALTONE

CLOCKF	CLOCK2

SETF	/DIALTONE

SETF	RING

CLOCKF	CLOCK2

CLOCKF	CLOCK2

SETF	ENDGREETING

CLOCKF	CLOCK2

CLOCKF	CLOCK2

SETF	/ENDGREETING ENDMESSAGES /RING

CLOCKF	CLOCK2

CLOCKF	CLOCK2

SETF	/ENDMESSAGES

CLOCKF	CLOCK2

CLOCKF	CLOCK2

TRACE_OFF

# Notes On Using the SimulatorXE "simulation:notes on using"§

| ã | Note: This section contains information on how the Simulator performs its normal functions. You do not need to know this information to run the Simulator successfully. It is provided as general background information for those who are interested. |
| --- | --- |

The following sections describe specific Simulator behavior, and offer suggestions on how to obtain the best performance from the Simulator.

## Modeling of Registers and LatchesXE "simulation:modeling of registers and latches"§XE "modeling of registers and latches, in simulation"§

The MACHXL Simulator models registers and latches as follows:

r	Registers and latches with unknown SET, RESET, or CLK/LE signals generate an unknown state at the output Q.

r	The clock triggers only if it rises from a low to a high in the case of an active-high clock, or from a high to a low in the case of an active-low clock.

r	The Simulator reflects the specifications in the MACH Family Data Book.  For example, in MACH 3xx/4xx designs, it is legal to have SET and RESET signals high at the same time (RESET dominates).

## Programmer Emulation at Power-UpXE "simulation:programmer emulation at power-up"§XE "power-up, in simulation"§XE "programmer emulation at power-up, in simulation"§

PLD programmers and testers force a default condition on pins set to unknown logic states.  On some programmers, the default test condition is programmable.  The JEDEC format for the default test condition is "X0" for a low state, "X1" for a high state.  This field must be placed before the first test vector and after the number of pins (QP) and the number of fuses (QF) fields.

Nearly all AMD-approved programmers support the default test condition "X0."  The Simulator assumes that all pins are forced to a soft-low before power-up, and places an "X0" before the first test vector.

The unknown state cannot be realized in a physical sense in hardware.  Individual programmers can set pins high or low, and some programmers are even able to set pins to float (normally not done because the effect of a floating pin cannot be determined for all devices and test cases).

Uninitialized pins are generally set to the default test condition before the power is turned on the device under test.  Programmers cannot determine which pins are input and which are outputs, and therefore must use "soft conditions."  Under soft conditions, pins are driven high or low by a resistance low enough to drive an input pin but not low enough to override or destroy an output pin.

The Simulator shows the default test condition for uninitialized pins in the history file.  However, uninitialized pins remain unknown (marked "X") in the JEDEC test vectors because some testers have a limit on the number of pins that can be toggled in a single test vector.

## Power-Up SequenceXE "power-up sequence, in simulation"§XE "simulation:power-up sequence"§

The simulator's two-stage power-up routine gives improved simulation of registered device behavior.  The routine evaluates the device state before the first user-defined test vector is applied, and takes into account all control signals connected to registers.

Stage 1

r	Load all inputs with the default condition (currently 0) and enter affected signals into the event queue.

r	Load all registers with power-up preload values and enter affected signals into the event queue.

r	Fix registers so they will not change in response to control signals CLK/LE, SET, RESET, and PRELOAD.

r	Evaluate until steady state.

Stage 2

r	Load all inputs with the default condition (currently 0) and enter affected signals into the simulator's event queue.

r	Allow registers to be affected by control signals.

r	Evaluate until steady state.

## Software Preload SequenceXE "preload sequence, in simulation"§XE "software preload sequence, in simulation"§XE "simulation:software preload sequence"§

A two-stage preload routine gives improved simulation of registered device behavior.  Control signals such as SET, RESET, CLK/LE and output enables can affect the register states after they have been preloaded.

Stage 1

r	Load all inputs with the preload value and enter affected signals into the event queue.

r	Fix registers so they will not change in response to control signals CLK/LE, SET, RESET, and PRELOAD.

r	Evaluate until steady state.

Stage 2

r	Allow registers to be affected by control signals.

r	Evaluate until steady state.

| ã | Note: JEDEC test vector output is turned off after the first occurrence of the PRELOAD keyword. Use the PRELOAD keyword only for preliminary software verification. |
| --- | --- |

## Full Evaluation of Input PinsXE "simulation:full evaluation of input pins"§XE "evaluation of input pins, in simulation"§

r	All input pins are assumed to be initialized to the default conditions at power-up.

r	The effect of all input pins is evaluated at power-up.

## Clock PolarityXE "simulation:product term-driven clocks"§XE "product term-driven clocks, in simulation"§

In MACH devices that support active low clocks (all MACH 3xx/4xx devices) it is important to distinguish between clock behavior during simulation and clock behavior when the device is programmed.

When the device is tested, whether the C or K clock triggers a register at the leading or trailing edge depends only on the state of PIN and .CLKF statements in the design file. If the total number of slashes on these two variables is odd, then it has a falling edge. If it is even, it has is at a trailing edge.

However, during simulation, whether a C or K clock statement triggers a register at a leading or a trailing edge depends on the state of three variables: the PIN statement, the pin's .CLKF statement, and the CLOCKF simulation statement.

r	If the total number of slashes on the three variables is an odd number then the clock is triggered on a trailing edge.

r	If the total number of slashes on the three variables is an even number then the clock is triggered on a leading edge.

The following example illustrates a trailing edge K clock and the resulting simulation waveform.

µ §

The following example illustrates a leading edge C clock and the resulting waveform.

µ §

## Driving Active-Low ClocksXE "simulation:driving active-low clocks"§XE "active-low clocks, in simulation"§

Active-low clocks can be driven with an active-low clock signal at the pin.  An active-low clock (a JEDEC "K" clock) is a high-to-low-to-high pulse.

Polarity conventions are consistent with the polarity convention for the SETF command:

To generate a JEDEC "C" clock force for the following clock types, follow these rules:

r	For an active-high pin, CLK, use the simulation command "CLOCKF CLK"

r	For an active-low pin, CLK, use the simulation command  "CLOCKF /CLK"

To generate a JEDEC "K" clock force for the following clock types, follow these rules:

r	For an active-high pin, CLK, use the simulation command "CLOCKF /CLK"

r	For an active-low pin, CLK, use the simulation command  "CLOCKF CLK"

PIN 1 CLK1	;active-high pin

PIN 2 /CLK2

SIMULATION

; For active-high pins

SETF  CLK1	;Generates JEDEC "1" force

SETF /CLK1	;Generates JEDEC "0" force

; For active-low pins

SETF  CLK2	;Generates JEDEC "0" force

SETF /CLK2	;Generates JEDEC "1" force

; Global clock

CLOCKF		; Generates JEDEC "C" clock on default clock

; For active-high pins

CLOCKF  CLK1	;Generates JEDEC "C" clock

CLOCKF /CLK1	;Generates JEDEC "K" clock

; For active-low pins

CLOCKF  CLK2	;Generates JEDEC "K" clock

CLOCKF /CLK2	;Generates JEDEC "C" clock

µ §

Errors are generated if a "C" clock is asserted on a pin the state of which is initially high.  Errors are generated if a "K" clock is asserted on a pin the state of which is initially low.

## Product Term-Driven ClocksXE "simulation:product term-driven clocks"§XE "product term-driven clocks, in simulation"§

The Simulator supports JEDEC "U" and "D" transitions for dedicated clock pins.  A SETF on a pin will generate "U" and "D" JEDEC states only if the pin name is placed on a dedicated clock pin or the pin drives nothing but clock signals.  A warning is generated if the pin is used both as a clock and a data input.

The purpose of "U" and "D" clocks is to allow data from all other inputs to be stable before a latch enable or clock transition occurs.  Some dedicated clock pins can be used both as clock and as data pins.  Be aware that on a JEDEC tester this can cause some data lines to be driven at the same time or later than clock signals.

To avoid potential test problems with the simulation command "SETF," the test data and CLK/LE transitions should occur in separate test vectors.  "C" and "K" clock transitions should be used to drive pins that affect register clocks.

The Simulator supports both fast-rise and fast-fall transitions for dedicated clock pins on all devices controlled by the SETF syntax.  Data from all other inputs must be stable before a latch enable or a clock transition occurs.

Some dedicated clock pins can be used as both clock and data pins.  (For example, in the MACH435 device, pins 20, 23, 62, and 65 can be used both as dedicated clocks and data inputs.)  When using such pins as data inputs, be aware that some data lines could be driven at the same time or later than clock signals on a JEDEC tester, leading to differences between simulated and observed programmer behavior.  To avoid this problem, write simulation SETF and test patterns so that data and CLK/LE transitions occur in separate test vectors.  AMD recommends that only CLOCKF commands be used to drive pins that affect register clocks.

## Simultaneous EventsXE "simulation:simultaneous events"§XE "simultaneous events, in simulation"§

While the MACH devices allow the application of SET and RESET signals at the same time, removing both signals at the same time results in an unknown state.  Always remove SET and RESET signals in separate test vectors.

## Power-Up Preload On Floating PinsXE "preload on floating pins, in simulation"§XE "power-up preload on floating pins, in simulation"§XE "simulation:power-up preload on floating pins"§

The Simulator requires a physical location to preload pins with a power-up state.  If there are floating pins, the register value will be set to "X" (the unknown value) at start-up.  As a result, some test cases will generate different results if they are executed with floating pins.  The work around is to simulate after back-annotating the design using the "Last successful placement" option

## Output BuffersXE "simulation:output buffers"§XE "output buffers, in simulation"§

The Simulator does not always choose the correct input symbol set over the output symbol set when a common clock is used both to load an input register and control the output enable. (1 and 0 form the input set; H and L form the output set.)

When the effect of using a pin in a single test vector for both input and output operations is considered, the problem becomes apparent.  When a pin changes from an input logic state (represented by "0" or "1" in the JEDEC signal vector) to an output, the Simulator cannot decide which symbol to use to denote the signal level during the transition period, resulting in possible simulation errors, as shown in the following figure.

µ §

To avoid this problem, separate the three-state buffer control from the clock event controlling the output register, by adding an extra input to the product term controlling the three-state buffer.

## Input Signal OrderingXE "simulation:input signal ordering"§XE "input signal ordering, in simulation"§

Programmers apply inputs to a device in different sequences.  Some apply inputs in sequential pin order, some apply them in groups of eight pins at a time, and others use different schemes.  With so many possibilities, no simulator can handle all situations.

To minimize the chance of errors, define test vectors with device logic in mind, avoiding potential races in test vector definition so that any variation in the input sequences will produce the same result.  In conventional synchronous logic, this is not a problem—all data input transitions are applied before the device is clocked.  However, you should avoid simultaneous clock events.

The problem is more difficult for asynchronous logic designs.  Control functions like SET and RESET should be applied and removed in separate test vectors with an "idle" state in between, so that even if input changes are skewed, both control functions will not be applied simultaneously.  Likewise, data changes should be separated from storage-enabling or clocking events, so that the ordering of input changes is less likely to have an effect on the output.

## Preventing Unexpected Simulation BehaviorXE "simulation:preventing unexpected behavior"§XE "preventing unexpected simulation behavior"§

The following subsections give work-arounds for common simulation problems:

### Placement Information MissingXE "simulation:placement information missing"§

The Simulator needs placement information, generated by the Fitter, in order to model correctly the Set and Reset functions for each register.  Always run the Fitter until a successful fit is found before running the Simulator.

### Set/Reset Signals SwappedXE "simulation:Set/Reset signals swapped"§

When the SET/RESET treated as DONT_CARE field (MACH Fitting Options form) is set to "Don't Care," the Fitter sometimes swaps the Set and Reset product terms of individual registers to allow them to be grouped together in the same PAL block of the MACH device.  You can prevent this by grouping registers that share identical Set and Reset product terms into the same PAL block, using the GROUP MACH_SEG_x statement.  Refer to MACH_SEG_x in Chapter 5, "Language Reference," for details.

### Set/Reset Signals Treated As "Don't Care"XE "simulation:Set/Reset signals treated as Don't Care"§

Setting the SET/RESET treated as DONT_CARE option in the MACH Fitter Options form to "Y" can result in unexpected behavior in registers for which you specified only the Set or only the Reset condition.  For example, if you write the following initialization equations:

OUT1.SETF = A * B
OUT1.RSTF = INIT
OUT2.SETF = A * B

and leave the Reset condition for OUT2 unspecified, the Fitter can (if the SET/RESET treated as DONT_CARE option is set to "Y") use the same Set and Reset lines for both OUT1 and OUT2.  This results in OUT2 resetting on the INIT condition—an unspecified behavior.

To prevent unexpected behavior, do one of the following:

r	Do not set the SET/RESET treated as DONT_CARE option to "Y"

r	If the Fitter adds a Set or Reset line and such behavior is acceptable, make it explicit by adding the missing .SETF or .RSTF functional equation(s) to your design.

### Uncontrollable Power-Up ConditionsXE "simulation:uncontrollable power-up conditions"§

Power-up conditions in registers are not under your explicit control.  The order in which Set and Reset equations are implemented by the Fitter determines, for each register, which product term (Set or Reset) is associated with the power-up detection circuit.

The Simulator models correctly the configuration information provided by the Fitter, but you cannot control the assignment of the power-up detection circuit.  To initialize the device dependably, provide initialization logic in your design, and provide an explicit initialization test vector at the beginning of the SIMULATION segment or auxiliary simulation file, to initialize registers to a known state.  Do this in either of the following ways:

r	Specify a set and reset product term for each macrocell.

r	Add an initialization product term to the sum-of-products logic for each signal you want to initialize.