| C | Creating a LIM File |
| --- | --- |

Contents

µOverview	359

LIM File Conventions	360

Syntax	360

BLOCK Statement	360

Parameters	362

# Overview

The MACHXL partitioning limits (LIM) control file specifies the maximum number of macrocells and logic array inputs (signals to route to a block) available to a design. If a limit less than the maximum available is set, the partitioner places only the specified number of signals per block, and leaves the remaining logic array inputs or macrocells available as reserves for future design additions.  Limiting the number of logic array inputs will also increase the number of routing resources available to the limited array inputs, therefore increasing the routing chances for all the signals.

Refer to the "Using Place and Route Data to Limit Placements" section in Chapter 9, "Report Files," for tips on using the Place and Route Data (PRD) file to determine if a LIM file can improve fitting performance in your design.

# LIM File Conventions

If no LIM file exists, the Partitioner fills blocks with array inputs (fanin), product term-driven signals, clusters, and I/O pins, to the physical limits of the device.  If a LIM file exists, the Partitioner limits fanin, signals, cluster, and I/O pins on a block-by-block basis as specified in the LIM file.

For a design file Design.PDS, the LIM file must have the name Design.LIM and reside in the same directory as the design file in order to be recognized by the Partitioner.  When a LIM file exists, the Partitioning section of the log file contains the following message:

Using Partitioning Control File Design.LIM.

The LIM file contains one or more block limit specifications, each of which takes the following general form:

BLOCK Block letter or range of letters
MAX_FANIN Value
MAX_SIGNAL Value
MAX_CLUSTER Value
MAX_IO_PIN Value

| ã | Note: Comments are not permitted. Blank lines are ignored. |
| --- | --- |

# Syntax

The LIM file must be an ASCII text file containing only alphanumeric characters.  The LIM file is not case-sensitive (that is, "BLOCK," "Block," and "block" are equivalent).

## BLOCK Statement

The set of parameters for each block or group of blocks must begin with the BLOCK statement.  The BLOCK statement consists of a line containing the BLOCK keyword followed by the letter designations of one or more blocks.  Subsequent lines contain the parameter settings to be applied to the specified block or blocks.

Example (Limits applied to Block A only)

BLOCK A
MAX_FANIN	30
MAX_SIGNAL	15
MAX_CLUSTER	15
MAX_IO_PIN	8

Ranges of letter designations (ascending or descending) are allowed in the BLOCK statement.  A range is defined by a letter designator followed by two or more periods and another letter designator.

Example (Limits applied to Blocks C, B, and A)

BLOCK C..A
MAX_FANIN	30
MAX_SIGNAL	15
MAX_CLUSTER	15
MAX_IO_PIN	8

BLOCK commands can include both individual block letter designations and ranges.

Example (Limits applied to Block A and Blocks F, G, and H)

BLOCK A F..H
MAX_FANIN	30
MAX_SIGNAL	15
MAX_CLUSTER	15
MAX_IO_PIN	8

| ã | Note: If the LIM file contains multiple sets of parameters for the same block or group of blocks, the last set of parameters is used. No warning is generated. |
| --- | --- |

## Parameters

Each of the four valid parameters has an acceptable range of values (device specific).

| ã | Note: If no value is given or if invalid values are given, the Partitioner uses the physical limit of the device. No warning is generated. If multiple parameters of the same type are given for the same block or group of blocks, the last value is used. No warning is generated. |
| --- | --- |

The acceptable range of values for each parameter is given in the following table.

| Parameter | Acceptable Values |
| --- | --- |
| MAX_FANIN | 0-33 |
| MAX_SIGNAL | 0-16 |
| MAX_CLUSTER | 0-16 |
| MAX_IO_PIN | 0-8 |

All MACH 3xx/4xx devices except MACH465.  For MACH465 devices, the acceptable range of values is 1-34.