| 10 | Device Reference |
| --- | --- |

Contents

µMACH Family Features Summary	377

MACH Features Locator, Part 1	378

MACH Features Locator, Part 2	379

MACH 1xx/2xx Design Considerations	380

Product Term Cluster Steering	380

Default Clock	380

XOR with D-Type Flip-Flops	381

T-Type Flip-Flops	381

Latches	382

MACH 1xx Latch Emulation	382

MACH 2xx Hardware Latches	383

Registered Inputs	384

Node Feedback vs. Pin Feedback	387

Registered Output with Node Feedback or Pin Feedback	388

Combinatorial Output with Node Feedback 
or Pin Feedback	391

Global Set and Reset	392

PAL22V10-Compatible Set/Reset Behavior	393

MACH 1xx/2xx Power-Up	393

Synchronous vs. Asynchronous Operation	393

Powerdown Feature	393

MACH 3xx/4xx Design Considerations	394

Cluster Size	394

Default Clock	395

XOR with D-Type Flip-Flops	395

T-Type Flip-Flops	396

Latches	399

MACH 3xx/4xx Hardware Latches	399

MACH 2xx/3xx/4xx vs. MACH 1xx 
Latch Implementation	400

Registered Inputs (MACH 4xx Devices Only)	401

Zero Hold Time for Input Registers	402

Node vs. Pin Feedback	403

Registered Output with Node Feedback or Pin Feedback	404

Combinatorial Output with Node Feedback 
or Pin Feedback	407

Flexible Clock Generator	408

Global Set and Reset	409

Set/Reset Compatibility	410

PAL22V10 Register Behavior	411

Controlling MACH 3xx/4xx Set/Reset Behavior	412

Set/Reset in MACH 3xx/4xx Asynchronous Macrocells	413

Higher Block Utilization with the Set/Reset 
Selector Fuse	414

MACH 3xx/4xx Power-Up	415

MACH 3xx/4xx Asynchronous Macrocell 
Power-Up Operation	416

Set/Reset Design Recommendations	416

Synchronous vs. Asynchronous Operation	417

Synchronous Mode	418

Asynchronous Mode	419

Forcing Configuration as a Synchronous Macrocell	419

Cross-Programming MACH435 Designs 
to the MACH445 Device	421

MACH110 Pin and Node Summary	423

MACH111 Pin and Node Summary	425

MACH120 Pin and Node Summary	427

MACH130 Pin and Node Summary	430

MACH131 Pin and Node Summary	433

MACH210 Pin and Node Summary	436

MACH211 Pin and Node Summary	438

MACH215 Pin and Node Summary	440

MACH220 Pin and Node Summary	442

MACH231 Pin and Node Summary	445

MACH355 Pin and Node Summary	448

MACH435 Pin and Node Summary	453

MACH445 Pin and Node Summary	456

MACH465 Pin and Node Summary	460

# MACH Family Features SummaryXE "MACH family features summary"§

| MACH Family | Implements (S)ync only (A)sync only (B)oth | Output Latches | Input Latch/Reg | Max # PT Without Gate Splitting |
| --- | --- | --- | --- | --- |
| MACH1xx | S |  |  | 12 |
| MACH 2xx (except MACH215) | S | 4 | 4d | 16 |
| MACH215 | Bb | 4 | 4e | 12 |
| MACH 3xx | Ba,b,c | 4 |  | 20 |
| MACH 4xx | Ba,b,c | 4 | 4e | 20 |

Notes to Family Features Table

a.	In asynchronous mode, the macrocell register can be clocked by a pin clock.

b.	In asynchronous mode, the macrocell register can be clocked by a product term clock.

c.	In asynchronous mode, the macrocell register has only one register control product term that can be used either for SET or RESET.

d.	I/O pins on these devices use the internal macrocells as input registers or latches through a direct connection.

e.	I/O pins on these devices have dedicated input registers/latches.

# MACH Features Locator, Part 1XE "features locator"§

| MACH Device | # Inputs, Outputs, I/Os | # Pins | Implements (S)ync (A)sync (B)oth | Output Latches | Input Latch/Reg | PT Cluster Size | Individual OE Control | Max Number of PT Clusters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MACH110 | 38 | 44 | S |  |  | 4 | See note g | 3 |
| MACH111 | 38 | 44 | S |  |  | 4 | See note g | 3 |
| MACH120 | 56 | 68 | S |  |  | 4 | See note g | 3 |
| MACH130 | 70 | 84 | S |  |  | 4 | See note g | 3 |
| MACH131 | 70 | 84 | S |  |  | 4 | See note g | 3 |
| MACH210 | 38 | 44 | S | 4 | 4d | 4 | See note g | 4 |
| MACH211 | 38 | 44 | S | 4 | 4d | 4 | See note g | 4 |
| MACH215 | 38 | 44 | Bb | 4 | 4e | 4 | 4 | 3 |
| MACH220 | 56 | 68 | S | 4 | 4d | 4 | See note g | 4 |
| MACH231 | 70 | 84 | S | 4 | 4d | 4 | See note g | 4 |
| MACH355 | 102 | 144 | Ba,b,c | 4 |  | 5f | 4 | 4 |
| MACH435 | 70 | 84 | Ba,b,c | 4 | 4e | 5f | 4 | 4 |
| MACH445 | 70 | 100 | Ba,b,c | 4 | 4e | 5f | 4 | 4 |
| MACH465 | 142 | 208 | Ba,b,c | 4 | 4e | 5f | 4 | 4 |

Notes to Features Locator Table, Part 1

a.	In asynchronous mode, the macrocell register can be clocked by a pin clock.

b.	In asynchronous mode, the macrocell register can be clocked by a product term clock.

c.	In asynchronous mode, the macrocell register has only one register control product term that can be used either for SET or RESET.

d.	I/O pins on these devices use the internal macrocells as input registers or latches through a direct connection.

e.	I/O pins on these devices have dedicated input registers/latches.

f.	The product term cluster can be used as a group of four product terms logically XORed with the one XOR product term or as a single group of five product terms.  In the asynchronous mode, the number of product terms in the cluster is reduced by two (that is, two product terms are reserved for SET, RESET, and CLOCK functions).  The cluster can be steered to adjacent macrocells to allow the adjacent macrocell to implement a larger equation.  If not needed by an adjacent macrocell, a single product term can remain at the original macrocell to implement an equation consisting of one product term.  Refer to the "Cluster Size" section in this chapter for more information.

g.	I/O pins are enabled in banks (see the device logic diagram in the data sheet for more information).

# MACH Features Locator, Part 2XE "features locator"§

| MACH Device | XOR Gate | 5 Volt In-Circuit Programing | Output MUXes | Input MUXes | Maximum Number of Inputs Into a Logic Block | Number of Clock Pins (Default Clock Pin) | Special Function Fuses |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MACH110 |  |  |  |  | 22 | 2 (35) |  |
| MACH111 |  |  |  |  | 26 | 4 (35) | 4i |
| MACH120 |  |  |  |  | 26 | 4(50) |  |
| MACH130 |  |  |  |  | 26 | 4 (65) |  |
| MACH131 |  |  |  |  | 26 | 4 (65) | 4i |
| MACH210 |  |  |  |  | 22 | 2 (35) |  |
| MACH211 |  |  |  |  | 26 | 4 (35) | 4i |
| MACH215 |  |  |  |  | 22 | 2 (13) |  |
| MACH220 |  |  |  |  | 26 | 4 (50) |  |
| MACH231 |  |  |  |  | 26 | 4(65) | 4i |
| MACH355 | 4 | 4 | 4 | 4 | 33 | 4(8) | 4i |
| MACH435 | 4 |  | 4 | 4 | 33 | 4 (20) |  |
| MACH445 | 4 | 4 | 4 | 4 | 33 | 4 (13) | 4h,j |
| MACH465 | 4 | 4 | 4 | 4 | 34 | 4 (187) | 4h,j |

Notes to Features Locator Table, Part 2

h.	Special function fuse: Zero Hold Time Fuse

i.	Special function fuse: Power-Down (Slew Rate Control) Fuse

j.	Signature Fuse

# MACH 1xx/2xx Design ConsiderationsXE "MACH 1xx/2xx:design considerations"§

The following sections contain information that is specific to the MACH 1xx and 2xx devices.

## Product Term Cluster SteeringXE "MACH 1xx/2xx:product term cluster steering"§XE "product term steering"§

Each macrocell is associated with a cluster of four product terms.  However, the macrocell's cluster of product terms can be steered, by the Fitter, to an adjacent macrocell to allow that macrocell to implement equations that have more than four product terms.  For most MACH 1xx macrocells, the maximum number of product terms that can be implemented is 12 in the: the 4 original product terms plus 4 product terms from the adjacent macrocells on either side.  The first and last macrocells in a block have only one adjacent macrocell and can consequently implement equations of eight or fewer product terms.

The MACH 2xx family can implement up to 16 product terms per equation.1  The first and last macrocells in a MACH 2xx block can implement equations of 12 or fewer product terms.

## Default ClockXE "MACH 1xx/2xx:default clock"§XE "default clock"§

The MACHXL software uses the default clock pin to clock any register for which you do not specify a clock signal.  The default clock pin for each device is listed in the "Pin and Node Summary" section at the end of this chapter.  In general, it is best to specify clock signals for all registers explicitly.

If you place a clock signal at the default clock pin and also use the default clock to clock registers for which no .CLKF equations are written, the Fitter will merge the named clock signal and the default clock signal.  (Note the difference between this and MACH 3xx/4xx behavior, described under "Default Clock" in the MACH 3xx/4xx Design Considerations" section.)

## XOR with D-Type Flip-FlopsXE "MACH 1XX/2XX:XOR with D-type flip-flops"§XE "XOR"§

The MACH 1xx/2xx devices do not contain hardware XOR capability.  Designs specifying XOR equations are converted to sum-of-products equations during minimization.  This usually increases the number of product terms required to implement the design.

## T-Type Flip-FlopsXE "MACH 1XX/2XX:T-type flip-flops"§XE "flip-flops:T-type"§

The MACH 1xx/2xx devices have programmable polarity after the macrocell.  For this reason, the implementation of active-low T-type equations differs in form, but not in functionality, from that of MACH 3xx/4xx devices.

Consider these complementary equations:

;active high

O1.T = IN1 * IN2

O1.SETF = SET_A

;active low

/O2.T = IN1 * IN2 	;different logic from active high

O2.SETF = SET_A	;but same initialization equation

When initialized, the MACH1xx/2xx device's active-high output goes high and the active-low output goes low.  Each time the equation IN1 * IN2 becomes true, both outputs change state.

The following figure shows how the complementary equations are implemented on MACH 1xx/2xx devices that have inverters after the macrocell.

µ §

## LatchesXE "MACH 1XX/2XX:latches"§XE "latches"§

MACH 1xx and 2xx devices implement latches differently.

### MACH 1xx Latch EmulationXE "latch emulation, MACH 1xx"§

MACH1xx devices have no hardware latches; you must implement latches using combinatorial logic, as shown in the following example.

;************************************************************

TITLE 		Latch Design File

PATTERN 	Latch.PDS

REVISION 	1.1

AUTHOR 		J. Engineer

COMPANY 	ADVANCED MICRO DEVICES, INC.

DATE 		9/16/93

CHIP Ltch_Tst  MACH111

;************************************************************

PIN 20 LE

PIN 5 RST

PIN 4 SET

...

PIN 8 Q2

PIN 3 D

;************************************************************

Continued...

...Continued

EQUATIONS

;************************************************************

...Minimize_off ; must be off to retain redundant logic

Q2 = D   *  LE             ; Loading Term - LE dominates

+ D   *  Q2 * /RST      ; Transient race - redundant logic

+ Q2  * /LE * /RST      ; Data Holding term

+ SET * /LE * /RST      ; Set term - LE & RST dominate

Minimize_on

In order to prevent the "extra cover" product term from minimizing out, you must add MINIMIZE_OFF and MINIMIZE_ON statements around the equations used to implement latches as shown here.  These MINIMIZE_OFF and MINIMIZE_ON statements are only required when you implement latches using combinatorial logic; they are not required when you use the hardware latches available on MACH 2xx/3xx/4xx devices.

### MACH 2xx Hardware LatchesXE "MACH 2xx:latches"§XE "latches"§

MACH 2xx hardware latches differ slightly from MACH 3xx/4xx hardware latches.  However, the MACHXL software automatically compensates for this hardware difference.  You do not need to write equations differently for MACH 2xx and MACH 3xx/4xx devices, except that the .CLKF equation for MACH 2xx latches must specify an active-low clock as shown in the example below:

OUT2.CLCF = /CLK

or

/OUT2.CLKF = CLK

| ã | Note: MACH 2xx devices do not support all combinations of Set, Reset, and clock signals. The following combinations are illegal: RESET=0, SET=1, LE=0 RESET=1, SET=0, LE=0 RESET=1, SET=1, LE=0 On the MACH210 device, the following combination is also illegal: RESET=1, SET=1, LE=1 |
| --- | --- |

The following table describes the behavior of MACH 2xx devices when programmed with the JEDEC file produced by MACHXL software.

| Active Low clock equation? | If LE input pin level is: | Then latch assumes state: |
| --- | --- | --- |
| Yes | Low | Transparent |
| Yes | High | Latched |
| No | Illegal | N/A |
| No | Illegal | N/A |

## Registered InputsXE "MACH 1xx/2xx:inputs:registered"§XE "registered inputs"§XE "inputs:registered"§

MACH 1xx devices have no provision for registering or latching input signals without routing the input through the AND/OR array.  You can emulate an input register using an output register as shown in the following design example.

...
CHIP	EMULATED_INPUT_REG	MACH110
...
PIN	?	IN1
PIN	?	IN2
PIN	?	CLOCK
NODE	?	INODE	REGISTERED    ;DEFINES NODE AS REGISTERED
PIN	?	OUT2
EQUATIONS

Continued...

...Continued

INODE      = IN1		;INODE REGISTERS THE INPUT SIGNAL
INODE.CLKF = CLOCK

OUT2       = INODE * IN2	;OUT2 USES THE REGISTERED SIGNAL

The following figure shows how the preceding design example is implemented.

µ §

Note that this technique results in an extra propagation delay through the AND/OR array and precludes further use of the macrocell register at which signal INODE is placed.

MACH 2xx (except MACH215) devices are similar to MACH 1xx devices in that they do not have dedicated input registers.  However, MACH 2xx devices can route input signals directly to a macrocell without passing through the AND/OR array, saving a propagation delay.

The following figure shows how the MACH 2xx device saves a propagation delay when implementing the same design example shown above for the MACH 1xx example.

µ §

The input registers in the MACH215 device are connected directly to the pin when the design specifies an input register, as illustrated in the following design example.

...
CHIP	INPUT_REG	MACH215
...
PIN	?	IN1	PAIR	IR1	;SPECIFIES INPUT PAIRING
PIN	?	IN2
PIN	?	CLOCK
PIN	?	OUT1	COMBINATORIAL

NODE	?	IR1	REGISTERED	;INPUT REGISTER

EQUATIONS

IR1      = IN1		;ASSIGNS PIN VALUE TO INPUT REGISTER
IR1.CLKF = CLOCK

OUT1     = IR1 * IN2

The following figure illustrates the registered input used in the preceding design example.

µ §

| ã | Note: In the MACH215 device, the input register is a separate flip-flop from the buried macrocell used in other MACH 2xx devices and thus does not have individual set or reset controls. Refer to "Pairing" in Chapter 7 for additional information. |
| --- | --- |

## Node Feedback vs. Pin FeedbackXE "MACH 1XX/2XX:node feedback vs. pin feedback"§XE "feedback"§

MACH devices support two types of feedback:

r	NODE feedback routed from the Q output of the flip-flop associated with the pinXE "node feedback"§XE "feedback:node"§

r	PIN feedback directly from the pinXE "pin feedback"§XE "feedback:pin"§

MACH 1xx/2xx devices have an output polarity mux after the register, so you can specify different polarity for the pin and the node.  If the pin/node name is uncomplemented in the PIN/NODE statement, then uncomplemented pin feedback has the same polarity as the pin and uncomplemented node feedback has the same polarity as the node.

Feedback signals are routed as follows (to emulate PAL22V10 behavior):

r	Feedback from any unpaired output pin defined as registered is routed from the node.

r	Feedback from any unpaired output pin defined as combinatorial is routed from the pin.

You can specify two additional types of feedback routing:

r	Node feedback from a pin defined as combinatorial.

r	Registered pin feedback from a pin defined as registered.

The following sections describe each of the possible feedback routings in detail.

### Registered Output with Node Feedback or Pin FeedbackXE "MACH 1XX/2XX:registered output with node feedback or pin feedback"§

In the following example, the register associated with pin OUT1 is not specified in the design, but is implied by the fact that the MACH 1xx/2xx device emulates the PAL22V10.XE "registered output"§XE "output:registered"§

The register associated with pin OUT2 is explicitly specified with a PAIR statement.  Note that when you explicitly pair a pin and a node, the default feedback routing no longer applies.  In this case, you must specify feedback from the node as shown in the design example, otherwise feedback comes from the pin.  The feedback signals from pin OUT2 are routed both ways to illustrate how to specify each type of feedback.

...
CHIP NODE_FB MACH210
...
PIN	?	IN1
PIN	?	IN2

PIN	?	IN3

PIN	?	IN4

PIN	?	RESET

PIN	?	SET
PIN	?	CLOCK

PIN	?	OE

Continued...

...Continued

PIN	?	OUT1  REGISTERED 	
PIN	?	OUT2  REGISTERED

PIN	?	OUT3  COMBINATORIAL

PIN	?	OUT4  COMBINATORIAL	
PIN	?	OUT5  COMBINATORIAL

NODE	?	BR2   REGISTERED PAIR OUT2 ;SPECIFIES OUTPUT PAIRING

EQUATIONS

;----- ILLUSTRATES IMPLIED BEHAVIOR (DEFAULT) ------------------
OUT1      = IN1        ;SINCE NO NODE IS SPECIFIED, THE MACHXL

;SOFTWARE ASSIGNS ONE (BECAUSE PIN WAS

;DEFINED AS REGISTERED)

OUT3      = IN2 * OUT1 ;BY DEFAULT, FEEDBACK FROM A PIN PAIRED

;WITH A NODE BY IMPLICATION (AS WAS PIN

;OUT1) IS ROUTED FROM THE NODE
OUT1.CLKF = CLOCK

OUT1.TRST = OE

;----- ILLUSTRATES EXPLICITLY-SPECIFIED BEHAVIOR ---------------

OUT2      = OUT1

BR2       = {OUT2}	 ;SURROUNDING THE PIN NAME IN BRACES {}

;COPIES THE EXACT EXPRESSION FROM THE

;RIGHT SIDE OF THE PIN'S EQUATION TO THE

;NODE, SO YOU DON'T HAVE TO RE-TYPE IT.

BR2.CLKF  = CLOCK

OUT2.TRST = OE

OUT4      = IN4 * BR2	 ;YOU MUST USE THE NODE NAME TO GET

;NODE FEEDBACK

OUT5      = IN5 * OUT2 ;IF YOU USE THE PIN NAME, YOU GET FEEDBACK

;FROM THE PIN

The following figure illustrates the default behavior of pin OUT1 as well as the explicitly-specified behavior of pin OUT2.

µ §

### Combinatorial Output with Node Feedback or Pin FeedbackXE "MACH 1XX/2XX:combinatorial output with node feedback or pin feedback"§

When no output pair is declared or generated, only pin feedback is available from pins declared as combinatorial.  When declared as part of an output pair, node feedback is also available (node feedback from a combinatorial pin remains available regardless of the state of the pin's output buffer).XE "combinatorial output"§XE "output:combinatorial"§  The following design example defines an equation (OUT1 =IN1 * IN2) and routes feedback as follows:

r	from the node to output OUT3

r	from the pin to output OUT2

...
CHIP	PIN_FB	MACH210
...
PIN	?	IN1
PIN	?	IN2

PIN	?	IN3

PIN	?	IN4

PIN	?	OE

PIN	?	OUT1	COMBINATORIAL
PIN	?	OUT2	COMBINATORIAL

PIN	?	OUT3	COMBINATORIAL

NODE	?	OUT_N1	COMBINATORIAL  PAIR  OUT1

EQUATIONS
OUT1      =	IN1    * IN2

OUT1.TRST =	OE

OUT2      =	OUT1   * IN3

OUT3	   =	OUT_N1 * IN4

The following figure illustrates the pin and node feedback from OUT1 described in the preceding design example.

µ §

## Global Set and ResetXE "MACH 1XX/2XX:global set and reset"§XE "global set and reset"§XE "set:global"§XE "reset:global"§

A global node is available to specify set and reset behavior for all synchronous macrocells in the MACH device.  In all MACH devices, node 1 is the global node.  Note that the global node, which is implemented in software, does not correspond to a physical location in the device.

To use the global node, define it in the pin/node declaration portion of the design file as follows:XE "set:global"§XE "reset:global"§XE "global set and reset"§

NODE  1  User_defined_name

Then write a .SETF and/or a .RSTF functional equation to control the corresponding global functions.  (Each global equation must consist of a single product term.)

If you write global .SETF and .RSTF equations and also write individual .SETF and .RSTF equations for one or more macrocells, the global .SETF and .RSTF equations take precedence.

## PAL22V10-Compatible Set/Reset BehaviorXE "MACH 1XX/2XX:PAL22V10-compatible register behavior"§XE "PAL22V10 compatibility"§

The level at the pin connected to a PAL22V10 macrocell after a set, reset, or power-up operation is determined by the pin's polarity, as shown in the following table. Note that the power-up reset line is active only when power is initially applied to the part. If the reset product term is active and the set product term is inactive, an active-high pin goes low, while an active-low pin goes high.XE "PAL22V10 compatibility"§

Levels detected at pins on Set, Reset, and Power-Up for PAL22V10 macrocells

| SET (PT0) | RESET (PT1) | Power-Up RESET | Level at Pin if Macrocell Active High | Level at Pin if Macrocell Active Low |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | L | H |
| 1 | 0 | 0 | H | L |
| 0 | 0 | 1 | L | H |

## MACH 1xx/2xx Power-UpXE "MACH 1XX/2XX:power-up"§XE "power-up"§

XE "power-up"§MACH 1xx/2xx devices have a power-up register initialization feature that forces active-high registers low and active-low registers high when power is applied (see the device data sheet for guidelines).

## Synchronous vs. Asynchronous OperationXE "MACH 1XX/2XX:synchronous vs. asynchronous operation"§XE "synchronous operation"§XE "asynchronous operation"§

XE "macrocells:asynchronous"§XE "macrocells:synchronous"§XE "synchronous macrocells"§XE "asynchronous macrocells"§The MACH 1xx/2xx (other than the MACH215 device) are always synchronous, use common clock pins, and do not support product-term clocks.

The MACH215 device supports synchronous or asynchronous logic.  It has one common clock pin and supports one product-term clock per output macrocell.  Input registers use either of the two global clocks.

## PowerdownXE "powerdown"§XE "E-field"§ Feature

MACH111, MACH131, MACH211, and MACH231 devices have the ability to power down unused macrocells and other macrocells specified in the design file.  Each powered-down macrocell is represented in the JEDEC file by an E-field bit set to 1.

The JEDEC file will contain E-field bit(s) set to 1 only under the following circumstances:

r	All unused cells are set to 1.

r	All cells used for input are set to 1.

r	All cells listed in the GROUP LOW_POWER_LIST list are set to 1.

All output cells not listed in the GROUP LOW_POWER_LIST list are set to 0.

See the LOW_POWER_LIST entry in Chapter 5, "Language Reference."

Powered-down macrocells reduce a device's overall power consumption.  In addition, a macrocell's slew rate is altered by the power-down state.  Refer to the device data sheet for specific information on the slew rate.

# MACH 3xx/4xx Design ConsiderationsXE "MACH 3xx/4xx:design considerations"§

The following sections contain information that is specific to the MACH 3xx and 4xx devices.

## Cluster SizeXE "MACH 3xx/4xx:cluster size"§XE "cluster size"§

Each MACH 3xx/4xx macrocell is associated with a "cluster" of five product terms that is available for various uses.  These five product terms (three in asynchronous-mode macrocells) available to implement logic equations or to be steered to adjacent macrocells.  Depending on a number of factors, the size of clusters available for steering can contain 2, 3, 4, or 5 product terms.

The XOR product term can be used to implement XOR logic or as a regular product term.  If a single-product term equation is implemented using the macrocell's XOR product term, the cluster from that macrocell that is available for steering to adjacent macrocells is reduced by one product term.

The nominal cluster size of synchronous and asynchronous macrocells is as follows:

r	Synchronous macrocells begin with five product terms available to implement logic equations.

r	Asynchronous macrocells begin with three product terms available to implement logic equations.  This is true because one product term is reserved for use as a product-term clock and another is reserved for use as a Set or Reset product term.

## Default ClockXE "MACH 3xx/4xx:default clock"§XE "default clock"§

The MACHXL software uses the default clock pin to clock any register for which you do not specify a clock signal.  For example, pin 20 is the default clock pin for the MACH435 device.  In general, it is best to specify clock signals for all registers explicitly.  If you want to have the MACHXL software use the default clock automatically, do NOT write a pin statement for the default clock pin or the compiler will report an error.    (Note the difference between this and MACH 1xx/2xx behavior, described under "Default Clock" in the MACH 1xx/2xx Design Considerations" section.)

## XOR with D-Type Flip-FlopsXE "MACH 3XX/4XX:XOR with D-type flip-flops"§XE "XOR"§

MACH 3xx and 4xx devices have an XOR gate for each product-term cluster.  The XOR gate can be used as follows:

r	To implement XOR logic without using extra product terms

r	To control output polarity before the register

The following design example illustrates the use of the XOR gate.

...
CHIP	XOR	MACH435
...
PIN	?	IN1
PIN	?	IN2
PIN	?	RESET_A
PIN	?	SET_A
PIN	?	CLOCK
PIN	?	OUT	REGISTERED

EQUATIONS

OUT = IN1 :+: IN2	; THE :+: OPERATOR SPECIFIES XOR
OUT.CLKF = CLOCK
OUT.RSTF = RESET_A
OUT.SETF = SET_A

The following figure illustrates the XOR gate as used in the preceding design example.

µ §

## T-Type Flip-FlopsXE "MACH 3XX/4XX:T-type flip-flops"§XE "flip-flops:T-type"§

The MACH 3xx/4xx devices do not have an inverter after the macrocell, unlike MACH 1xx/2xx devices.  For this reason, the implementation of active-low T-type equations differs in form, but not in functionality, from that of MACH 1xx/2xx devices.

Consider these complementary equations:

;active high

O1.T = IN1 * IN2

O1.SETF = SET_A

;active low

/O2.T = IN1 * IN2 	;different logic from active high

O2.SETF = SET_A	;but same initialization equation

The figure below shows how the complementary equations are implemented on a device, such as the MACH 1xx/2xx, that has an inverter after the macrocell.

µ §

The MACH 3xx/4xx devices have no inverter after the macrocell, so they must implement the same equation for both the active-high and active-low forms.  The MACHXL software routes the initialization signal (in this example, SET_A) to the Set or Reset line to produce, respectively, the equivalents of active-high or active-low behavior.

The following figure shows how the same complementary equations are implemented on a MACH 3xx/4xx device:

µ §

These equations, as implemented on the MACH 3xx/4xx device, can be expressed as follows:

;active high

O1.T = IN1 * IN2

O1.SETF = SET_A

;active low

O2.T = IN1 * IN2 	;same logic as active high

O2.RSTF = SET_A	;but different initialization equation

The rules for handling T-type flip-flops can be summarized as follows:

r	For T-type flip-flops that feed active-low I/O pins, Set and Reset are always swapped.

r	For active-low T-type flip-flops implemented on buried nodes, references to the buried node are always inverted (regardless of the 22V10/MACH1xx/2xx S/R Compatibility option setting).

Example
Feedback from the active-low T-type signal TSIG (implemented on a buried node) is referenced as TSIG on the right side of an equation for the signal OUT2.  In the JEDEC file, the reference to TSIG appears as /TSIG.

| ã | Note: After disassembling a JEDEC file, references to TSIG will appear as /TSIG. |
| --- | --- |

## LatchesXE "MACH 3XX/4XX:latches"§XE "latches"§

MACH 2xx/3xx/4xx devices implement latches in hardware.  MACH1xx devices require that you implement latches using combinatorial logic.

### MACH 3xx/4xx Hardware LatchesXE "MACH 3XX/4XX:hardware latches"§XE "latches:hardware"§

The following design example illustrate the use of latches.

CHIP	LATCH	MACH435
...
PIN	?	IN1
PIN	?	IN2
PIN	?	RESET_A
PIN	?	SET_A
PIN	?	CLOCK
PIN	?	OUT	LATCHED

EQUATIONS

 OUT      = IN1 * IN2
/OUT.CLKF = CLOCK

OUT.RSTF = RESET_A

OUT.SETF = SET_A

The following figure illustrates the latch as used in the preceding design example.

µ §

| ã |  | Note: The MACH 3xx/4xx devices implement latches in hardware differently from the MACH 2xx devices, but the MACHXL software automatically compensates for this hardware difference. You do not need to write equations differently for MACH 2xx and MACH 3xx/4xx devices. In the MACH 3xx/4xx devices, hardware latches are transparent when the LE input is high. However, the MACHXL software automatically inverts the LE input, so that equations written for MACH 2xx devices behave the same on the MACH 3xx/4xx devices. The following table describes the behavior of MACH 3xx/4xx latches when using MACHXL software. |  |  |
| --- | --- | --- | --- | --- |
|  | Slash before .CLKF equation? |  | If LE input pin level is: | Then latch assumes state: |
|  | Yes |  | Low | Transparent |
|  | Yes |  | High | Latched |
|  | No |  | Low | Latched |
|  | No |  | High | Transparent |

### MACH 2xx/3xx/4xx vs. MACH 1xx Latch ImplementationXE "MACH 2xx/3XX/4XX vs. MACH1xx latch implementation"§XE "latches:implementation"§

When converting MACH1xx designs containing latch logic to MACH 2xx/3xx/4xx designs, take advantage of the MACH 2xx/3xx/4xx device's hardware latches.

The following design example shows how to implement latch behavior using the MACH 3xx/4xx hardware latches.

;************************************************************

TITLE 		Latch Design File

PATTERN 	Latch.PDS

REVISION 	1.1

AUTHOR 		J. Engineer

COMPANY 	ADVANCED MICRO DEVICES, INC.

DATE 		9/16/93

Continued...

...Continued

CHIP Ltch_Tst  MACH435

;************************************************************

PIN 20 LE

PIN 5 RST

PIN 4 SET

PIN 9 Q1	LATCHED

PIN 3 D

;************************************************************

EQUATIONS

;************************************************************

; With explicit S/R equations "Native" Latch in Macrocell

Q1 = D

Q1.CLKF  =  LE

Q1.RSTF  =  RST

Q1.SETF  =  SET

## Registered Inputs (MACH 4xx Devices Only)XE "MACH 4xx:registered inputs"§XE "inputs:registered"§XE "registered inputs"§

The input registers in MACH 4xx devices are connected directly to the pin, as illustrated in the following design example.  (MACH 3xx devices do not have input registers.)

...
CHIP	INPUT_REG	MACH435
...
PIN	?	IN1	PAIR  INODE
PIN	?	IN2
PIN	?	CLOCK
PIN	?	OUT2	COMBINATORIAL

NODE	?	INODE	REGISTERED ;DEFINES INPUT REGISTER

EQUATIONS

INODE      = IN1		;ASSIGNS PIN VALUE TO INPUT REGISTER
INODE.CLKF = CLOCK

OUT2       = IN1 * IN2

The following figure illustrates the registered input as used in the preceding design example.

µ §

| ã | Note: In MACH 4xx devices, the input register is a separate flip-flop from the buried macrocell used in the MACH 2xx device and thus does not have individual set or reset controls. Refer to the "Pairing" section in Chapter 7 for additional information. |
| --- | --- |

### Zero Hold Time for Input RegistersXE "zero hold time for input registers"§

All MACH 4xx devices (except the MACH435) have a zero hold time (ZHT) fuse. This fuse controls the time delay associated with the data path to all input registers and latches in MACH 4xx devices (except the MACH435).

The ZHT fuse gives you the ability to control the hold time requirements of the input register.  If the fuse is programmed, the input register has the same timing characteristics as an input register in devices without the ZHT option, such as the MACH435 device.  If the fuse is not programmed, the input register requires a longer setup time while needing no hold time.

µ §

µ §

When programmed, the ZHT fuse increases the data path delays to input storage elements, matching equivalent delays in the clock path. When the fuse is erased, the setup time required by the input storage element is minimized and the device timing is compatible with the MACH435 device.

This feature facilitates doing worst-case designs for which data is loaded from sources which have low (or zero) minimum output propagation delays from clock edges. It also simplifies datapath interfaces to microprocessor and other devices which provide data late in the clock cycle and do not hold data constant long into the next clock cycle. See the MACH Family Device Data Book for more details.

## Node vs. Pin FeedbackXE "MACH 3XX/4XX:node vs. pin feedback"§XE "feedback"§

MACH 3xx/4xx devices support two types of feedback:

r	NODE feedback routed from the Q output of the flip-flop associated with the pinXE "node feedback"§XE "feedback:node"§

r	PIN feedback directly from the pinXE "pin feedback"§XE "feedback:pin"§

By default, feedback signals are routed as follows (to emulate PAL22V10 behavior):

r	Feedback from any unpaired output pin defined as registered is routed from the node.

r	Feedback from any unpaired output pin defined as combinatorial is routed from the pin.

You can specify two additional types of feedback routing:

r	Node feedback (before the three-state output buffer) from a pin defined as combinatorial.

r	Registered pin feedback from a pin defined as registered.

The following sections describe each of the possible feedback routings in detail.

### Registered Output with Node Feedback or Pin FeedbackXE "MACH 3XX/4XX:registered output with node feedback or pin feedback"§

In the following example, the register associated with pin OUT1 is not specified in the design, but is implied by the fact that the MACH 3xx/4xx device emulates the PAL22V10 by default.XE "registered output"§XE "output:registered"§

The buried register associated with pin OUT2 is explicitly specified with a PAIR statement.  Note that when you explicitly pair a pin and a node, the default feedback routing no longer applies.  In this case, you must specify feedback from the node as shown in the design example, otherwise feedback comes from the pin.  The feedback signals from pin OUT2 are routed both ways to illustrate how to specify each type of feedback.

CHIP NODE_FB MACH435
PIN	?	IN1
PIN	?	IN2

PIN	?	IN3

PIN	?	IN4

PIN	?	RESET

PIN	?	SET
PIN	?	CLOCK

PIN	?	OUT1	REGISTERED
PIN	?	OUT2	REGISTERED

PIN	?	OUT3	COMBINATORIAL

PIN	?	OUT4	COMBINATORIAL
PIN	?	OUT5	COMBINATORIAL

NODE	?	BR2	REGISTERED PAIR OUT2 ;DEFINES OUTPUT PAIRING

Continued...

...Continued

EQUATIONS
;----- ILLUSTRATES IMPLIED BEHAVIOR (DEFAULT) ------------------
OUT1 = IN1             ;SINCE NO NODE IS SPECIFIED, THE MACHXL

;SOFTWARE ASSIGNS ONE (BECAUSE PIN WAS

;DEFINED AS REGISTERED)

OUT3 = IN2 * OUT1      ;BY DEFAULT, FEEDBACK FROM A PIN PAIRED

;WITH A NODE BY IMPLICATION (AS WAS PIN

;OUT1) IS ROUTED FROM THE NODE
OUT1.CLKF = CLOCK

;----- ILLUSTRATES EXPLICITLY-SPECIFIED BEHAVIOR ---------------

OUT2 = IN3

BR2 = {OUT2}           ;SURROUNDING THE PIN NAME IN BRACES {}

;COPIES THE EXACT EXPRESSION FROM THE

;RIGHT SIDE OF THE PIN'S EQUATION TO THE

;NODE, SO YOU DON'T HAVE TO RE-TYPE IT.

BR2.CLKF = CLOCK

OUT4 = IN4 * BR2       ;YOU MUST USE THE NODE NAME TO GET

;NODE FEEDBACK

OUT5 = IN5 * OUT2      ;IF YOU USE THE PIN NAME, YOU GET FEEDBACK

;FROM THE PIN

The following figure illustrates the default behavior of pin OUT1 as well as the explicitly-specified behavior of pin OUT2.

µ §

### Combinatorial Output with Node Feedback or Pin FeedbackXE "MACH 3XX/4XX:combinatorial output with node feedback or pin feedback"§

When no output pair is declared or generated, only pin feedback is available from pins declared as combinatorial.  When declared as part of an output pair, node feedback is also available (node feedback from a combinatorial pin remains available regardless of the state of the pin's output buffer).XE "combinatorial output"§XE "output:combinatorial"§  The following design example defines an equation (OUT1 =IN1 * IN2) and routes feedback as follows:

r	from the node to output OUT3

r	from the pin to output OUT2

...
CHIP	PIN_FB	MACH435
...
PIN	?	IN1
PIN	?	IN2

PIN	?	IN3

PIN	?	IN4

PIN	?	OE

PIN	?	OUT1	COMBINATORIAL
PIN	?	OUT2	COMBINATORIAL

PIN	?	OUT3	COMBINATORIAL

NODE	?	OUT_N1	COMBINATORIAL  PAIR  OUT1

EQUATIONS
OUT1      =	IN1    * IN2

OUT1.TRST =	OE

OUT2      =	OUT1   * IN3

OUT3	   =	OUT_N1 * IN4

The following figure illustrates the pin and node feedback from OUT1 described in the preceding design example.

µ §

## Flexible Clock GeneratorXE "MACH 3XX/4XX:flexible clock generator"§XE "clock generator, MACH 3xx/4xx"§XE "flexible clock generator"§

The MACH 3xx/4xx architecture contains a flexible clocking scheme for each PAL block.XE "clock:flexible generator"§XE "flexible clock generator"§  Each PAL block has its own clock generator that can provide up to four different clock signals to the block.  The process through which up to four global clock signals are made available to all macrocells in the block is commonly referred to throughout this user's guide as "the block clock mechanism."XE "block clock mechanism"§XE "clock:block mechanism"§XE "clock:flexible generator"§

Each PAL block's clock generator receives its clock signals from two sources:

r	The MACH 3xx/4xx device's four global clock pins

r	The complements of the four global clock pins

Therefore, eight clock signals are available to each PAL block's clock generator.

Each PAL block's clock generator can select the polarity of each clock signal as low-to-high or high-to-low.  Through the block clock mechanism, a total of four global clock signals can be made available to macrocells in a given block.  For example, if the signals assigned to the four global clock pins are named CLKA, CLKB, CLKC, and CLKD, each block's flexible clock generator can select a different set of four signals, as shown in the example below.

| Block | Signal at block clock 0 | Signal at block clock 1 | Signal at block clock 2 | Signal at block clock 3 |
| --- | --- | --- | --- | --- |
| A | CLKA | CLKB | CLKC | CLKD |
| B | CLKA | /CLKA | CLKC | CLKD |
| C | /CLKB | CLKA | CLKC | /CLKC |
| etc. |  |  |  |  |

All 16 macrocells in each PAL block (if synchronous), and all input registers, have access, through the block clock mechanism, to the block clocks 0 through 3.XE "block clock"§XE "clock:block"§  Asynchronous macrocells have access, through the block clock mechanism, to block clocks 0 and 1 only.  However, asynchronous macrocells can receive clock signals as product-term clocks, and hence can receive signals from any global clock pin that can route its signal through the central switch matrix.

## Global Set and ResetXE "MACH 3XX/4XX:global set and reset"§XE "global set and reset"§XE "set:global"§XE "reset:global"§

A global node is available to specify set and reset behavior for all synchronous macrocells in the MACH 3xx/4xx device.  In all MACH devices, node 1 is the global node.  Note that the global node, which is implemented in software, does not correspond to a physical location in the device.

To use the global node, define it in the pin/node declaration portion of the design file as follows:XE "set:global"§XE "reset:global"§XE "global set and reset"§

NODE  1  User_defined_name

Then write a .SETF and/or a .RSTF functional equation to control the corresponding global functions.  (Each global equation must consist of a single product term.)

If you write global .SETF and .RSTF equations and also write individual .SETF and .RSTF equations for one or more macrocells, the global  .SETF and .RSTF equations take precedence.

## Set/Reset CompatibilityXE "MACH 3XX/4XX:set/reset compatibility"§XE "set/reset compatibility"§

The Fitter Options form contains the following option: 22V10/MACH 1xx/2xx S/R compatibility.XE "set:compatibility"§XE "reset:compatibility"§XE "compatibility:set and reset"§

Set this option to "Y" when processing MACH 3xx/4xx designs to maintain set/reset compatibility with devices that have macrocells similar to the MACH1xx/2xx and the PAL22V10.  Setting this option to "Y" has another result: equations are minimized to have the same form and polarity that they would have if implemented on a PAL22V10 device.

The MACH 3xx/4xx macrocell controls polarity before the register, while the PAL22V10 controls polarity after the register, as shown in the figure below.  The Fitter achieves compatibility by programming the set/reset selector fuse.

µ §

The set/reset selector fuse swaps the register control terms so that the product terms PT0 and PT1 can be used as either set or reset control terms. By programming the set/reset selector fuse correctly, the MACH 3xx/4xx macrocell can emulate the set and reset behavior of a PAL22V10 register. This section describes how the Fitter determines the proper state of the set/reset selector fuse.

### PAL22V10 Register BehaviorXE "MACH 3XX/4XX:PAL22V10-compatible register behavior"§XE "PAL22V10 compatibility"§

The level at the pin connected to a PAL22V10 macrocell after a set, reset, or power-up operation is determined by the pin's polarity, as shown in the following table. Note that the power-up reset line is active only when power is initially applied to the part.  (See the "MACH 3xx/4xx Power-Up" section later in this chapter.)

If the reset product term is active and the set product term is inactive, an active-high pin displays a low level while an active-low pin displays a high level.XE "PAL22V10 compatibility"§

Levels detected at pins on Set, Reset, and Power-Up for PAL22V10 macrocells

| SET (PT0) | RESET (PT1) | Power-Up RESET | Level at Pin if Macrocell Active High | Level at Pin if Macrocell Active Low |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | L | H |
| 1 | 0 | 0 | H | L |
| 0 | 0 | 1 | L | H |

### Controlling MACH 3xx/4xx Set/Reset BehaviorXE "MACH 3XX/4XX:controlling MACH 3XX/4XX Set/Reset behavior"§

When moving designs with PAL22V10-type macrocell registers to the MACH 3xx/4xx, the Fitter software will assign the first reset logic term in the block to PT1 (which is logically ORed with the power-up reset line).  If you specified set/reset compatibility, the Fitter programs the Set/Reset selector fuse to produce PAL22V10-combatible behavior (see the following two tables).XE "registers:controlling Set and Reset"§

Set/Reset Selector Fuse State When Macrocell Register Has Active-High Polarity

| PT0 | PT1 | Power-Up RESET | Level at Pin if Macrocell Active High | Set/Reset Selector Fuse Setting |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | H | 1 |
| 1 | 0 | 0 | L | 1 |
| 0 | 0 | 1 | L | 1 |

Set/Reset Selector Fuse State When Macrocell Register Has Active-Low Polarity

| PT0 | PT1 | Power-Up RESET | Level at Pin if Macrocell Active Low | Set/Reset Selector Fuse Setting |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | L | 0 |
| 1 | 0 | 0 | H | 0 |
| 0 | 0 | 1 | H | 0 |

PT0 and PT1 are the register control product terms. The Set/Reset selector fuse directs the appropriate register control terms to make the MACH 3xx/4xx register respond like the PAL22V10 register on set, reset and power-up operations. If the reset term is assigned to PT0 and the set term is assigned to PT1, the Fitter programs the Set/Reset selector fuse to 1 for active-high registers, 0 for active-low registers.

### Set/Reset in MACH 3xx/4xx Asynchronous MacrocellsXE "MACH 3XX/4XX:MACH 3XX/4XX asynchronous macrocells"§

XE "macrocells:asynchronous, Set and Reset"§XE "asynchronous macrocells:Set and Reset"§The Fitter will configure macrocells as asynchronous to implement signals that have a product-term clock. When a MACH 3xx/4xx macrocell is configured as asynchronous, one product term performs register initialization.  This product term is programmed as either the set or reset control line, as shown in the figure below. To achieve PAL22V10 set/reset compatibility, the Fitter must use the Set/Reset selector fuse to direct the control product term to the correct register control line based on the polarity of the asynchronous signal.

µ §

Example

The reset product term for an active-high asynchronous signal is directed to the reset line, while the reset product term for an active-low asynchronous signal must be directed to the set control line, as shown in the following figure. Set/reset behavior is thus the same as on the PAL22V10.

µ §

If you do not require PAL22V10 set/reset compatibility, and want the set logic to set the register high and the reset logic to reset the register low, respectively, independent of polarity, set 22V10/MACH1xx/2xx S/R Compatibility option to "N" (File:Set up:Compilation options).

### Higher Block Utilization with the Set/Reset Selector FuseXE "MACH 3XX/4XX:higher block utilization with the Set/Reset selector fuse"§XE "block utilization"§

XE "set/reset selector"§The MACHXL software automatically uses the Set/Reset Selector register control term swapping feature to provide higher block utilization under certain conditions. For example: If a design has two synchronous signals A and B, and the set and reset control terms for A are the same as the reset and set control terms for B, respectively, these two signals can be put into the same block. By programming Set/Reset Selector accordingly, these two signals can use the same block set and reset control terms, as shown below.

µ §

The Set/Reset selector fuse settings shown above are for A and B active high. If A were active low and B active high, the Set/Reset selector fuses for signals A and B should be both 0 to maintain PAL22V10 register set and reset behavior compatibility.

### MACH 3xx/4xx Power-UpXE "MACH 3XX/4XX:power-up"§XE "power-up"§

XE "power-up"§The MACH 3xx/4xx has a power-up register initialization feature that forces the registers to either 0 or 1 when power is applied to a part according to certain guidelines given in the device data sheet. A register initializes to either 0 or 1, depending on how the Fitter partitions and fits the design.

If a signal has both set and reset logic defined, the Fitter has to configure as synchronous the macrocell to which it is mapped. If the Fitter packed signals with complementary set and reset logic into the same block, a set product term for one signal is used as a reset for another signal. As shown in the figure above, product term X (which is ORed with the power-up line) resets signal A and sets signal B. Assuming that signals A and B are both active-high signals, on power-up register A is reset to 0 while register B is set to 1.

If you use GROUP MACH_SEG_X statements to force the Fitter to put signals with common reset and set product terms into the same block, the Fitter assigns the common reset product term to PT0 which is logically ORed with the power-up line and the set to PT1. If the Maintain 22V10 Set/Reset compatibility  Y/N option is set to "Y," the Set/Reset selector fuse is programmed as shown in the tables titled "Set/Reset Fuse State When Macrocell Register Has Active-High Polarity" and "Set/Reset Fuse State When Macrocell Register Has Active-Low Polarity."  On device power-up, the registers reset as shown in the tables. If the Maintain 22V10 Set/Reset compatibility  Y/N option is set to "N," the registers reset to 0 on power-up, and the level detected at the pin is low.

### MACH 3xx/4xx Asynchronous Macrocell Power-Up OperationXE "MACH 3XX/4XX:asynchronous macrocell power-up operation"§

XE "power-up:asynchronous"§The Fitter configures a macrocell as asynchronous if

r	A signal requires a product term clock

r	A signal has a set or a reset logic term (but not both)

and the partitioner decides to put it into a block which already has synchronous signals using the block-level set and reset terms. In an asynchronous macrocell on the MACH 3xx/4xx the power-up line is logically ORed with the single product term that serves as either the SET or RESET term. When the power-up line is activated on device power-up, the asynchronous register is initialized as if the control product term were activated.

Example
If an active-high registered signal with a SET product term is assigned to an asynchronous macrocell, on device power-up, the register is set (Q is high) and the level detected at the pin will be 1. If the PAL22V10 set/reset compatibility option is set to "Y" and the signal is active-low, the Set/Reset selector fuse is programmed to direct the SET product term to the register's reset line. When the device powers up or the SET product term is active, the register is reset (Q is low) and the level detected at the pin is 0.

### Set/Reset Design RecommendationsXE "MACH 3XX/4XX:set/reset design recommendations"§

To reduce the chance of unexpected behavior, write all the necessary set and/or reset product terms for registered signals. After device power-up, activate the set and reset logic explicitly to initialize the registers as specified before commencing normal device operation. These precautions guard against instances where system power-up does not adhere to the power-up reset guidelines given in the device data sheet.

## Synchronous vs. Asynchronous OperationXE "MACH 3XX/4XX:synchronous vs. asynchronous operation"§XE "synchronous operation"§XE "asynchronous operation"§

XE "macrocells:asynchronous"§XE "macrocells:synchronous"§XE "synchronous macrocells"§XE "asynchronous macrocells"§The MACH 3xx/4xx can support both synchronous and asynchronous logic on an individual macrocell basis in the same design and within the same PAL block.  Each macrocell has a special control for configuring it either as a synchronous macrocell or as an asynchronous macrocell.  The major differences between the synchronous and asynchronous modes are:

r	Number of product terms available for sum-of-products logic

r	Clocking flexibility

r	Reset/Set control

In either mode, the macrocell has its own, individually-programmable output enable product term.

It is important to note that the Fitter sometimes implements equations that appear to be either synchronous or asynchronous using macrocells of the opposite type.  If it is important that an equation be implemented in a specific mode, you can force the Fitter to implement it the way you want by providing a forcing condition in the equations for that signal in your design file.  Refer to "Forcing Configuration as a Synchronous Macrocell" in this chapter for more information.

| ã | Note: To maximize device efficiency, the Fitter prefers to implement clocks as global clocks rather than as product-term clocks and prefers to configure macrocells as synchronous macrocells rather than as asynchronous macrocells. |
| --- | --- |

### Synchronous ModeXE "MACH 3XX/4XX:synchronous mode"§

All synchronous mode macrocells in the same PAL block are initialized with a single, common, asynchronous Reset or Set product term.  However, each macrocell in the PAL block can swap the Set/Reset function on an individual macrocell basis, so that the same initialization signal either sets or resets each synchronous macrocell.

Each synchronous macrocell in a PAL block must be clocked by one of the four block clock signals available to the block containing the macrocell.  Refer to "Global Clock Rules" elsewhere in Chapter 7 and "Flexible Clock Generator" in this chapter for additional details.

The Fitter is forced to configure a macrocell in the synchronous mode if any of the following conditions is true:

r	There is a non-GND Set condition and a non-GND Reset condition.

r	After minimization, the primary equation has more than 18 product terms.

r	The macrocell is clocked by a signal preplaced at a global clock pin that is not accessible to asynchronous macrocells through the block clock mechanism if either of the following conditions exists:

u	The device is a MACH465 device.

u	The Global clocks routable as PT clocks? option of the MACH Fitting Options form is set to "N."

Each synchronous macrocell has five product terms available in its own product term cluster for sum-of-products logic.  Using product-term steering, a synchronous macrocell can support logic requiring up to 20 product terms.

### Asynchronous ModeXE "MACH 3XX/4XX:asynchronous mode"§

XE "asynchronous macrocells"§In asynchronous mode, each logic macrocell operates with independent clock control and set/reset control.

The Fitter is forced to configure a macrocell in the asynchronous mode if the specified clock was implemented (for any reason, including resource constraints) as a non-global clock.  (Note that the opposite is not true: the Fitter may, based on other constraints, configure a macrocell as asynchronous, even though the macrocell is clocked by a global clock.)

When a macrocell is configured as an asynchronous macrocell, one of the five product terms in the macrocell's product term cluster is reassigned from sum-of-products use to define the macrocell's individual product-term clock.  Specify the clock signal using the following general form:

Pin/node_name.CLKF = Boolean_expression.

When a macrocell is configured as an asynchronous macrocell, one of the five product terms in the macrocell's product term cluster is reassigned from sum-of-products use to define either the macrocell's individual set function or its reset function.  Specify the desired function using either of the following statements:

Pin/node_name.SETF = Boolean_expression

or

Pin/node_name.RSTF = Boolean_expression

Each asynchronous macrocell has three product terms available in its own product term cluster for sum-of-products logic.  Using product-term steering, an asynchronous macrocell can support logic requiring up to 18 product terms.

### Forcing Configuration as a Synchronous MacrocellXE "forcing synchronous"§XE "MACH 3xx/4xx:synchronous forcing conditions"§

The simplest way to force a signal to be synchronous is to include the signal name in a GROUP SYNC_LIST in the declaration segment of the design file.  The following discussion describes how the Fitter determines, in the absence of a GROUP SYNC_LIST statement, whether a signal should be implemented as a synchronous signal.  For more information on GROUP SYNC_LIST and GROUP ASYNC_LIST, refer to Chapter 5, "Language Reference."

Other than its effect on gate-splitting (and on the fact that synchronous registers must be clocked by global clock pins), it does not matter whether the Fitter implements your synchronous equation in a synchronous or asynchronous macrocell.  Both types of macrocell provide equivalent behavior as long as sufficient resources are available to implement the required logic.  Therefore, forcing the Fitter to configure a macrocell as synchronous is useful for only one purpose: to influence the Minimizer as it makes gate-splitting decisions.

The Minimizer's decision process is simple:

r	If an equation is combinatorial, it is split at whichever is lower: 20 product terms or the user-defined gate-split threshold.

r	If an equation is registered, the Minimizer considers the mode (synchronous or asynchronous) of the macrocell to which the equation will be mapped:

u	If the equation is forced to be synchronous, it is split at whichever is lower: 20 product terms or the user-defined gate-split threshold.

u	All other registered equations are limited to the maximum capacity of an asynchronous macrocell (18 product terms), in case the Fitter maps them to a macrocell that is configured in the asynchronous mode.  (Hence, such equations are split at whichever is lower: 18 product terms or the user-defined gate-split threshold.)

If you have an equation with 19 or 20 product terms and you do not want the Minimizer to split the equation, provide at least one forcing condition for the synchronous mode by doing one of the following:

r	Place the equation's clock at global clock pin 2 or 3.  In addition, if the device is not a MACH465 device, set the Global clocks routable as PT clocks? option of the MACH Fitting Options form to "N."  (This is generally the simplest solution.)

r	Provide a non-GND .SETF condition and a non-GND .RSTF condition for the equation.  (This solution is necessary when there are several macrocells to be forced synchronous and the clock signals for these macrocells cannot all be placed on global clock pins 0 and 1.)

## Cross-Programming MACH435 Designs to the MACH445 DeviceXE "MACH435:cross-programming designs"§XE "MACH445:cross-programming with MACH435 designs"§XE "cross-programming"§

The L field fuses of MACH435 and MACH445 JEDEC files are identical. Therefore, a MACH435 JEDEC file can be converted to a MACH445 JEDEC file which can then be used to program a MACH445 device. This is called cross programming.

To cross program a MACH435 design to a MACH445 using AMD's MACHXL and MACHPRO software, use MACH445 when asked for "Part Name" and the name of the MACH435 JEDEC file when asked "JEDEC file for the specified operation" on the Create/Edit configuration file menu (see section E).

Note that the pinout will change if a MACH435 JEDEC file is cross programmed as a MACH445 device. When restricted to IOs and dedicated inputs or input/clocks the pin mapping from a MACH435 to MACH445 device is a one-to-one relationship as shown below:

There are 64 IOs for both MACH435 and MACH445.

BLOCK A-435 3, 4, 5, 6, 7, 8, 9, 10
BLOCK A-445 93, 94, 95, 96, 97, 98, 99, 100

BLOCK B-435 19, 18, 17, 16, 15, 14, 13, 12
BLOCK B-445 12, 11, 10, 9, 8, 7, 6, 5

BLOCK C-435 24, 25, 26, 27, 28, 29, 30, 31
BLOCK C-445 19, 20, 21, 22, 23, 24, 25, 26

BLOCK D-435 40, 39, 38, 37, 36, 35, 34, 33
BLOCK D-445 38, 37, 36, 35, 34, 33, 32, 31

BLOCK E-435 45, 46, 47, 48, 49, 50, 51, 52
BLOCK E-445 43, 44, 45, 46, 47, 48, 49, 50

BLOCK F-435 61, 60, 59, 58, 57, 56, 55, 54
BLOCK F-445 62, 61, 60, 59, 58, 57, 56, 55

BLOCK G-435 66, 67, 68, 69, 70, 71, 72, 73
BLOCK G-445 69, 70, 71, 72, 73, 74, 75, 76

BLOCK H-435 82, 81, 80, 79, 78, 77, 76, 75
BLOCK H-445 88, 87, 86, 85, 84, 83, 82, 81

There are 2 input-only pins for both MACH435: 41 83 
and the MACH445: 54 4

There are 4 clock/input pins for both MACH435: 20 23 62 65
and the MACH445: 13 18 63 68

There are 8 ground pins for MACH435: 1, 11, 22, 32, 43, 53, 64, 74
There are 16 ground pins for MACH445: {1, 2, 16, 17, 29, 30, 40, 41, 51, 52, 66, 67, 79, 80, 90, 91

There are 6 VCC pins for MACH435: 2, 21, 42, 44, 63, 84
There are 8 VCC pins for MACH445: 14, 15, 39, 42, 64, 65, 89, 92

There are 6 JTAG pins for MACH445: 3, 27, 28, 53, 77, 78
with no counterpart in MACH435.

# MACH110 Pin and Node SummaryXE "MACH110:pin and node summary"§XE "pin and node summary:MACH110"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH110:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH110

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | GND | N/A | N/A |
| 2 | I/O_0 | 2 | A0 |
| 3 | I/O_1 | 3 | A1 |
| 4 | I/O_2 | 4 | A2 |
| 5 | I/O_3 | 5 | A5 |
| 6 | I/O_4 | 6 | A6 |
| 7 | I/O_5 | 7 | A5 |
| 8 | I/O_6 | 8 | A6 |
| 9 | I/O_7 | 9 | A7 |
| 10 | I0 | N/A | N/A |
| 11 | I1 | N/A | N/A |
| 12 | GND | N/A | N/A |
| 13 | CLK0/I2 | N/A | N/A |
| 14 | I/O_8 | 10 | A8 |
| 15 | I/O_9 | 11 | A9 |
| 16 | I/O_10 | 12 | A10 |
| 17 | I/O_11 | 13 | A11 |
| 18 | I/O_12 | 14 | A12 |
| 19 | I/O_13 | 15 | A13 |
| 20 | I/O_14 | 16 | A14 |
| 21 | I/O_15 | 17 | A15 |
| 22 | VCC | N/A | N/A |
| 23 | GND | N/A | N/A |

Continued...

Pin/Node Table, MACH110, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 24 | I/O_16 | 18 | B15 |
| 25 | I/O_17 | 19 | B14 |
| 26 | I/O_18 | 20 | B13 |
| 27 | I/O_19 | 21 | B12 |
| 28 | I/O_20 | 22 | B11 |
| 29 | I/O_21 | 23 | B10 |
| 30 | I/O_22 | 24 | B9 |
| 31 | I/O_22 | 25 | B8 |
| 32 | I3 | N/A | N/A |
| 33 | I4 | N/A | N/A |
| 34 | GND | N/A | N/A |
| 35 | CLK1/I5 | N/A | N/A |
| 36 | I/O_24 | 26 | B7 |
| 37 | I/O_25 | 27 | B6 |
| 38 | I/O_26 | 28 | B5 |
| 39 | I/O_27 | 29 | B4 |
| 40 | I/O_28 | 30 | B3 |
| 41 | I/O_29 | 31 | B2 |
| 42 | I/O_30 | 32 | B1 |
| 43 | I/O_31 | 33 | B0 |
| 44 | VCC | N/A | N/A |

# MACH111 Pin and Node SummaryXE "MACH111:pin and node summary"§XE "pin and node summary:MACH111"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH111:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH111

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | GND | N/A | N/A |
| 2 | I/O_0 | 2 | A0 |
| 3 | I/O_1 | 3 | A1 |
| 4 | I/O_2 | 4 | A2 |
| 5 | I/O_3 | 5 | A5 |
| 6 | I/O_4 | 6 | A6 |
| 7 | I/O_5 | 7 | A5 |
| 8 | I/O_6 | 8 | A6 |
| 9 | I/O_7 | 9 | A7 |
| 10 | I_0 | N/A | N/A |
| 11 | CLK2/I1 | N/A | N/A |
| 12 | GND | N/A | N/A |
| 13 | CLK0/I2 | N/A | N/A |
| 14 | I/O_8 | 10 | A8 |
| 15 | I/O_9 | 11 | A9 |
| 16 | I/O_10 | 12 | A10 |
| 17 | I/O_11 | 13 | A11 |
| 18 | I/O_12 | 14 | A12 |
| 19 | I/O_13 | 15 | A13 |
| 20 | I/O_14 | 16 | A14 |
| 21 | I/O_15 | 17 | A15 |
| 22 | VCC | N/A | N/A |
| 23 | GND | N/A | N/A |
| 24 | I/O_16 | 18 | B0 |
| 25 | I/O_17 | 19 | B1 |
| 26 | I/O_18 | 20 | B2 |
| 27 | I/O_19 | 21 | B3 |

Continued...

Pin/Node Table, MACH111, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 28 | I/O_20 | 22 | B4 |
| 29 | I/O_21 | 23 | B5 |
| 30 | I/O_22 | 24 | B6 |
| 31 | I/O_22 | 25 | B7 |
| 32 | I_3 | N/A | N/A |
| 33 | CLK3/I4 | N/A | N/A |
| 34 | GND | N/A | N/A |
| 35 | CLK1/I5 | N/A | N/A |
| 36 | I/O_24 | 26 | B8 |
| 37 | I/O_25 | 27 | B9 |
| 38 | I/O_26 | 28 | B10 |
| 39 | I/O_27 | 29 | B11 |
| 40 | I/O_28 | 30 | B12 |
| 41 | I/O_29 | 31 | B13 |
| 42 | I/O_30 | 32 | B14 |
| 43 | I/O_31 | 33 | B15 |
| 44 | VCC | N/A | N/A |

# MACH120 Pin and Node SummaryXE "MACH120:pin and node summary"§XE "pin and node summary:MACH120"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH120:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH120

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | GND | N/A | N/A |
| 2 | I/O_0 | 2 | A0 |
| 3 | I/O_1 | 3 | A1 |
| 4 | I/O_2 | 4 | A2 |
| 5 | I/O_3 | 5 | A5 |
| 6 | I/O_4 | 6 | A6 |
| 7 | I/O_5 | 7 | A5 |
| 8 | GND | N/A | N/A |
| 9 | I/O_6 | 8 | A6 |
| 10 | I/O_7 | 9 | A7 |
| 11 | I/O_8 | 10 | A8 |
| 12 | I/O_9 | 11 | A9 |
| 13 | I/O_10 | 12 | A10 |
| 14 | I/O_11 | 13 | A11 |
| 15 | I0/CLK0 | N/A | N/A |
| 16 | I1/CLK1 | N/A | N/A |
| 17 | I2 | N/A | N/A |
| 18 | VCC | N/A | N/A |
| 19 | GND | N/A | N/A |
| 20 | I3 | N/A | N/A |
| 21 | I/O_12 | 25 | B11 |
| 22 | I/O_13 | 24 | B10 |
| 23 | I/O_14 | 23 | B9 |
| 24 | I/O_15 | 22 | B8 |
| 25 | I/O_16 | 21 | B7 |
| 26 | I/O_17 | 20 | B6 |
| 27 | GND | N/A | N/A |
| 28 | I/O_18 | 19 | B5 |

Continued...

Pin/Node Table, MACH120, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 29 | I/O_19 | 18 | B4 |
| 30 | I/O_20 | 17 | B3 |
| 31 | I/O_21 | 16 | B2 |
| 32 | I/O_22 | 15 | B1 |
| 33 | I/O_23 | 14 | B0 |
| 34 | VCC | N/A | N/A |
| 35 | GND | N/A | N/A |
| 36 | I/O_24 | 26 | C0 |
| 37 | I/O_25 | 27 | C1 |
| 38 | I/O_26 | 28 | C2 |
| 39 | I/O_27 | 29 | C3 |
| 40 | I/O_28 | 30 | C4 |
| 41 | I/O_29 | 31 | C5 |
| 42 | GND | N/A | N/A |
| 43 | I/O_30 | 32 | C6 |
| 44 | I/O_31 | 33 | C7 |
| 45 | I/O_32 | 34 | C8 |
| 46 | I/O_33 | 35 | C9 |
| 47 | I/O_34 | 36 | C10 |
| 48 | I/O_35 | 37 | C11 |
| 49 | I4/CLK2 | N/A | N/A |
| 50 | I5/CLK3 | N/A | N/A |
| 51 | I6 | N/A | N/A |
| 52 | VCC | N/A | N/A |
| 53 | GND | N/A | N/A |
| 54 | I7 | N/A | N/A |
| 55 | I/O_36 | 49 | D11 |
| 56 | I/O_37 | 48 | D10 |
| 57 | I/O_38 | 47 | D9 |
| 58 | I/O_39 | 46 | D8 |
| 59 | I/O_40 | 45 | D7 |

Continued...

Pin/Node Table, MACH120, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 60 | I/O_41 | 44 | D6 |
| 61 | GND | N/A | N/A |
| 62 | I/O_42 | 43 | D5 |
| 63 | I/O_43 | 42 | D4 |
| 64 | I/O_44 | 41 | D3 |
| 65 | I/O_45 | 40 | D2 |
| 66 | I/O_46 | 39 | D1 |
| 67 | I/O_47 | 38 | D0 |
| 68 | VCC | N/A | N/A |

# MACH130 Pin and Node SummaryXE "MACH130:pin and node summary"§XE "pin and node summary:MACH130"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH130:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH130

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | GND | N/A | N/A |
| 2 | VCC | N/A | N/A |
| 3 | I/O_0 | 2 | A0 |
| 4 | I/O_1 | 3 | A1 |
| 5 | I/O_2 | 4 | A2 |
| 6 | I/O_3 | 5 | A3 |
| 7 | I/O_4 | 6 | A4 |
| 8 | I/O_5 | 7 | A5 |
| 9 | I/O_6 | 8 | A6 |
| 10 | I/O_7 | 9 | A7 |
| 11 | GND | N/A | N/A |
| 12 | I/O_8 | 10 | A8 |
| 13 | I/O_9 | 11 | A9 |
| 14 | I/O_10 | 12 | A10 |
| 15 | I/O_11 | 13 | A11 |
| 16 | I/O_12 | 14 | A12 |
| 17 | I/O_13 | 15 | A13 |
| 18 | I/O_14 | 16 | A14 |
| 19 | I/O_15 | 17 | A15 |
| 20 | I0/CLK0 | N/A | N/A |
| 21 | VCC | N/A | N/A |
| 22 | GND | N/A | N/A |
| 23 | I1/CLK1 | N/A | N/A |
| 24 | I/O_16 | 33 | B15 |
| 25 | I/O_17 | 32 | B14 |
| 26 | I/O_18 | 31 | B13 |
| 27 | I/O_19 | 30 | B12 |

Continued...

Pin/Node Table, MACH130, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 28 | I/O_20 | 29 | B11 |
| 29 | I/O_21 | 28 | B10 |
| 30 | I/O_22 | 27 | B9 |
| 31 | I/O_23 | 26 | B8 |
| 32 | GND | N/A | N/A |
| 33 | I/O_24 | 25 | N/A |
| 34 | I/O_25 | 24 | N/A |
| 35 | I/O_26 | 23 | N/A |
| 36 | I/O_27 | 22 | N/A |
| 37 | I/O_28 | 21 | N/A |
| 38 | I/O_29 | 20 | N/A |
| 39 | I/O_30 | 19 | N/A |
| 40 | I/O_31 | 18 | N/A |
| 41 | I2 | N/A | N/A |
| 42 | VCC | N/A | N/A |
| 43 | GND | N/A | N/A |
| 44 | VCC | N/A | N/A |
| 45 | I/O_32 | 34 | C0 |
| 46 | I/O_33 | 35 | C1 |
| 47 | I/O_34 | 36 | C2 |
| 48 | I/O_35 | 37 | C3 |
| 49 | I/O_36 | 38 | C4 |
| 50 | I/O_37 | 39 | C5 |
| 51 | I/O_38 | 40 | C6 |
| 52 | I/O_39 | 41 | C7 |
| 53 | GND | N/A | N/A |
| 54 | I/O_40 | 42 | C8 |
| 55 | I/O_41 | 43 | C9 |
| 56 | I/O_42 | 44 | C10 |
| 57 | I/O_43 | 45 | C11 |
| 58 | I/O_44 | 46 | C12 |
| 59 | I/O_45 | 47 | C13 |
| 60 | I/O_46 | 48 | C14 |
| 61 | I/O_47 | 49 | C15 |

Continued...

Pin/Node Table, MACH130, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 62 | 13/CLK2 | N/A | N/A |
| 63 | VCC | N/A | N/A |
| 64 | GND | N/A | N/A |
| 65 | I4/CLK3 | N/A | N/A |
| 66 | I/O_48 | 65 | D15 |
| 67 | I/O_49 | 64 | D14 |
| 68 | I/O_50 | 63 | D13 |
| 69 | I/O_51 | 62 | D12 |
| 70 | I/O_52 | 61 | D11 |
| 71 | I/O_53 | 60 | D10 |
| 72 | I/O_54 | 59 | D9 |
| 73 | I/O_55 | 58 | D8 |
| 74 | GND | N/A | N/A |
| 75 | I/O_56 | 57 | D7 |
| 76 | I/O_57 | 56 | D6 |
| 77 | I/O_58 | 55 | D5 |
| 78 | I//O_59 | 54 | D4 |
| 79 | I/O_60 | 53 | D3 |
| 80 | I/O_61 | 52 | D2 |
| 81 | I/O_62 | 51 | D1 |
| 82 | I/O_63 | 50 | D0 |
| 83 | I4 | N/A | N/A |
| 84 | VCC | N/A | N/A |

# MACH131 Pin and Node SummaryXE "MACH131:pin and node summary"§XE "pin and node summary:MACH131"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH131:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH131

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | GND | N/A | N/A |
| 2 | VCC | N/A | N/A |
| 3 | I/O_0 | 2 | A0 |
| 4 | I/O_1 | 3 | A1 |
| 5 | I/O_2 | 4 | A2 |
| 6 | I/O_3 | 5 | A3 |
| 7 | I/O_4 | 6 | A4 |
| 8 | I/O_5 | 7 | A5 |
| 9 | I/O_6 | 8 | A6 |
| 10 | I/O_7 | 9 | A7 |
| 11 | GND | N/A | N/A |
| 12 | I/O_8 | 10 | A8 |
| 13 | I/O_9 | 11 | A9 |
| 14 | I/O_10 | 12 | A10 |
| 15 | I/O_11 | 13 | A11 |
| 16 | I/O_12 | 14 | A12 |
| 17 | I/O_13 | 15 | A13 |
| 18 | I/O_14 | 16 | A14 |
| 19 | I/O_15 | 17 | A15 |
| 20 | I0/CLK0 | N/A | N/A |
| 21 | VCC | N/A | N/A |
| 22 | GND | N/A | N/A |
| 23 | I1/CLK1 | N/A | N/A |
| 24 | I/O_16 | 33 | B15 |
| 25 | I/O_17 | 32 | B14 |
| 26 | I/O_18 | 31 | B13 |
| 27 | I/O_19 | 30 | B12 |
| 28 | I/O_20 | 29 | B11 |

Continued...

Pin/Node Table, MACH131, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 29 | I/O_21 | 28 | B10 |
| 30 | I/O_22 | 27 | B9 |
| 31 | I/O_23 | 26 | B8 |
| 32 | GND | N/A | N/A |
| 33 | I/O_24 | 25 | B7 |
| 34 | I/O_25 | 24 | B6 |
| 35 | I/O_26 | 23 | B5 |
| 36 | I/O_27 | 22 | B4 |
| 37 | I/O_28 | 21 | B3 |
| 38 | I/O_29 | 20 | B2 |
| 39 | I/O_30 | 19 | B1 |
| 40 | I/O_31 | 18 | B0 |
| 41 | I2 | N/A | N/A |
| 42 | VCC | N/A | N/A |
| 43 | GND | N/A | N/A |
| 44 | VCC | N/A | N/A |
| 45 | I/O_32 | 34 | C0 |
| 46 | I/O_33 | 35 | C1 |
| 47 | I/O_34 | 36 | C2 |
| 48 | I/O_35 | 37 | C3 |
| 49 | I/O_36 | 38 | C4 |
| 50 | I/O_37 | 39 | C5 |
| 51 | I/O_38 | 40 | C6 |
| 52 | I/O_39 | 41 | C7 |
| 53 | GND | N/A | N/A |
| 54 | I/O_40 | 42 | C8 |
| 55 | I/O_41 | 43 | C9 |
| 56 | I/O_42 | 44 | C10 |
| 57 | I/O_43 | 45 | C11 |
| 58 | I/O_44 | 46 | C12 |
| 59 | I/O_45 | 47 | C13 |
| 60 | I/O_46 | 48 | C14 |
| 61 | I/O_47 | 49 | C15 |
| 62 | 13/CLK2 | N/A | N/A |
| 63 | VCC | N/A | N/A |

Continued...

Pin/Node Table, MACH131, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 64 | GND | N/A | N/A |
| 65 | I4/CLK3 | N/A | N/A |
| 66 | I/O_48 | 65 | D15 |
| 67 | I/O_49 | 64 | D14 |
| 68 | I/O_50 | 63 | D13 |
| 69 | I/O_51 | 62 | D12 |
| 70 | I/O_52 | 61 | D11 |
| 71 | I/O_53 | 60 | D10 |
| 72 | I/O_54 | 59 | D9 |
| 73 | I/O_55 | 58 | D8 |
| 74 | GND | N/A | N/A |
| 75 | I/O_56 | 57 | D7 |
| 76 | I/O_57 | 56 | D6 |
| 77 | I/O_58 | 55 | D5 |
| 78 | I//O_59 | 54 | D4 |
| 79 | I/O_60 | 53 | D3 |
| 80 | I/O_61 | 52 | D2 |
| 81 | I/O_62 | 51 | D1 |
| 82 | I/O_63 | 50 | D0 |
| 83 | I4 | N/A | N/A |
| 84 | VCC | N/A | N/A |

# MACH210 Pin and Node SummaryXE "MACH210:pin and node summary"§XE "pin and node summary:MACH210"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. XE "MACH210:global node"§XE "set and reset:global"§Node numbers for other nodes are listed below under Output Macrocell and Input Register. |
| --- | --- |

Pin/Node Table, MACH210

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A |
| 2 | I/O_0 | 2 | 3 | A0 |
| 3 | I/O_1 | 4 | 5 | A2 |
| 4 | I/O_2 | 6 | 7 | A4 |
| 5 | I/O_3 | 8 | 9 | A6 |
| 6 | I/O_4 | 10 | 11 | A8 |
| 7 | I/O_5 | 12 | 13 | A10 |
| 8 | I/O_6 | 14 | 15 | A12 |
| 9 | I/O_7 | 16 | 17 | A14 |
| 10 | I0 | N/A | N/A | N/A |
| 11 | I1 | N/A | N/A | N/A |
| 12 | GND | N/A | N/A | N/A |
| 13 | CLK0/I2 | N/A | N/A | N/A |
| 14 | I/O_8 | 32 | 33 | B14 |
| 15 | I/O_9 | 30 | 31 | B12 |
| 16 | I/O_10 | 28 | 29 | B10 |
| 17 | I/O_11 | 26 | 27 | B8 |
| 18 | I/O_12 | 24 | 25 | B6 |
| 19 | I/O_13 | 22 | 23 | B4 |
| 20 | I/O_14 | 20 | 21 | B2 |
| 21 | I/O_15 | 18 | 19 | B0 |
| 22 | VCC | N/A | N/A | N/A |
| 23 | GND | N/A | N/A | N/A |
| 24 | I/O_16 | 34 | 35 | C0 |
| 25 | I/O_17 | 36 | 37 | C2 |

Continued...

Pin/Node Table, MACH210, Continued

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 26 | I/O_18 | 38 | 39 | C4 |
| 27 | I/O_19 | 40 | 41 | C6 |
| 28 | I/O_20 | 42 | 43 | C8 |
| 29 | I/O_21 | 44 | 45 | C10 |
| 30 | I/O_22 | 46 | 47 | C12 |
| 31 | I/O_22 | 48 | 49 | C14 |
| 32 | I3 | N/A | N/A | N/A |
| 33 | I4 | N/A | N/A | N/A |
| 34 | GND | N/A | N/A | N/A |
| 35 | CLK1/I5 | N/A | N/A | N/A |
| 36 | I/O_24 | 64 | 65 | D14 |
| 37 | I/O_25 | 62 | 63 | D12 |
| 38 | I/O_26 | 60 | 61 | D10 |
| 39 | I/O_27 | 58 | 59 | D8 |
| 40 | I/O_28 | 56 | 57 | D6 |
| 41 | I/O_29 | 54 | 55 | D4 |
| 42 | I/O_30 | 52 | 53 | D2 |
| 43 | I/O_31 | 50 | 51 | D1 |
| 44 | VCC | N/A | N/A | N/A |

# MACH211 Pin and Node SummaryXE "MACH211:pin and node summary"§XE "pin and node summary:MACH211"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. XE "MACH211:global node"§XE "set and reset:global"§Node numbers for other nodes are listed below under Output Macrocell and Input Register. |
| --- | --- |

Pin/Node Table, MACH211

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A |
| 2 | I/O_0 | 2 | 3 | A0 |
| 3 | I/O_1 | 4 | 5 | A2 |
| 4 | I/O_2 | 6 | 7 | A4 |
| 5 | I/O_3 | 8 | 9 | A6 |
| 6 | I/O_4 | 10 | 11 | A8 |
| 7 | I/O_5 | 12 | 13 | A10 |
| 8 | I/O_6 | 14 | 15 | A12 |
| 9 | I/O_7 | 16 | 17 | A14 |
| 10 | I0 | N/A | N/A | N/A |
| 11 | CLK0/I1 | N/A | N/A | N/A |
| 12 | GND | N/A | N/A | N/A |
| 13 | CLK1/I2 | N/A | N/A | N/A |
| 14 | I/O_8 | 32 | 33 | B14 |
| 15 | I/O_9 | 30 | 31 | B12 |
| 16 | I/O_10 | 28 | 29 | B10 |
| 17 | I/O_11 | 26 | 27 | B8 |
| 18 | I/O_12 | 24 | 25 | B6 |
| 19 | I/O_13 | 22 | 23 | B4 |
| 20 | I/O_14 | 20 | 21 | B2 |
| 21 | I/O_15 | 18 | 19 | B0 |
| 22 | VCC | N/A | N/A | N/A |
| 23 | GND | N/A | N/A | N/A |
| 24 | I/O_16 | 34 | 35 | C0 |
| 25 | I/O_17 | 36 | 37 | C2 |
| 26 | I/O_18 | 38 | 39 | C4 |

Continued...

Pin/Node Table, MACH211

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 27 | I/O_19 | 40 | 41 | C6 |
| 28 | I/O_20 | 42 | 43 | C8 |
| 29 | I/O_21 | 44 | 45 | C10 |
| 30 | I/O_22 | 46 | 47 | C12 |
| 31 | I/O_22 | 48 | 49 | C14 |
| 32 | I3 | N/A | N/A | N/A |
| 33 | CLK2/I4 | N/A | N/A | N/A |
| 34 | GND | N/A | N/A | N/A |
| 35 | CLK3/I5 | N/A | N/A | N/A |
| 36 | I/O_24 | 64 | 65 | D14 |
| 37 | I/O_25 | 62 | 63 | D12 |
| 38 | I/O_26 | 60 | 61 | D10 |
| 39 | I/O_27 | 58 | 59 | D8 |
| 40 | I/O_28 | 56 | 57 | D6 |
| 41 | I/O_29 | 54 | 55 | D4 |
| 42 | I/O_30 | 52 | 53 | D2 |
| 43 | I/O_31 | 50 | 51 | D1 |
| 44 | VCC | N/A | N/A | N/A |

# MACH215 Pin and Node SummaryXE "MACH215:pin and node summary"§XE "pin and node summary:MACH215"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. XE "MACH215:global node"§XE "set and reset:global"§Node numbers for other nodes are listed below under Output Macrocell and Input Register. |
| --- | --- |

Pin/Node Table, MACH215

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A |
| 2 | I/O_0 | 2 | 3 | A0 |
| 3 | I/O_1 | 4 | 5 | A2 |
| 4 | I/O_2 | 6 | 7 | A4 |
| 5 | I/O_3 | 8 | 9 | A6 |
| 6 | I/O_4 | 10 | 11 | A8 |
| 7 | I/O_5 | 12 | 13 | A10 |
| 8 | I/O_6 | 14 | 15 | A12 |
| 9 | I/O_7 | 16 | 17 | A14 |
| 10 | I0 | N/A | N/A | N/A |
| 11 | I1 | N/A | N/A | N/A |
| 12 | GND | N/A | N/A | N/A |
| 13 | CLK0/I2 | N/A | N/A | N/A |
| 14 | I/O_8 | 32 | 33 | B14 |
| 15 | I/O_9 | 30 | 31 | B12 |
| 16 | I/O_10 | 28 | 29 | B10 |
| 17 | I/O_11 | 26 | 27 | B8 |
| 18 | I/O_12 | 24 | 25 | B6 |
| 19 | I/O_13 | 22 | 23 | B4 |
| 20 | I/O_14 | 20 | 21 | B2 |
| 21 | I/O_15 | 18 | 19 | B0 |

Continued...

Pin/Node Table, MACH215, Continued

| Pin # | Default Name | Output Macro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 22 | VCC | N/A | N/A | N/A |
| 23 | GND | N/A | N/A | N/A |
| 24 | I/O_16 | 34 | 35 | C0 |
| 25 | I/O_17 | 36 | 37 | C2 |
| 26 | I/O_18 | 38 | 39 | C4 |
| 27 | I/O_19 | 40 | 41 | C6 |
| 28 | I/O_20 | 42 | 43 | C8 |
| 29 | I/O_21 | 44 | 45 | C10 |
| 30 | I/O_22 | 46 | 47 | C12 |
| 31 | I/O_22 | 48 | 49 | C14 |
| 32 | I3 | N/A | N/A | N/A |
| 33 | I4 | N/A | N/A | N/A |
| 34 | GND | N/A | N/A | N/A |
| 35 | CLK1/I5 | N/A | N/A | N/A |
| 36 | I/O_24 | 64 | 65 | D14 |
| 37 | I/O_25 | 62 | 63 | D12 |
| 38 | I/O_26 | 60 | 61 | D10 |
| 39 | I/O_27 | 58 | 59 | D8 |
| 40 | I/O_28 | 56 | 57 | D6 |
| 41 | I/O_29 | 54 | 55 | D4 |
| 42 | I/O_30 | 52 | 53 | D2 |
| 43 | I/O_31 | 50 | 51 | D1 |
| 44 | VCC | N/A | N/A | N/A |

# MACH220 Pin and Node SummaryXE "MACH220:pin and node summary"§XE "pin and node summary:MACH220"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. XE "MACH220:global node"§XE "set and reset:global"§Node numbers for other nodes are listed below under Output Macrocell and Input Register. |
| --- | --- |

Pin/Node Table, MACH220

| Pin # | Default Name | OutputMacro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A |
| 2 | I/O_0 | 2 | 3 | A0 |
| 3 | I/O_1 | 4 | 5 | A2 |
| 4 | I/O_2 | 6 | 7 | A4 |
| 5 | I/O_3 | 8 | 9 | A6 |
| 6 | I/O_4 | 10 | 11 | A8 |
| 7 | I/O_5 | 12 | 13 | A10 |
| 8 | GND | N/A | N/A | N/A |
| 9 | I/O_6 | 24 | 25 | B10 |
| 10 | I/O_7 | 22 | 23 | B8 |
| 11 | I/O_8 | 20 | 21 | B6 |
| 12 | I/O_9 | 18 | 19 | B4 |
| 13 | I/O_10 | 16 | 17 | B2 |
| 14 | I/O_11 | 14 | 15 | B0 |
| 15 | I0/CLK0 | N/A | N/A | N/A |
| 16 | I1/CLK1 | N/A | N/A | N/A |
| 17 | I2 | N/A | N/A | N/A |
| 18 | VCC | N/A | N/A | N/A |
| 19 | GND | N/A | N/A | N/A |
| 20 | I3 | N/A | N/A | N/A |
| 21 | I/O_12 | 26 | 27 | C0 |
| 22 | I/O_13 | 28 | 29 | C2 |
| 23 | I/O_14 | 30 | 32 | C4 |
| 24 | I/O_15 | 32 | 33 | C6 |
| 25 | I/O_16 | 34 | 35 | C8 |

Continued...

Pin/Node Table, MACH220, Continued

| Pin # | Default Name | OutputMacro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 26 | I/O_17 | 36 | 37 | C10 |
| 27 | GND | N/A | N/A | N/A |
| 28 | I/O_18 | 48 | 49 | D10 |
| 29 | I/O_19 | 46 | 47 | D8 |
| 30 | I/O_20 | 44 | 45 | D6 |
| 31 | I/O_21 | 42 | 43 | D4 |
| 32 | I/O_22 | 40 | 41 | D2 |
| 33 | I/O_23 | 38 | 39 | D0 |
| 34 | VCC | N/A | N/A | N/A |
| 35 | GND | N/A | N/A | N/A |
| 36 | I/O_24 | 50 | 51 | E0 |
| 37 | I/O_25 | 52 | 53 | E2 |
| 38 | I/O_26 | 54 | 55 | E4 |
| 39 | I/O_27 | 56 | 57 | E6 |
| 40 | I/O_28 | 58 | 59 | E8 |
| 41 | I/O_29 | 60 | 61 | E10 |
| 42 | GND | N/A | N/A | N/A |
| 43 | I/O_30 | 72 | 73 | F10 |
| 44 | I/O_31 | 70 | 71 | F8 |
| 45 | I/O_32 | 68 | 69 | F6 |
| 46 | I/O_33 | 66 | 67 | F4 |
| 47 | I/O_34 | 64 | 65 | F2 |
| 48 | I/O_35 | 62 | 63 | F0 |
| 49 | I4/CLK2 | N/A | N/A | N/A |
| 50 | I5/CLK3 | N/A | N/A | N/A |
| 51 | I6 | N/A | N/A | N/A |
| 52 | VCC | N/A | N/A | N/A |
| 53 | GND | N/A | N/A | N/A |
| 54 | I7 | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH220, Continued

| Pin # | Default Name | OutputMacro- cell | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- |
| 55 | I/O_36 | 74 | 75 | G0 |
| 56 | I/O_37 | 76 | 77 | G2 |
| 57 | I/O_38 | 78 | 79 | G4 |
| 58 | I/O_39 | 80 | 81 | G6 |
| 59 | I/O_40 | 82 | 83 | G8 |
| 60 | I/O_41 | 84 | 85 | G10 |
| 61 | GND | N/A | N/A | N/A |
| 62 | I/O_42 | 96 | 97 | H10 |
| 63 | I/O_43 | 94 | 95 | H8 |
| 64 | I/O_44 | 92 | 93 | H6 |
| 65 | I/O_45 | 90 | 91 | H4 |
| 66 | I/O_46 | 88 | 89 | H2 |
| 67 | I/O_47 | 86 | 87 | H0 |
| 68 | VCC | N/A | N/A | N/A |

# MACH231 Pin and Node SummaryXE "MACH231:pin and node summary"§XE "pin and node summary:MACH231"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH231:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH231

| Pin # | Default Name | OutputMacro- cell |  | Pin Feedback |
| --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A |
| 2 | VCC | N/A | N/A | N/A |
| 3 | I/O_0 | 2 | 3 | A0 |
| 4 | I/O_1 | 4 | 5 | A2 |
| 5 | I/O_2 | 6 | 7 | A4 |
| 6 | I/O_3 | 8 | 9 | A6 |
| 7 | I/O_4 | 10 | 11 | A8 |
| 8 | I/O_5 | 12 | 13 | A10 |
| 9 | I/O_6 | 14 | 15 | A12 |
| 10 | I/O_7 | 16 | 17 | A14 |
| 11 | GND | N/A | N/A | N/A |
| 12 | I/O_8 | 32 | 33 | B14 |
| 13 | I/O_9 | 30 | 31 | B12 |
| 14 | I/O_10 | 28 | 29 | B10 |
| 15 | I/O_11 | 26 | 27 | B8 |
| 16 | I/O_12 | 24 | 25 | B6 |
| 17 | I/O_13 | 22 | 23 | B4 |
| 18 | I/O_14 | 20 | 21 | B2 |
| 19 | I/O_15 | 18 | 19 | B0 |
| 20 | I0/CLK0 | N/A | N/A | N/A |
| 21 | VCC | N/A | N/A | N/A |
| 22 | GND | N/A | N/A | N/A |
| 23 | I1/CLK1 | N/A | N/A | N/A |
| 24 | I/O_16 | 34 | 35 | C0 |
| 25 | I/O_17 | 36 | 37 | C2 |
| 26 | I/O_18 | 38 | 39 | C4 |

Continued...

Pin/Node Table, MACH231, Continued

| Pin # | Default Name | OutputMacro- cell |  | Pin Feedback |
| --- | --- | --- | --- | --- |
| 27 | I/O_19 | 40 | 41 | C6 |
| 28 | I/O_20 | 42 | 43 | C8 |
| 29 | I/O_21 | 44 | 45 | C10 |
| 30 | I/O_22 | 46 | 47 | C12 |
| 31 | I/O_23 | 48 | 49 | C14 |
| 32 | GND | N/A | N/A | N/A |
| 33 | I/O_24 | 64 | 65 | D14 |
| 34 | I/O_25 | 62 | 63 | D12 |
| 35 | I/O_26 | 60 | 61 | D10 |
| 36 | I/O_27 | 58 | 59 | D8 |
| 37 | I/O_28 | 56 | 57 | D6 |
| 38 | I/O_29 | 54 | 55 | D4 |
| 39 | I/O_30 | 52 | 23 | D2 |
| 40 | I/O_31 | 50 | 51 | D0 |
| 41 | I2 | N/A | N/A | N/A |
| 42 | VCC | N/A | N/A | N/A |
| 43 | GND | N/A | N/A | N/A |
| 44 | VCC | N/A | N/A | N/A |
| 45 | I/O_32 | 66 | 67 | E0 |
| 46 | I/O_33 | 68 | 69 | E2 |
| 47 | I/O_34 | 70 | 71 | E4 |
| 48 | I/O_35 | 72 | 73 | E6 |
| 49 | I/O_36 | 74 | 75 | E8 |
| 50 | I/O_37 | 76 | 77 | E10 |
| 51 | I/O_38 | 78 | 79 | E12 |
| 52 | I/O_39 | 80 | 81 | E14 |
| 53 | GND | N/A | N/A | N/A |
| 54 | I/O_40 | 96 | 97 | F14 |
| 55 | I/O_41 | 94 | 95 | F12 |
| 56 | I/O_42 | 92 | 93 | F10 |
| 57 | I/O_43 | 90 | 91 | F8 |

Continued...

Pin/Node Table, MACH231, Continued

| Pin # | Default Name | OutputMacro- cell |  | Pin Feedback |
| --- | --- | --- | --- | --- |
| 58 | I/O_44 | 88 | 89 | F6 |
| 59 | I/O_45 | 86 | 87 | F4 |
| 60 | I/O_46 | 84 | 85 | F2 |
| 61 | I/O_47 | 82 | 83 | F0 |
| 62 | 13/CLK2 | N/A | N/A | N/A |
| 63 | VCC | N/A | N/A | N/A |
| 64 | GND | N/A | N/A | N/A |
| 65 | I4/CLK3 | N/A | N/A | N/A |
| 66 | I/O_48 | 98 | 99 | G0 |
| 67 | I/O_49 | 100 | 101 | G2 |
| 68 | I/O_50 | 102 | 103 | G4 |
| 69 | I/O_51 | 104 | 105 | G6 |
| 70 | I/O_52 | 106 | 107 | G8 |
| 71 | I/O_53 | 108 | 109 | G10 |
| 72 | I/O_54 | 110 | 111 | G12 |
| 73 | I/O_55 | 112 | 113 | G14 |
| 74 | GND | N/A | N/A | N/A |
| 75 | I/O_56 | 128 | 129 | H14 |
| 76 | I/O_57 | 126 | 127 | H12 |
| 77 | I/O_58 | 124 | 125 | H0 |
| 78 | I//O_59 | 122 | 123 | H8 |
| 79 | I/O_60 | 120 | 1221 | H6 |
| 80 | I/O_61 | 118 | 119 | H4 |
| 81 | I/O_62 | 116 | 117 | H2 |
| 82 | I/O_63 | 114 | 115 | H0 |
| 83 | I4 | N/A | N/A | N/A |
| 84 | VCC | N/A | N/A | N/A |

# MACH355 Pin and Node SummaryXE "MACH355:pin and node summary"§XE "pin and node summary:MACH355"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Macrocell.XE "MACH355:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH355

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 1 | I/O_13 | 15 | A13 |
| 2 | I/O_14 | 16 | A14 |
| 3 | I/O_15 | 17 | A15 |
| 4 | VCC | N/A | N/A |
| 5 | TD1 | N/A | N/A |
| 6 | I5 | N/A | N/A |
| 7 | GND | N/A | N/A |
| 8 | I0/CLK0 | N/A | N/A |
| 9 | I1/CLK1 | N/A | N/A |
| 10 | I/O_16 | 33 | B15 |
| 11 | I/O_17 | 32 | B14 |
| 12 | VCC | N/A | N/A |
| 13 | I/O_18 | 31 | B13 |
| 14 | I/O_19 | 30 | B12 |
| 15 | GND | N/A | N/A |
| 16 | I/O_20 | 29 | B11 |
| 17 | I/O_21 | 28 | B10 |
| 18 | I/O_22 | 27 | B9 |
| 19 | I/O_23 | 26 | B8 |
| 20 | I/O_24 | 18 | B0 |
| 21 | I/O_25 | 19 | B1 |
| 22 | VCC | N/A | N/A |
| 23 | GND | N/A | N/A |
| 24 | I/O_26 | 20 | B2 |

Continued...

Pin/Node Table, MACH355, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 25 | I/O_27 | 21 | B3 |
| 26 | I/O_28 | 22 | B4 |
| 27 | I/O_29 | 23 | B5 |
| 28 | I/O_30 | 24 | B6 |
| 29 | I/O_31 | 25 | B7 |
| 30 | GND | N/A | N/A |
| 31 | TMS | N/A | N/A |
| 32 | TCK | N/A | N/A |
| 33 | VCC | N/A | N/A |
| 34 | I/O_32 | 49 | C15 |
| 35 | I/O_33 | 48 | C14 |
| 36 | I/O_34 | 47 | C13 |
| 37 | I/O_35 | 47 | C12 |
| 38 | GND | N/A | N/A |
| 39 | I/O_36 | 45 | C11 |
| 40 | I/O_37 | 44 | C10 |
| 41 | I/O_38 | 43 | C9 |
| 42 | I/O_39 | 42 | C8 |
| 43 | VCC | N/A | N/A |
| 44 | I/O_40 | 41 | C7 |
| 45 | I/O_41 | 40 | C6 |
| 46 | GND | N/A | N/A |
| 47 | I/O_42 | 39 | C5 |
| 48 | I/O_43 | 38 | C4 |
| 49 | I/O_44 | 37 | C3 |
| 50 | I/O_45 | 36 | C2 |
| 51 | I/O_46 | 35 | C1 |
| 52 | I/O_47 | 34 | C0 |
| 53 | VCC | N/A | N/A |
| 54 | GND | N/A | N/A |
| 55 | GND | N/A | N/A |
| 56 | VCC | N/A | N/A |
| 57 | I/O_48 | 50 | D0 |
| 58 | I/O_49 | 51 | D1 |

Continued...

Pin/Node Table, MACH355, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 59 | I/O_50 | 52 | D2 |
| 50 | I/O_51 | 53 | D3 |
| 61 | I/O_52 | 54 | D4 |
| 62 | I/O_53 | 55 | D5 |
| 63 | GND | N/A | N/A |
| 64 | I/O_54 | 56 | D6 |
| 65 | I/O_55 | 57 | D7 |
| 66 | VCC | N/A | N/A |
| 67 | I/O_56 | 58 | D8 |
| 68 | I/O_57 | 59 | D9 |
| 69 | I/O_58 | 60 | D10 |
| 60 | I/O_59 | 61 | D11 |
| 71 | GND | N/A | N/A |
| 72 | I/O_60 | 62 | D12 |
| 73 | I/O_61 | 63 | D13 |
| 74 | I/O_62 | 64 | D14 |
| 75 | I/O_63 | 65 | D15 |
| 76 | VCC | N/A | N/A |
| 77 | ENABLE | N/A | N/A |
| 78 | I2 | N/A | N/A |
| 79 | GND | N/A | N/A |
| 80 | I/O_71 | 73 | E7 |
| 81 | I/O_70 | 72 | E6 |
| 82 | I/O_69 | 71 | E5 |
| 83 | I/O_68 | 70 | E4 |
| 84 | I/O_67 | 69 | E3 |
| 85 | I/O_66 | 68 | E2 |
| 86 | GND | N/A | N/A |
| 87 | VCC | N/A | N/A |
| 88 | I/O_65 | 67 | E1 |
| 89 | I/O_64 | 66 | E0 |
| 90 | I/O_72 | 74 | E8 |
| 91 | I/O_73 | 75 | E9 |
| 92 | I/O_74 | 76 | E10 |

Continued...

Pin/Node Table, MACH355, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 93 | I/O_75 | 77 | E11 |
| 94 | GND | N/A | N/A |
| 95 | I/O_76 | 78 | E12 |
| 96 | I/O_77 | 79 | E13 |
| 97 | VCC | N/A | N/A |
| 98 | I/O_78 | 80 | E14 |
| 99 | I/O_79 | 81 | E15 |
| 100 | I3/CLK3 | N/A | N/A |
| 101 | I4/CLK4 | N/A | N/A |
| 102 | GND | N/A | N/A |
| 103 | TRST | N/A | N/A |
| 104 | TDO | N/A | N/A |
| 105 | VCC | N/A | N/A |
| 106 | I/O_80 | 97 | F15 |
| 107 | I/O_81 | 96 | F14 |
| 108 | I/O_82 | 95 | F13 |
| 109 | I/O_83 | 94 | F12 |
| 110 | GND | N/A | N/A |
| 111 | I/O_84 | 93 | F11 |
| 112 | I/O_85 | 92 | F10 |
| 113 | I/O_86 | 91 | F9 |
| 114 | I/O_87 | 90 | F8 |
| 115 | VCC | N/A | N/A |
| 116 | I/O_88 | 89 | F7 |
| 117 | I/O_89 | 88 | F6 |
| 118 | GND | N/A | N/A |
| 119 | I/O_90 | 87 | F5 |
| 120 | I/O_91 | 86 | F4 |
| 121 | I/O_92 | 85 | F3 |
| 122 | I/O_93 | 84 | F2 |
| 123 | I/O_94 | 83 | F1 |
| 124 | I/O_95 | 82 | F0 |
| 125 | VCC | N/A | N/A |
| 126 | GND | N/A | N/A |

Continued...

Pin/Node Table, MACH355, Continued

| Pin # | Default Name | Macro- cell | Pin Feedback |
| --- | --- | --- | --- |
| 127 | GND | N/A | N/A |
| 128 | VCC | N/A | N/A |
| 129 | I/O_0 | 2 | A0 |
| 130 | I/O_1 | 3 | A1 |
| 131 | I/O_2 | 4 | A2 |
| 132 | I/O_3 | 5 | A3 |
| 133 | I/O_4 | 6 | A4 |
| 134 | I/O_5 | 7 | A5 |
| 135 | GND | N/A | N/A |
| 136 | I/O_6 | 8 | A6 |
| 137 | I/O_7 | 9 | A7 |
| 138 | VCC | N/A | N/A |
| 139 | I/O_8 | 10 | A8 |
| 140 | I/O_9 | 11 | A9 |
| 141 | I/O_10 | 12 | A10 |
| 142 | I/O_11 | 13 | A11 |
| 143 | GND | N/A | N/A |
| 144 | I/O_12 | 14 | A12 |

# MACH435 Pin and Node SummaryXE "MACH435:pin and node summary"§XE "pin and node summary:MACH435"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Even Macro, Odd Macro, and Input Register.XE "MACH435:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH435

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A | N/A |
| 2 | VCC | N/A | N/A | N/A | N/A |
| 3 | IO_0 | 2 | 3 | 130 | A0 |
| 4 | IO_1 | 4 | 5 | 131 | A1 |
| 5 | IO_2 | 6 | 7 | 132 | A2 |
| 6 | IO_3 | 8 | 9 | 133 | A3 |
| 7 | IO_4 | 10 | 11 | 134 | A4 |
| 8 | IO_5 | 12 | 13 | 135 | A5 |
| 9 | IO_6 | 14 | 15 | 136 | A6 |
| 10 | IO_7 | 16 | 17 | 137 | A7 |
| 11 | GND | N/A | N/A | N/A | N/A |
| 12 | IO_8 | 32 | 33 | 145 | B7 |
| 13 | IO_9 | 30 | 31 | 144 | B6 |
| 14 | IO_10 | 28 | 29 | 143 | B5 |
| 15 | IO_11 | 26 | 27 | 142 | B4 |
| 16 | IO_12 | 24 | 25 | 141 | B3 |
| 17 | IO_13 | 22 | 23 | 140 | B2 |
| 18 | IO_14 | 20 | 21 | 139 | B1 |
| 19 | IO_15 | 18 | 19 | 138 | B0 |
| 20 | I0/CLK0 | N/A | N/A | N/A | N/A |
| 21 | VCC | N/A | N/A | N/A | N/A |
| 22 | GND | N/A | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH435, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 23 | I1/CLK1 | N/A | N/A | N/A | N/A |
| 24 | IO_16 | 34 | 35 | 146 | C0 |
| 25 | IO_17 | 36 | 37 | 147 | C1 |
| 26 | IO_18 | 38 | 39 | 148 | C2 |
| 27 | IO_19 | 40 | 41 | 149 | C3 |
| 28 | IO_20 | 42 | 43 | 150 | C4 |
| 29 | IO_21 | 44 | 45 | 151 | C5 |
| 30 | IO_22 | 46 | 47 | 152 | C6 |
| 31 | IO_23 | 48 | 49 | 153 | C7 |
| 32 | GND | N/A | N/A | N/A | N/A |
| 33 | IO_24 | 64 | 65 | 161 | D7 |
| 34 | IO_25 | 62 | 63 | 160 | D6 |
| 35 | IO_26 | 60 | 61 | 159 | D5 |
| 36 | IO_27 | 58 | 59 | 158 | D4 |
| 37 | IO_28 | 56 | 57 | 157 | D3 |
| 38 | IO_29 | 54 | 55 | 156 | D2 |
| 39 | IO_30 | 52 | 53 | 155 | D1 |
| 40 | IO_31 | 50 | 51 | 154 | D0 |
| 41 | I2 | N/A | N/A | N/A | N/A |
| 42 | VCC | N/A | N/A | N/A | N/A |
| 43 | GND | N/A | N/A | N/A | N/A |
| 44 | VCC | N/A | N/A | N/A | N/A |
| 45 | IO_32 | 66 | 67 | 162 | E0 |
| 46 | IO_33 | 68 | 69 | 163 | E1 |
| 47 | IO_34 | 70 | 71 | 164 | E2 |
| 48 | IO_35 | 72 | 73 | 165 | E3 |
| 49 | IO_36 | 74 | 75 | 166 | E4 |
| 50 | IO_37 | 76 | 77 | 167 | E5 |
| 51 | IO_38 | 78 | 79 | 168 | E6 |
| 52 | IO_39 | 80 | 81 | 169 | E7 |
| 53 | GND | N/A | N/A | N/A | N/A |
| 54 | IO_40 | 96 | 97 | 177 | F7 |
| 55 | IO_41 | 94 | 95 | 176 | F6 |
| 56 | IO_42 | 92 | 93 | 175 | F5 |

Continued...

Pin/Node Table, MACH435, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 57 | IO_43 | 90 | 91 | 174 | F4 |
| 58 | IO_44 | 88 | 89 | 173 | F3 |
| 59 | IO_45 | 86 | 87 | 172 | F2 |
| 60 | IO_46 | 84 | 85 | 171 | F1 |
| 61 | IO_47 | 82 | 83 | 170 | F0 |
| 62 | I3/CLK2 | N/A | N/A | N/A | N/A |
| 63 | VCC | N/A | N/A | N/A | N/A |
| 64 | GND | N/A | N/A | N/A | N/A |
| 65 | I4/CLK3 | N/A | N/A | N/A | N/A |
| 66 | IO_48 | 98 | 99 | 178 | G0 |
| 67 | IO_49 | 100 | 101 | 179 | G1 |
| 68 | IO_50 | 102 | 103 | 180 | G2 |
| 69 | IO_51 | 104 | 105 | 181 | G3 |
| 70 | IO_52 | 106 | 107 | 182 | G4 |
| 71 | IO_53 | 108 | 109 | 183 | G5 |
| 72 | IO_54 | 110 | 111 | 184 | G6 |
| 73 | IO_55 | 112 | 113 | 185 | G7 |
| 74 | GND | N/A | N/A | N/A | N/A |
| 75 | IO_56 | 128 | 129 | 193 | H7 |
| 76 | IO_57 | 126 | 127 | 192 | H6 |
| 77 | IO_58 | 124 | 125 | 191 | H5 |
| 78 | IO_59 | 122 | 123 | 190 | H4 |
| 79 | IO_60 | 120 | 121 | 189 | H3 |
| 80 | IO_61 | 118 | 119 | 188 | H2 |
| 81 | IO_62 | 116 | 117 | 187 | H1 |
| 82 | IO_63 | 114 | 115 | 186 | H0 |
| 83 | I5 | N/A | N/A | N/A | N/A |
| 84 | VCC | N/A | N/A | N/A | N/A |

# MACH445 Pin and Node SummaryXE "MACH445:pin and node summary"§XE "pin and node summary:MACH445"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Even Macro, Odd Macro, and Input Register.XE "MACH435:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH445

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A | N/A |
| 2 | GND | N/A | N/A | N/A | N/A |
| 3 | TDI | N/A | N/A | N/A | N/A |
| 4 | I5 | N/A | N/A | N/A | N/A |
| 5 | I0_8 | 32 | 33 | 145 | B7 |
| 6 | IO_9 | 30 | 31 | 144 | B6 |
| 7 | IO_10 | 28 | 29 | 143 | B5 |
| 8 | IO_11 | 26 | 27 | 142 | B4 |
| 9 | IO_12 | 24 | 25 | 141 | B3 |
| 10 | IO_13 | 22 | 23 | 140 | B2 |
| 11 | IO_14 | 20 | 21 | 139 | B1 |
| 12 | IO_15 | 18 | 19 | 138 | B0 |
| 13 | I0_CLK0 | N/A | N/A | N/A | N/A |
| 14 | VCC | N/A | N/A | N/A | N/A |
| 15 | VCC | N/A | N/A | N/A | N/A |
| 16 | GND | N/A | N/A | N/A | N/A |
| 17 | GND | N/A | N/A | N/A | N/A |
| 18 | I1_CLK1 | N/A | N/A | N/A | N/A |
| 19 | IO_16 | 34 | 35 | 146 | C0 |
| 20 | IO_17 | 36 | 37 | 147 | C1 |
| 21 | IO_18 | 38 | 39 | 148 | C2 |
| 22 | IO_19 | 40 | 41 | 149 | C3 |
| 23 | IO_20 | 42 | 43 | 150 | C4 |

Continued...

Pin/Node Table, MACH445, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 24 | IO_21 | 44 | 45 | 151 | C5 |
| 25 | IO_22 | 46 | 47 | 152 | C6 |
| 26 | IO_23 | 48 | 49 | 153 | C7 |
| 27 | TMS | N/A | N/A | N/A | N/A |
| 28 | TCK | N/A | N/A | N/A | N/A |
| 29 | GND | N/A | N/A | N/A | N/A |
| 30 | GND | N/A | N/A | N/A | N/A |
| 31 | IO_24 | 64 | 65 | 161 | D7 |
| 32 | IO_25 | 62 | 63 | 160 | D6 |
| 33 | IO_26 | 60 | 61 | 159 | D5 |
| 34 | IO_27 | 58 | 59 | 158 | D4 |
| 35 | IO_28 | 56 | 57 | 157 | D3 |
| 36 | IO_29 | 54 | 55 | 156 | D2 |
| 37 | IO_30 | 52 | 53 | 155 | D1 |
| 38 | IO_31 | 50 | 51 | 154 | D0 |
| 39 | VCC | N/A | N/A | N/A | N/A |
| 40 | GND | N/A | N/A | N/A | N/A |
| 41 | GND | N/A | N/A | N/A | N/A |
| 42 | VCC | N/A | N/A | N/A | N/A |
| 43 | IO_32 | 66 | 67 | 162 | E0 |
| 44 | IO_33 | 68 | 69 | 163 | E1 |
| 45 | IO_34 | 70 | 71 | 164 | E2 |
| 46 | IO_35 | 72 | 73 | 165 | E3 |
| 47 | IO_36 | 74 | 75 | 166 | E4 |
| 48 | IO_37 | 76 | 77 | 167 | E5 |
| 49 | IO_38 | 78 | 79 | 168 | E6 |
| 50 | IO_39 | 80 | 81 | 169 | E7 |
| 51 | GND | N/A | N/A | N/A | N/A |
| 52 | GND | N/A | N/A | N/A | N/A |
| 53 | ENABLE | N/A | N/A | N/A | N/A |
| 54 | I2 | N/A | N/A | N/A | N/A |
| 55 | IO_40 | 96 | 97 | 177 | F7 |
| 56 | IO_41 | 94 | 95 | 176 | F6 |
| 57 | IO_42 | 92 | 93 | 175 | F5 |

Continued...

Pin/Node Table, MACH445, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 58 | IO_43 | 90 | 91 | 174 | F4 |
| 59 | IO_44 | 88 | 89 | 173 | F3 |
| 60 | IO_45 | 86 | 87 | 172 | F2 |
| 61 | IO_46 | 84 | 85 | 171 | F1 |
| 62 | IO_47 | 82 | 83 | 170 | F0 |
| 63 | I3_CLK2 | N/A | N/A | N/A | N/A |
| 64 | VCC | N/A | N/A | N/A | N/A |
| 65 | VCC | N/A | N/A | N/A | N/A |
| 66 | GND | N/A | N/A | N/A | N/A |
| 67 | GND | N/A | N/A | N/A | N/A |
| 68 | I4_CLK3 | N/A | N/A | N/A | N/A |
| 69 | IO_48 | 98 | 99 | 178 | G0 |
| 70 | IO_49 | 100 | 101 | 179 | G1 |
| 71 | IO_50 | 102 | 103 | 180 | G2 |
| 72 | IO_51 | 104 | 105 | 181 | G3 |
| 73 | IO_52 | 106 | 107 | 182 | G4 |
| 74 | IO_53 | 108 | 109 | 183 | G5 |
| 75 | IO_54 | 110 | 111 | 184 | G6 |
| 76 | IO_55 | 112 | 113 | 185 | G7 |
| 77 | TRST | N/A | N/A | N/A | N/A |
| 78 | TDO | N/A | N/A | N/A | N/A |
| 79 | GND | N/A | N/A | N/A | N/A |
| 80 | GND | N/A | N/A | N/A | N/A |
| 81 | IO_56 | 128 | 129 | 193 | H7 |
| 82 | IO_57 | 126 | 127 | 192 | H6 |
| 83 | IO_58 | 124 | 125 | 191 | H5 |
| 84 | IO_59 | 122 | 123 | 190 | H4 |
| 85 | IO_60 | 120 | 121 | 189 | H3 |
| 86 | IO_61 | 118 | 119 | 188 | H2 |
| 87 | IO_62 | 116 | 117 | 187 | H1 |
| 88 | IO_63 | 114 | 115 | 186 | H0 |
| 89 | VCC | N/A | N/A | N/A | N/A |
| 90 | GND | N/A | N/A | N/A | N/A |
| 91 | GND | N/A | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH445, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 92 | VCC | N/A | N/A | N/A | N/A |
| 93 | IO_0 | 2 | 3 | 130 | A0 |
| 94 | IO_1 | 4 | 5 | 131 | A1 |
| 95 | IO_2 | 6 | 7 | 132 | A2 |
| 96 | IO_3 | 8 | 9 | 133 | A3 |
| 97 | IO_4 | 10 | 11 | 134 | A4 |
| 98 | IO_5 | 12 | 13 | 135 | A5 |
| 99 | IO_6 | 14 | 15 | 136 | A6 |
| 100 | IO_7 | 16 | 17 | 137 | A7 |

# MACH465 Pin and Node SummaryXE "MACH465:pin and node summary"§XE "pin and node summary, MACH435"§

| ã | Note: Node 1 is the global Set/Reset nodeXE "global Set/Reset node"§. Node numbers for other nodes are listed below under Even Macro, Odd Macro, and Input Register.XE "MACH435:global node"§XE "set and reset:global"§ |
| --- | --- |

Pin/Node Table, MACH465

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | N/A | N/A | N/A | N/A |
| 2 | TDI | N/A | N/A | N/A | N/A |
| 3 | IO_16 | 48 | 49 | 281 | C7 |
| 4 | IO_17 | 46 | 47 | 280 | C6 |
| 5 | IO_18 | 44 | 45 | 279 | C5 |
| 6 | IO_19 | 42 | 43 | 278 | C4 |
| 7 | IO_20 | 40 | 41 | 277 | C3 |
| 8 | IO_21 | 38 | 39 | 276 | C2 |
| 9 | IO_22 | 36 | 37 | 275 | C1 |
| 10 | IO_23 | 34 | 35 | 274 | C0 |
| 11 | VCC | N/A | N/A | N/A | N/A |
| 12 | GND | N/A | N/A | N/A | N/A |
| 13 | IO_24 | 64 | 65 | 289 | D7 |
| 14 | IO_25 | 62 | 63 | 288 | D6 |
| 15 | IO_26 | 60 | 61 | 287 | D5 |
| 16 | IO_27 | 58 | 59 | 286 | D4 |
| 17 | IO_28 | 56 | 57 | 285 | D3 |
| 18 | IO_29 | 54 | 55 | 284 | D2 |
| 19 | IO_30 | 52 | 53 | 283 | D1 |
| 20 | IO_31 | 50 | 51 | 282 | D0 |
| 21 | I2 | N/A | N/A | N/A | N/A |
| 22 | I3 | N/A | N/A | N/A | N/A |
| 23 | GND | N/A | N/A | N/A | N/A |
| 24 | VCC | N/A | N/A | N/A | N/A |
| 25 | VCC | N/A | N/A | N/A | N/A |
| 26 | GND | N/A | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 27 | GND | N/A | N/A | N/A | N/A |
| 28 | VCC | N/A | N/A | N/A | N/A |
| 29 | VCC | N/A | N/A | N/A | N/A |
| 30 | GND | N/A | N/A | N/A | N/A |
| 31 | I4 | N/A | N/A | N/A | N/A |
| 32 | IO_32 | 66 | 67 | 290 | E0 |
| 33 | IO_33 | 68 | 69 | 291 | E1 |
| 34 | IO_34 | 70 | 71 | 292 | E2 |
| 35 | IO_35 | 72 | 73 | 293 | E3 |
| 36 | IO_36 | 74 | 75 | 294 | E4 |
| 37 | IO_37 | 76 | 77 | 295 | E5 |
| 38 | IO_38 | 78 | 79 | 296 | E6 |
| 39 | IO_39 | 80 | 81 | 297 | E7 |
| 40 | GND | N/A | N/A | N/A | N/A |
| 41 | VCC | N/A | N/A | N/A | N/A |
| 42 | IO_40 | 82 | 83 | 298 | F0 |
| 43 | IO_41 | 84 | 85 | 299 | F1 |
| 44 | IO_42 | 86 | 87 | 300 | F2 |
| 45 | IO_43 | 88 | 89 | 301 | F3 |
| 46 | IO_44 | 90 | 91 | 302 | F4 |
| 47 | IO_45 | 92 | 93 | 303 | F5 |
| 48 | IO_46 | 94 | 95 | 304 | F6 |
| 49 | IO_47 | 96 | 97 | 305 | F7 |
| 50 | TMS | N/A | N/A | N/A | N/A |
| 51 | TCK | N/A | N/A | N/A | N/A |
| 52 | GND | N/A | N/A | N/A | N/A |
| 53 | GND | N/A | N/A | N/A | N/A |
| 54 | IO_48 | 112 | 113 | 313 | G7 |
| 55 | IO_49 | 110 | 111 | 312 | G6 |
| 56 | IO_50 | 108 | 109 | 311 | G5 |
| 57 | IO_51 | 106 | 107 | 310 | G4 |
| 58 | IO_52 | 104 | 105 | 309 | G3 |
| 59 | IO_53 | 102 | 103 | 308 | G2 |
| 60 | IO_54 | 100 | 101 | 307 | G1 |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 61 | IO_55 | 98 | 99 | 306 | G0 |
| 62 | GND | N/A | N/A | N/A | N/A |
| 63 | VCC | N/A | N/A | N/A | N/A |
| 64 | IO_56 | 128 | 129 | 321 | H7 |
| 65 | IO_57 | 126 | 127 | 320 | H6 |
| 66 | IO_58 | 124 | 125 | 319 | H5 |
| 67 | IO_59 | 122 | 123 | 318 | H4 |
| 68 | IO_60 | 120 | 121 | 317 | H3 |
| 69 | IO_61 | 118 | 119 | 316 | H2 |
| 70 | IO_62 | 116 | 117 | 315 | H1 |
| 71 | IO_63 | 114 | 115 | 314 | H0 |
| 72 | I5 | N/A | N/A | N/A | N/A |
| 73 | I6 | N/A | N/A | N/A | N/A |
| 74 | CLK1 | N/A | N/A | N/A | N/A |
| 75 | VCC | N/A | N/A | N/A | N/A |
| 76 | GND | N/A | N/A | N/A | N/A |
| 77 | GND | N/A | N/A | N/A | N/A |
| 78 | VCC | N/A | N/A | N/A | N/A |
| 79 | VCC | N/A | N/A | N/A | N/A |
| 80 | GND | N/A | N/A | N/A | N/A |
| 81 | GND | N/A | N/A | N/A | N/A |
| 82 | VCC | N/A | N/A | N/A | N/A |
| 83 | CLK2 | N/A | N/A | N/A | N/A |
| 84 | I7 | N/A | N/A | N/A | N/A |
| 85 | I8 | N/A | N/A | N/A | N/A |
| 86 | IO_64 | 130 | 131 | 322 | I0 |
| 87 | IO_65 | 132 | 133 | 323 | I1 |
| 88 | IO_66 | 134 | 135 | 324 | I2 |
| 89 | IO_67 | 136 | 137 | 325 | I3 |
| 90 | IO_68 | 138 | 139 | 326 | I3 |
| 91 | IO_69 | 140 | 141 | 327 | I5 |
| 92 | IO_70 | 142 | 143 | 328 | I6 |
| 93 | IO_71 | 144 | 145 | 329 | I7 |
| 94 | VCC | N/A | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 95 | GND | N/A | N/A | N/A | N/A |
| 96 | IO_72 | 146 | 147 | 330 | J0 |
| 97 | IO_73 | 148 | 149 | 331 | J1 |
| 98 | IO_74 | 150 | 151 | 332 | J2 |
| 99 | IO_75 | 152 | 153 | 333 | J3 |
| 100 | IO_76 | 154 | 155 | 334 | J4 |
| 101 | IO_77 | 156 | 157 | 335 | J5 |
| 102 | IO_78 | 158 | 159 | 336 | J6 |
| 103 | IO_79 | 160 | 161 | 337 | J7 |
| 104 | GND | N/A | N/A | N/A | N/A |
| 105 | GND | N/A | N/A | N/A | N/A |
| 106 | ENABLE | N/A | N/A | N/A | N/A |
| 107 | IO_80 | 176 | 177 | 345 | K7 |
| 108 | IO_81 | 174 | 175 | 344 | K6 |
| 109 | IO_82 | 172 | 173 | 343 | K5 |
| 110 | IO_83 | 170 | 171 | 342 | K4 |
| 111 | IO_84 | 168 | 169 | 341 | K3 |
| 112 | IO_85 | 166 | 167 | 340 | K2 |
| 113 | IO_86 | 164 | 165 | 339 | K1 |
| 114 | IO_87 | 162 | 163 | 338 | K0 |
| 115 | VCC | N/A | N/A | N/A | N/A |
| 116 | GND | N/A | N/A | N/A | N/A |
| 117 | IO_88 | 192 | 193 | 353 | L7 |
| 118 | IO_89 | 190 | 191 | 352 | L6 |
| 119 | IO_90 | 188 | 189 | 351 | L5 |
| 120 | IO_91 | 186 | 187 | 350 | L4 |
| 121 | IO_92 | 184 | 185 | 349 | L3 |
| 122 | IO_93 | 182 | 183 | 348 | L2 |
| 123 | IO_94 | 180 | 181 | 347 | L1 |
| 124 | IO_95 | 178 | 179 | 346 | L0 |
| 125 | I9 | N/A | N/A | N/A | N/A |
| 126 | I10 | N/A | N/A | N/A | N/A |
| 127 | GND | N/A | N/A | N/A | N/A |
| 128 | VCC | N/A | N/A | N/A | N/A |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 129 | VCC | N/A | N/A | N/A | N/A |
| 130 | GND | N/A | N/A | N/A | N/A |
| 131 | GND | N/A | N/A | N/A | N/A |
| 132 | VCC | N/A | N/A | N/A | N/A |
| 133 | VCC | N/A | N/A | N/A | N/A |
| 134 | GND | N/A | N/A | N/A | N/A |
| 135 | I11 | N/A | N/A | N/A | N/A |
| 136 | IO_96 | 194 | 195 | 354 | M0 |
| 137 | IO_97 | 196 | 197 | 355 | M1 |
| 138 | IO_98 | 198 | 199 | 356 | M2 |
| 139 | IO_99 | 200 | 201 | 357 | M3 |
| 140 | IO_100 | 202 | 203 | 358 | M4 |
| 141 | IO_101 | 204 | 205 | 359 | M5 |
| 142 | IO_102 | 206 | 207 | 360 | M6 |
| 143 | IO_103 | 208 | 209 | 361 | M7 |
| 144 | GND | N/A | N/A | N/A | N/A |
| 145 | VCC | N/A | N/A | N/A | N/A |
| 146 | IO_104 | 210 | 211 | 362 | N0 |
| 147 | IO_105 | 212 | 213 | 363 | N1 |
| 148 | IO_106 | 214 | 215 | 364 | N2 |
| 149 | IO_107 | 216 | 217 | 365 | N3 |
| 150 | IO_108 | 218 | 219 | 366 | N4 |
| 151 | IO_109 | 220 | 221 | 367 | N5 |
| 152 | IO_110 | 222 | 223 | 368 | N6 |
| 153 | IO_111 | 224 | 225 | 369 | N7 |
| 154 | TRST | N/A | N/A | N/A | N/A |
| 155 | TDO | N/A | N/A | N/A | N/A |
| 156 | GND | N/A | N/A | N/A | N/A |
| 157 | GND | N/A | N/A | N/A | N/A |
| 158 | IO_112 | 240 | 241 | 377 | O7 |
| 159 | IO_113 | 238 | 239 | 376 | O6 |
| 160 | IO_114 | 236 | 237 | 375 | O5 |
| 161 | IO_115 | 234 | 235 | 374 | O4 |
| 162 | IO_116 | 232 | 233 | 373 | O3 |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 163 | IO_117 | 230 | 231 | 372 | O2 |
| 164 | IO_118 | 228 | 229 | 371 | O1 |
| 165 | IO_119 | 226 | 227 | 370 | O0 |
| 166 | GND | N/A | N/A | N/A | N/A |
| 167 | VCC | N/A | N/A | N/A | N/A |
| 168 | IO_120 | 256 | 257 | 385 | P7 |
| 169 | IO_121 | 254 | 255 | 384 | P6 |
| 170 | IO_122 | 252 | 253 | 383 | P5 |
| 171 | IO_123 | 250 | 251 | 382 | P4 |
| 172 | IO_124 | 248 | 249 | 381 | P3 |
| 173 | IO_125 | 246 | 247 | 380 | P2 |
| 174 | IO_126 | 244 | 245 | 379 | P1 |
| 175 | IO_127 | 242 | 243 | 378 | P0 |
| 176 | I12 | N/A | N/A | N/A | N/A |
| 177 | I13 | N/A | N/A | N/A | N/A |
| 178 | CLK3 | N/A | N/A | N/A | N/A |
| 179 | VCC | N/A | N/A | N/A | N/A |
| 180 | GND | N/A | N/A | N/A | N/A |
| 181 | GND | N/A | N/A | N/A | N/A |
| 182 | VCC | N/A | N/A | N/A | N/A |
| 183 | VCC | N/A | N/A | N/A | N/A |
| 184 | GND | N/A | N/A | N/A | N/A |
| 185 | GND | N/A | N/A | N/A | N/A |
| 186 | VCC | N/A | N/A | N/A | N/A |
| 187 | CLK0 | N/A | N/A | N/A | N/A |
| 188 | I0 | N/A | N/A | N/A | N/A |
| 189 | I1 | N/A | N/A | N/A | N/A |
| 190 | IO_0 | 2 | 3 | 258 | A0 |
| 191 | IO_1 | 4 | 5 | 259 | A1 |
| 192 | IO_2 | 6 | 7 | 260 | A2 |
| 193 | IO_3 | 8 | 9 | 261 | A3 |

Continued...

Pin/Node Table, MACH465, Continued

| Pin # | Default Name | Even Macro | Odd Macro | Input Register | Pin Feedback |
| --- | --- | --- | --- | --- | --- |
| 194 | IO_4 | 10 | 11 | 262 | A4 |
| 195 | IO_5 | 12 | 13 | 263 | A5 |
| 196 | IO_6 | 14 | 15 | 264 | A6 |
| 197 | IO_7 | 16 | 17 | 265 | A7 |
| 198 | VCC | N/A | N/A | N/A | N/A |
| 199 | GND | N/A | N/A | N/A | N/A |
| 200 | IO_8 | 18 | 19 | 266 | B0 |
| 201 | IO_9 | 20 | 21 | 267 | B1 |
| 202 | 1O_10 | 22 | 23 | 268 | B2 |
| 203 | IO_11 | 24 | 25 | 269 | B3 |
| 204 | IO_12 | 26 | 27 | 270 | B4 |
| 205 | IO_13 | 28 | 29 | 271 | B5 |
| 206 | IO_14 | 30 | 31 | 272 | B6 |
| 207 | IO_15 | 32 | 33 | 273 | B7 |
| 208 | GND | N/A | N/A | N/A | N/A |

Larger numbers of product terms can be implemented using a technique called gate splitting (see the "Gate Splitting" section in Chapter 8, "Using the Fitter").

Macrocells 7 and 8 (zero-based numbering) of the MACH110 and MACH111 devices can receive clusters steered from only one neighboring macrocell.  Macrocells 0, 7, 8, and 15 can therefore implement equations of up to eight product terms.

The same equations shown for the MACH1xx design example will produce true input pairing on MACH 2xx devices when the Use automatic pin/node pairing? option of the Logic Synthesis Options form is set to "Y."  If the Use automatic pin/node pairing? option is not set to "Y," you can still produce true input pairing on MACH 2xx devices by writing explicit input pairing statements.  Refer to the "Explicit Pairing Rules and Behavior" section of Chapter 6, "Equations Segment In Depth," for details.

All MACH devices except the MACH465 can route global clock pin signals through the central switch matrix.  If you need clock/input functionality in a MACH465 design, use two device pins, one clock and one input, and wire them together on the circuit board (these pins must have different names in the design file).

Equations processed with the 22V10/MACH 1xx/2xx S/R compatibility option set to "Y" have the same form and polarity as that produced by the MACHXL version 1.0 Minimizer with the Ensure polarity after minimization is option of the Logic Synthesis Options form set to "Best for device."  Equations processed with the 22V10/MACH 1xx/2xx S/R compatibility option set to "N" have the same polarity regardless of whether the Ensure polarity after minimization is option is set to "Best for device" or "As specified in design": the equation is minimized to have the fewest product terms possible, and the polarity is matched to that specified in the design file using the MACH device's polarity XOR.

Refer to the Place and Route Data file (Design.PRD) to see how the Fitter implemented each type of signal.  This information is contained in the "Block x Macrocell (MCell) Cluster Assignments" portion of the report, in a column labeled "Sync/Async."  An "S" in the row corresponding to the signal means that the macrocell is in the synchronous mode; an "A" means the macrocell is in the asynchronous mode.

Global clocks 0 and 1 are accessible to all macrocells through the block clock mechanism.  Global clocks 2 and 3 are accessible only to synchronous macrocells through the block clock mechanism.

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock

Default clock