| 1 | Installing the MACHXL Software |
| --- | --- |

Contents

µHardware Requirements	2

Software Requirements	3

Installation Procedure	4

Updating System Files	6

AUTOEXEC.BAT	7

CONFIG.SYS	7

Creating a Windows Icon for MACHXL	8

The MACHXLXE "PALASM"§® software installation process takes about 10 minutes.

| ã | Note: Installing MACHXL software does not affect PALASMXE "PALASM"§® software that is installed on the same computer, as long as each application is installed in its own directory. Unless you specify otherwise, PALASM software is installed in a directory named \PALASM and MACHXL software is installed in its own directory, named \MACHXL. |
| --- | --- |

# Hardware RequirementsXE "Hardware Requirements"§

MACHXL software is supported in single-user, non-networked environmentsXE "network environments"§.  The following hardware is required:

A. 	An IBM® PC or 100% compatible computer equipped with a 386, 486, or Pentiumä microprocessor.

B.	A minimum of 500 kilobytes (kB) of system RAM available after all device drivers and TSRs have been installed.  At least eight megabytes (MB) memory is recommended.  More memory may be required for extremely large designs.

C.	A minimum of 6 MB of hard disk space available for MACHXL software (separate from disk space used for PALASM software, if installed).  An additional 5 MB of free disk space is recommended for processing designs.

D.	A 3-½" floppy disk drive.

E.	An EGA, VGA, or Super VGA monitor and graphics adapter.

| ã | Note: If you are using a monochrome screen XE "monochrome screen "§with a graphics board, use the following commands to set up your system after installing the software: CD \MACHXL\DAT [Enter] DEL MACHXL.PCX [Enter] SET MODE=MONO [Enter] (Include the last command in your AUTOEXEC.BAT file also.) Deleting the graphics file MACHXL.PCX forces the software to display a text-based start-up screen that is easier to read on monochrome screens, but has no other effect on software operation. |
| --- | --- |

F. 	Optionally, a parallel port for programming MACH devices that have JTAG via cable.  5-Volt in-circuit JTAG programming kits for MACH devices with JTAG are available from AMD.

| ã | Note: The JTAG cable cannot be used with some software keys. Remove any software keys from the parallel port before attaching the JTAG cable to the parallel port of your PC. Attach the other end to the 10 pin header on your JTAG board, as shown in the MACHPRO programming manual found in the 5-Volt In-Circuit JTAG Programming kit. |  |
| --- | --- | --- |

G.	Optionally, one of the following printersXE "printers"§:

r	IBM ProPrinterXE "IBM ProPrinter"§

r	Epson® FX 80 XE "Epson FX 80 "§series

r	HP® LaserJetXE "HP LaserJet"§® and HP LaserJet Series IIXE "HP LaserJet Series II"§

(The HP LaserJet printers must include a font cartridge XE "font cartridge "§with line-drawing capability, such as cartridge #HP3659A.)

| ã | Note: All printer output from the MACHXL software is text-based except for the simulation waveforms, which use the line-drawing characters of the extended IBM character set. |
| --- | --- |
| ã | Note: Refer to the README file on disk 1 for information that became available after this user's guide was printed. |

# Software RequirementsXE "Software Requirements"§

The minimum software requirement, which must be met before installing and running the MACHXL software is listed below.

A.	MS-DOS™ version XE "MS-DOS version "§5.0 or later

B.	Proper software driver files for peripheral devices

MACHXL software can be installed and run in a DOS box under Microsoft Windowsâ version 3.1 or higher. Directions on creating a Windows icon for MACHXL are presented later in this chapter.

| ã | Note: Check the README file on disk 1 for the latest information on OS/2âand Windows NT support. |
| --- | --- |
| ã | Note: You cannot run the MACHXL software from a floppy disk. You must use the MACHXL installation program to install the MACHXL software; simply copying the files to your hard disk does not work. The steps below guide you through the installation and configuration process. |

# Installation Procedure

Before you begin, you may want to check the path (typically C:\) to the file COMMAND.COM so you can complete the installation form appropriately.

1.	Place installation disk XE "installation disk "§1 into drive A or drive B.

2.	From the DOS prompt, type

A:INSTALL and press the Enter key

or to install from drive BXE "drive B, installing from"§, type

B:INSTALL and press the Enter key

Wait for the MACHXL title screen to appear.

3.	Press any key to proceed with the installation.

The installation menu and form appears; the menu covers part of the form.  The menu has two options:

r	Install MACHXL Software installs the software and related files. You must use this command initially.

r	Reinitialize MACHXL Setup files XE "Reinitialize MACHXL Setup files "§reinstates the standard SETUP.MXL file, and updates the CONFIG.SYS and AUTOEXEC.BAT files to let them run MACHXL software.

4.	Select Install MACHXL Software and press the Enter key.

The menu is dismissed and the form is now completely visible, as shown on the next page.

| ã | Note: If you do not ask the installation program to update files automatically, the changes added to the dummy files CONFIG.M94 and AUTOEXEC.M94, or to files you specify In this case, you may need to update information in the AUTOEXEC.BAT or CONFIG.SYS files manually before you can use the MACHXL software. See the "Updating System Files" section later in this chapter for details. If no changes are required to AUTOEXEC.BAT or CONFIG.SYS, no changes are made. |
| --- | --- |
| ã | Note: As of this printing, AMD could not guarantee support in OS/2 and network environments. See the README file on disk 1 for any last minute information on support for Windows NT, OS/2, and network environments. |

5.	To change options in the form, use the up- and down-arrow keys to highlight the desired field, type the appropriate information, then press the Enter key.  After you press the Enter key, the next field on the form is automatically highlighted.

6.	The default for the field Which parallel port will be used for programming via cable? is lpt1. This is the port used for programming MACH devices with JTAG.  To change it, use the up- and down-arrow keys to highlight the desired port (lpt1, lpt2 or lpt3), then press the Enter key.

| ã | Note: You must select a parallel port for programming via cable, even if you do not plan to use this feature. The port selection may be changed after installation through the Download menu, under Download:Program via cable:Program Device, as shown in Chapter 4, "Menu Reference." |
| --- | --- |

7.	When you are finished making changes to the data-entry fields on the form, press the F10 key to confirm your settings.

A window opens in the lower half of the screen and the process begins.  Messages keep you informed and prompt you to insert disks as required. A message signals the successful completion of installation.

8.	If you did not ask the installation program to change the AUTOEXEC.BAT and CONFIG.SYS files for you, follow the instructions in the "Updating System Files" section, below.  Otherwise, proceed directly to step 9.

9.	Remove the last installation diskette from the floppy drive and restart the computer by holding down the Ctrl and Alt keys while you press the Del key.

# Updating System Files

You are asked about updating the AUTOEXEC.BAT and CONFIG.SYS files on the installation form.  The following discussions indicate the information that is added during the automatic update.  If you did not specify an automatic update, you must ensure that this information appears in the appropriate file.

## AUTOEXEC.BATXE "AUTOEXEC.BAT, updating"§

| ã | Note: If you entered the letter N in the update AUTOEXEC.BAT field, you must manually edit the file to include the commands listed below. Remember to edit AUTOEXEC.BAT and restart your computer before invoking MACHXL. |
| --- | --- |

The following information must appear in the AUTOEXEC.BAT file before you can use the MACHXL software.

r	The PATH statement XE "PATH statement "§must include the directory containing the MACHXL executable files (named \MACHXL\EXE by default).

For example, if your old path statement was

PATH C:\DOS;C:\UTIL;C:\CAD

...and your system boots on drive c:, your new path statement should look like this:

PATH C:\MACHXL\EXE;C:\;C:\DOS;C:\UTIL;C:\CAD

r	A new variable that indicates the starting directory for the software:

SET MACHXL=C:\MACHXL\

| ã | Note: If you are using a monochrome screen, add the following line to your AUTOEXEC.BAT file: SET MODE=MONO |
| --- | --- |

## CONFIG.SYSXE "CONFIG.SYS, updating"§

| ã | Note: If you entered the letter N in the update CONFIG.SYS field, you must manually edit the file so that the FILES= variable is set to 35 or greater. |
| --- | --- |

If your CONFIG.SYS file already contains a FILES= environment variable XE "FILES= environment variable "§and if the FILES= variable is 35 or greater, no change is required.  Depending upon your system, it may be necessary to have FILES= to more than 35 if you have network drivers and TSRs.  If the FILES= variable does not exist or is less than 35, add the following line to the CONFIG.SYS file:

FILES=35

# Creating a Windows Icon for MACHXLXE "Hardware Requirements"§

Although MACHXL is not a Windows application, MACHXL may be run in a DOS box under Windows or Windows NT.  To create a Windows icon for MACHXL software after you have installed it, follow these steps:

1.	Start the Windows File Manager.

2.	Drag the file MACHXL.PIF from the \MACHXL\DAT directory to the Program Manager window of your choice. A place holder icon appears in the window.  (If you are using Windows NT, drag the MACHXLNT.PIF file instead of the MACHXL.PIF file.)

3.	Select this new place holder icon by clicking the mouse button once.

4.	Modify the properties of the MACHXL place holder icon by pressing ALT+ENTER. Enter the path to the file MACHXL.PIF or MACHXLNT.PIF. For example, if MACHXL is installed on the C drive in the directory MACHXL, type "C:\MACHXL\DAT\MACHXL.PIF" or "C:\MACHXL\DAT\MACHXLNT.PIF".

5.	Choose the Change Icon button, then enter the drive and location of MACHXL and its icon in the \MACHXL\DAT directory.  For example, if MACHXL is installed in C:\MACHXL, type 
"C:\MACHXL\DAT\-AMD.ICO".

The MACHXL software icon (a green AMD logo) is displayed.

6.	 Choose the OK button.

| ã | Note: If you have installed MACHXL anywhere other than in c:\MACHXL, you must also update the MACHXL.PIF file. Use the PIF editor to change the path under "Program Filename" to correctly represent your own path to the MACHXL\EXE\MACHXL.EXE file. Save the corrected MACHXL.PIF file. |
| --- | --- |
| ã | Note: When you start MACHXL software for the first time using the Windows icon, you will end up in the C:\ or in the C:\windows directory. Once you change to a design directory, MACHXL will remember that directory. The next time you start MACHXL software, you will automatically be placed in your design directory. |
| ã | Note: For more information on creating Windows icons, refer to your Microsoft Windows documentation. |