| 8 | Using the Fitter |
| --- | --- |

Contents

µOverview	281

The Fitting Process	281

Initialization	281

Normalization	282

Design Rule Check	282

Block Partitioning	282

Iterative versus Non-Iterative Partitioning	283

Manual Partitioning	284

Resource Assignment (Placement and Routing)	284

Designing to Fit	285

Methodology	285

Analyze Device Resources	286

Clock Signals	286

All Devices	286

MACH 3xx/4xx	286

MACH 215/3xx/4xx	287

Set/Reset Signals	288

Available Set and Reset Lines	288

MACH 3xx/4xx	288

Interaction of Set and Reset Signals (All Devices Except MACH215)	288

Reserving Unused Macrocells and I/O Pins	289

Product Terms	290

Strategies for Fitting Your Designs	291

Fitting with Unconstrained Pinout	293

Fitting with Constrained Pinout	293

Interconnection Resources	295

Oversubscribed Macrocells and/or Inputs	295

Large Functions at the End of a Block	296

Adjacent Macrocell Use	297

Grouping Logic	297

Setting Compilation and Fitting Options	298

Reducing Non-Forced Global Clocks	298

Gate Splitting	298

All MACH Devices	300

MACH 3xx/4xx Devices	300

Failure to Fit on Second Pass	302

Understanding Global Clock Signals	303

Balancing Clock Resources and Requirements	303

Global Clock Rules	304

Conditions Forcing Placement at a Global Clock Pin	305

Manually Forcing a Clock Signal to be Global	306

Conditions Forcing Non-Global Clocks	307

Resolving Contradictions	308

# OverviewXE "Fitter report:overview"§

The last phase of the compilation process for MACH-device designs is the fitting process.  During fitting, the design is mapped to the physical resources of the specified MACH device.   The goal of the fitting process is to discover a set of pin/node placements and signal routings that satisfy design requirements.

# The Fitting ProcessXE "fitting process"§

The fitting process consists of four phases.  An understanding of each phase can help you choose the best corrective action if the design does not fit.

r	Initialization

r	Block partitioning

r	Resource assignment

r	Report file generation

| ã | Note: Before the Fitter can operate, the Logic Minimizer must process the design. Like all of the earlier process modules, the Logic Minimizer produces a .TRE file that you can disassemble to study the effects of the Logic Minimizer on the design. The Logic Minimizer also produces a .PLA file, which the Fitter uses to complete the compilation process. |
| --- | --- |

## InitializationXE "fitting:initialization"§

One or both of the following files are read by the MACH Fitter during the initialization phase:

r	Design.PLA is produced during compilation and is always read by the Fitter.  It contains the target device type, signal information from pin and node statements, and the design description encoded in Boolean sum-of-products form.

r	Design.PLC contains data generated during the last successful fitting process or during a previous, saved fitting process.  It includes pin and node placement information that reflects the compilation and MACH fitting options you've specified.

Fitter initialization includes the following two processes:

### Normalization

Each clock signal is evaluated and classified as a global clock or a non-global clock.  The Fitter attempts to place all global clock signals at global clock pins (check the log file for the status of all clock signals after Normalization).  Undefined pins/nodes and nodes that are defined but not referenced are discarded from the design during Normalization (warning messages are generated).

Errors are reported if the design exceeds the device's product term, macrocell, pin, or clock resources.

### Design Rule Check

Information about the internal architecture of the specified device is loaded and resource checks are performed on the design.

## Block PartitioningXE "fitting:block partitioning"§XE "block partitioning"§XE "partitioning"§

After initialization, the design is segmented into individual blocks of the specified MACH device.  Segmentation is achieved by assigning logic to specific PAL blocks, based on the following considerations:

r	Individual signal preplacements and GROUP MACH_SEG_x block-grouping preplacements

r	A block's available internal resources (free macrocells, product terms, clock signals, and so forth)

r	The switch-matrix interconnect resources available to the block

### Iterative versus Non-Iterative Partitioning

The Partitioner considers commonality of signals, macrocell requirments, Set/Reset requirements, product-term requirements, and other factors to determine which partition is mostlikely to succeed in fitting the design.  Only partitions that are likely to succeed (according to the Partitioner's rules) are attempted, regardless of whether you select iterative or non-iterative partitioning.

If the Iterate between partition and place/route option of the MACH Fitting Options form is set to "Y," the Partitioner chooses the partition that is most likely to succeed, proceeds to the Resource Assignment phase described below, and attempts a finite number of placements.  If none of these placements result in a successful fit, the Partitioner uses the data from the last place-and-route attempt to pick a new partition and then the place-and-route cycle is repeated.  This continues until one of the following occurs:

r	The design is fitted successfully

r	The user-set time limit expires

r	All of the likely-to-succeed partitions have been exhausted

If the Iterate between partition and place/route option of the MACH Fitting Options form is set to "N," the Partitioner chooses the "best" partition (the one that is most likely to succeed, according to the Partitioner's rules) and performs exhaustive placement and routing until one of the following occurs:

r	The design is fitted successfully

r	The user-set time limit expires

r	All placements within the "best" partition have been exhausted

### Manual Partitioning

| ã | Note: Except for the purpose of matching a desired pinout, manually preplacing signals at pins should be a last resort. Before attempting this, try setting the Gate split max num. pterms per eqn option of the Logic Synthesis Options form to a lower value (which affects all equations in the design) or create a LIM file to reduce the number of macrocells and logic array inputs to be allocated in individual PAL blocks. (Refer to Appendix C, "Creating a LIM file," for more information.) |
| --- | --- |

Proper block partitioning is critical for a successful fit.  Block partitioning is usually best left to the Fitter, but you can manually guide this process by doing the following:

r	Preplacing portions of the logic in specific blocks using the reserved word, MACH_SEG_x, as a name in a GROUP statement (where x represents the letter that corresponds to a PAL block).  For example, the following statement preplaces signals A2, B3, and C4 in PAL block C:

GROUP  MACH_SEG_C  A2 B3 C4

r	Preplacing individual signals at physical pins and nodes (not recommended).

## Resource Assignment (Placement and Routing)XE "fitting:placment and routing"§

Placement XE "placement"§is the assignment of physical block resources such as I/O pins, XORs, registers, and product-term clusters to logic equations.

Routing XE "routing"§is the assignment of switch-matrix interconnect resources to logic equations.

In the placement phase of the fitting process, individual equations are assigned to physical resources, as follows:

r	Logic equations associated with specific pins are assigned first.

r	Buried logic functions are placed in the remaining unused macrocells.

r	Inputs are assigned to any available pins last.  These pins can be dedicated inputs pins, clock/input pins, or I/O pins that correspond to macrocells that are either unused or used to implement buried logic functions.

In the routing phase, the Fitter attempts to route input, output, and feedback signals to and from the physical resources assigned in the placement phase.  If the Fitter fails to route all signals, another placement is tried.  The Fitter continues trying different placements, and different routing options within each placement, until a successful fit is found or the time allotted for fitting is exceeded.

# Designing to FitXE "designing to fit"§

A clear understanding of the fitting process and the resources available in the MACH device can help you make sound decisions to achieve the density and performance you need.  Study the device data sheet for insights on how best to structure your designs to fit the target MACH device.  Decisions you make when entering the design and the logic synthesis, compilation, and fitting options affect the amount of logic that can fit in the device.  Some of your decisions also affect design performance.

| ã | IMPORTANT: The recommended methodology is to float all signals initially. With all signals floating, the software determines placements and has the greatest chance of achieving a successful fit. |
| --- | --- |

After finding a successful fit you can try modifying the placements to achieve a more desirable pinout.

## MethodologyXE "fitting:methodology"§

The following sections explain how to evaluate your design in terms of the MACH device's resources

### Analyze Device ResourcesXE "fitting:analyze device resources"§

A preliminary analysis of the device resources required by your design can help you identify potential resource deficiencies early.

#### Clock Signals

All MACH devices support multiple clock signals.  However, clock configurations differ across MACH families.

r	MACH1xx/2xx devices have either two or four clock pins.  All registers are synchronous: each register must be clocked by one of the global clocks.

r	The MACH215 device has two global clock pins.  Registers can be synchronous or asynchronous: each register can be clocked by one of the global clocks or by a product-term clock.

r	MACH 3xx/4xx devices have four global clock pins.  Registers can be either synchronous or asynchronous: each synchronous register must be clocked by one of the global clocks; each asynchronous register must be clocked by one of the global clocks or by a product-term clock.

##### All Devices

The Device-Resource Check portion of the fitting report shows the number of clock pins used in the design.  For example, a typical report may show the following:

Available       Used       Remaining

Clocks:            4             1            3

In the example above, only one clock pin is used, though four were available.  Three clock pins remain available in this case.

##### MACH 3xx/4xx

Each register can be either synchronous or asynchronous.  All four global clock signals (and their complements) are available to every synchronous macrocell in the device through the block clock mechanism (but not all combinations of clock polarity are available at the same time).  If your design requires more clock signals than there are global clock pins, you can define a product-term clock for some or all of the macrocells (but synchronous macrocells must use global clocks rather than product-term clocks).

##### MACH 215/3xx/4xx

It is important to understand how the Fitter determines whether a clock signal is global or non-global, because MACH 215/3xx/4xx devices can accommodate no more than four global clock signals.  (Refer to "Understanding Global Clock Signals" later in this chapter for more information.)

| ã | Note: The log file shows how each clock signal was implemented: as a global clock or as a product-term clock, and why. Refer to "Log File" in Chapter 9, "Report Files," for more information. |
| --- | --- |

Given adequate resources and the default menu options, the Fitter will place single-literal, floating, non-block-restricted clock signals at global clock pins.  If your MACH 3xx/4xx design requires fewer than four clocks for synchronous signals, you can facilitate fitting by reducing the number of clock signals that the Partitioner places at global clock pins.  (Refer to "Reduce Non-Forced Global Clocks Option" in this chapter for details.)

One way to speed partitioning is to block-restrict single-literal clock signals that are intended to be non-global.  (Refer to "MACH_SEG_x" in Chapter 5 for instructions on restricting signals to specific blocks.)

Avoid preplacing global clocks where possible.  Preplacing global clocks reduces partitioning flexibility and can, in some cases prevent the Fitter from finding a successful fit.  Preplacing a global clock signal also reduces the permutations of clock assignments, which can result in a failure to fit or require you to preplace all global clock signals.

#### Set/Reset SignalsXE "fitting:Set/Reset signals"§

When designing to fit, you must consider the following:

r	The number of Set and Reset lines available to each macrocell

r	How the Set and Reset signals of synchronous macrocells interact when they are partitioned into a common block

##### Available Set and Reset Lines

In synchronous mode, each macrocell has a full set of product terms (five before steering product terms from adjacent macrocells) as well as one line for each of the following: block clock, block Set, and block Reset.

##### MACH 3xx/4xx

In asynchronous mode, the maximum number of product terms available for logic equations in the macrocell (without steering product terms from adjacent macrocells) is reduced from five to three, as follows:

r	One of the original five product terms is reassigned to control either set or reset.  This gives each asynchronous macrocell either a Set or a Reset line (but not both).

r	One of the original five product terms is reassigned to define the clock product term for the macrocell.

The reduction in available product terms affects only the product-term cluster aligned with the asynchronous macrocell.  (Refer to "Asynchronous Mode" and "Cluster Size" in Chapter 10 for details.)

##### Interaction of Set and Reset Signals (All Devices Except MACH215)

The Fitter will avoid partitioning a synchronous signal in a PAL block in which the following conditions would exist:

r	The signal to be partitioned has a Set or a Reset condition, but not both.

r	Placement in that PAL block would cause the signal to inherit a Set or Reset condition that contains a term in common with the Set or Reset signal it already has.

This is true even if the SET/RESET treated as DONT CARE option in the Logic Synthesis Options form is set to "Y."

For example, the signal OUT2 that has a Reset equation OUT2.RSTF = X * Y will not be placed in a block in which it would inherit the Set condition X * Z, because the term "X" is common to both equations.  However, OUT2 could be placed in a block where it will inherit the Set equation /X * Z, because there is no overlap between "X" and "/X."

It is important to note that the Fitter does not check exhaustively for  overlapping Set and Reset conditions, but only for Set or Reset equations that have terms in common with the pre-existing Set or Reset conditions.

#### Reserving Unused Macrocells and I/O PinsXE "reserving unused macrocells and I/O pins"§

The addition of logic to a design that previously fit on a given MACH device can sometimes make it impossible to fit the design on the same device.  In some cases, the amount of new logic is not enough to prevent a successful fit, but does require changing the pinout.  To improve the odds of being able to add logic later, without changing the pinout, many designers add one or more "dummy" product terms to existing equations during design development and Fitting.  After the design is fit on a MACH device with the "dummy" product terms, the design file is back-annotated to lock in the pinout, the "dummy" product terms are commented out, and the Fitter is run again.

The process of adding "dummy" product terms is simple: each product ORed with the equation increases the equation's product-term utilization by one.  The only potential for difficulty arises because the Fitter discards from the finished design all pins and nodes that are unreferenced (not used in equations), even if they are declared properly using PIN or NODE statements.

Equations of the form

rpin1 = rpin1 + ... + rpinN

will inhibit the discard of pins rpin1, .. , rpinN, which are otherwise not used in the design file. The variable on the left side of the equation must be referenced (that is, appear on the right side of the same equation) in order not to be discarded. It is important to note that both product term and fanin requirements are imposed on the block where rpin1 ultimately resides and the size of those requirements is equal to the number of distinct variables on the right side of the equation. Therefore, if many pins are to be reserved during early iterations of a fitting process, it is prudent to write several equations using the form given above.

You can reserve an XOR term for the pin referenced on the left side of the equation by changing one of the '+' symbols on the right side to the XOR operator (:+:).

#### Product TermsXE "fitting:product terms"§

MACH devices have a varying number of product terms. For example, the MACH435 can support up to 640 product terms.

Five product terms are available to each macrocell in the MACH 3xx/4xx device in the synchronous mode (three in the asynchronous mode).  Synchronous equations with multiples of five product terms, and asynchronous equations with multiples of three product terms, make the most efficient use of device resources.  Equations with more product terms are realized using product-term steering or gate splitting.

r	(MACH 3xx/4xx only) Product-term steering uses resources from more than one macrocell but requires only one pass through the array.  Equations with up to 20 product terms (for synchronous operation, 18 product terms for asynchronous operation) in the can be implemented using this method.

r	If you enable the automatic gate-splitting option, equations containing more than the maximum number of product terms are implemented using gate splitting.  This requires multiple passes through the array and results in increased propagation delay.

Each method has its advantages.  Implementing a design without gate-splitting results in faster designs because propagation delay is reduced to a single pass.  On the other hand, gate-splitting facilitates fitting by reducing the maximum number of product terms needed for any one equation.  Equations with many product terms are often difficult to fit even if they are within nominal limits (for example, MACH 3xx/4xx devices have 20 product terms for combinatorial and synchronous macrocells, 18 product terms for asynchronous macrocells) because available clusters may have fewer product terms than expected.  Refer to "Cluster Size" in Chapter 10 for details.

# Strategies for Fitting XE "fitting:strategies"§Your DesignsXE "strategies for fitting"§

The MACHXL software attempts every possible signal placement within the partitioning arrangement chosen by the Fitter (unless you specify otherwise, for MACH 3xx/4xx designs only, by setting the Reduce Routes Per Placement? option of the MACH Fitting Options form to "Y.")Manual-assisted fitting consists of grouping certain signals in the same PAL block and/or bindingcertain signals to specific I/O pins.  When using MACHXL software, manual-assisted fitting will sometimes:

r	Fit a design that fails to fit automatically, especially if the failure to fit was the result of poor partitioning

r	Help designs that would fit anyway to fit faster and allow you to specify the desired pinout. Manual-assisted fitting can be done by placing signals at specific pins or by using GROUP  MACH_SEG_x statements to force the Fitter to partition certain signals in the same PAL block.  For example, the following statement forces the Fitter to partition signals A2, B3, and C4 into PAL block D:

GROUP  MACH_SEG_D  A2  B3  C4

The  fitting strategy for MACHXL is different from the strategies used to fit designs using PALASM 4 software.  The MACHXL fitting strategies can be divided into two classes:

r	Exhaustive fitting

r	Manual-assisted fitting

| Exhaustive fittingXE "exhaustive fitting"§ | To perform exhaustive fitting, float all pins and nodes, and set the following options in the MACH Fitting Options form as follows: |
| --- | --- |
|  | r Set Reduce Routes Per Placement? to "N" r Set Iterate between partition and place/route to "Y" r Set Run Time Upper Bound in 15 minutes to "0" If the design fails to fit, you will have to modify the design. |
| Manual-assisted fittingXE "manual-assisted fitting"§ | Manual-assisted fitting will sometimes find a fit for a design that failed to fit using exhaustive fitting. If you already know that the design will fit when partitioned a certain way and/or with fixed pin locations, manual-assisted fitting can greatly increase the speed of fitting. Manual-assisted fitting is also the only way to specify a desired pinout. |

There are two fundamental fitting scenarios, each of which requires a different approach:

r	Fitting a design with no regard to the resulting pinout.

r	Fitting a design in which the mapping of some or all I/O signals is constrained.  (This often happens when modifying an existing design or when substituting a different MACH device for the device on which the design was originally implemented.)

Fitting a design with no regard to the resulting pinout is the ideal, and recommended, situation, since it allows the Fitter maximum freedom to find a suitable fit.  Fixing the location of signals reduces the Fitter's opportunities to find a fit.  On the other hand, if the device can accommodate the design with the specified pinout, fixing the locations of signals can greatly reduce fitting time.

There is a different fitting strategy for each of these scenarios.  These strategies are covered in the following two subsections.

## Fitting with Unconstrained PinoutXE "unconstrained pinout"§XE "fitting:unconstrained pinout"§

The following diagram illustrates the procedure for fitting a design with unconstrained pinout.

µ §

## Fitting with Constrained PinoutXE "fitting:constrained pinout"§XE "constrained pinout"§

The diagram on the next page illustrates the procedure for fitting a design with constrained pinout.  Begin with the design file back-annotated from the previous fitting pass or manually specify the location of critical pins.

| ã | Note: Use GROUP MACH_SEG_x statements (refer to Chapter 5 for details) to partition equations manually into specific PAL blocks, if necessary. If you are sure the design will fit with all output- and buried- macrocell positions fixed, you can speed processing by beginning with the Handling of preplacements field on the MACH Fitting Options form set to "No Change." In all other cases, begin with this field set to "Float non-input nodes, groups" to maximize your chances of a successful fit. |
| --- | --- |

µ §

You can frequently reduce the amount of time it takes to fit a design by setting the Reduce Routes Per Placement XE "reduce routes per placement"§option in the MACH Fitting Options form (MACH 3xx/4xx only) to "Y."  Refer to the "MACH Fitting Options" section of Chapter 4 for more information on this option.

## Interconnection ResourcesXE "fitting:interconnection resources"§XE "interconnection resources"§

Utilization of interconnection resources is one factor in the fitting process.  The software considers device utilization as part of the MACH fitting process.  You can reduce device utilization using techniques described in the next three sections.  These techniques will improve the efficiency of fitting your design in a MACH device.

### Oversubscribed Macrocells and/or Inputs

If the Balanced partitioning field of the Logic Synthesis Options form is set to "N," the Partitioner places as many signals as possible in each block before moving on to the next block.  This may result in some blocks in which all available macrocells and/or input signals are used, while other blocks are relatively empty.

The easiest way to avoid oversubscribed macrocells is to leave the Balanced partitioning field of the Logic Synthesis Options form set to its default value: "Y."  You can also create a Partitioning Limit (LIM) control file to limit, on a block-by-block basis, the number of macrocells and/or inputs partitioned in one or more blocks, in order to balance resource utilization across blocks and speed fitting.  (Refer to Appendix C, "Creating an LIM File" for instructions.)

In many cases, reducing the maximum number of array inputs and the maximum number of macrocells that can be assigned to a block improves the speed of fitting.  There are some cases, however, in which reducing either of these resources can slow or prevent a successful fit.  For example, if there are eight signals that share 20 inputs in common, they obviously belong in the same block, since placing any one of them in a different block results in the immediate consumption of 20 array inputs in the second block, if inputs are not common to any other signals.  If the other signals that must be placed in the second block require more array inputs than are available, the strategy of reducing the first block's maximum number of macrocells will result in failure to fit a design that otherwise might have fit successfully.

The Place and Route Data (PRD) file shows how each block's resources are utilized.  This utilization data is useful in determining whether a LIM file is needed.  (Refer to "Using Place and Route Data to Limit Placements" in Chapter 9 for more information.)

### Large Functions at the End of a BlockXE "fitting:large functions at the end of a block"§

The macrocells at the end of a block have access to fewer product terms than other macrocells.

r	Cell number 0, the first cell in all MACH devices, can access the product term clusters from adjacent, higher-numbered cells (two clusters for MACH 2xx/3xx/4xx devices; one cluster for MACH 1xx devices), but cannot access any lower-numbered cells (cell 0 being the lowest-numbered cell in the block).  Therefore, equations assigned to the first cell in a block can use no more than n product terms (MACH 3xx/4xx devices: n = 15; MACH215 device: n = 8; MACH 1xx devices: n = 8; MACH 2xx devices: n = 12).

r	The last cell in a block can access the product term cluster from the adjacent lower-numbered cell, but cannot access any higher-numbered cells.  Therefore, equations assigned to the last cell in a block can use no more than n product terms (MACH 3xx/4xx devices: n = 10; MACH 1xx/2xx devices: n = 8).  This is not an issue if you float output and buried nodes, which should never be pre-placed under normal circumstances.

Refer to Chapter 10, "Device Reference," for more information.

### Adjacent Macrocell UseXE "fitting:adjacent macrocell use"§

If you want to preplace signals (not recommended unless pinout configuration is important), follow these guidelines:

r	Do not place large equations at the beginning or end of a PAL block.

r	Signals that share many common inputs should generally be grouped in the same PAL block (the Partitioner does this automatically).  Signals that do not share many common inputs should generally be distributed across several PAL blocks to avoid overburdening the switch matrix for a single block.

r	Leave adjacent macrocells empty in a MACH 4xx design when placing functions using double feedback and input registers.  Additional interconnection resources are needed for functions that use feedback from the output macrocell and the buried macrocell.   This is also true for functions that use input registers.  Leave adjacent macrocells empty when placing these functions.

### Grouping LogicXE "fitting:grouping logic"§XE "grouping logic"§

Block partitioning is one of the most important phases of the fitting process.  In this phase, the software segments the design into groups to be fit into blocks in the MACH device.  In general, manual attempts at partitioning through pin grouping (using the GROUP MACH_SEG_x syntax described in Chapter 5) do not help—and may hinder—the Fitter software in its task.  If you are updating a design, however, preserving the old signal grouping can sometimes speed fitting.  Manual grouping has the best chance of success when both of the following conditions are satisfied:

r	The new design makes only minor modifications to the original design.

r	The original design had excess resources in the PAL blocks to which you are making changes.

Fitting success is largely a function of the number of available placement permutations within and between blocks.  Logic grouping allows you to place a subset of the logic into a particular block without placing any other restrictions on specific cell placement.

If you do attempt manual grouping, try to place logic with common inputs and feedback in the same block.  This minimizes the number of signals crossing between blocks, which results in a lower demand for interconnection resources and an increased likelihood of a successful fit.

### Setting Compilation and Fitting OptionsXE "fitting:setting options"§

You can affect the fitting process by changing the setting of various compilation and fitting options.  A common design methodology is to use the default settings in the Compilation, MACH Fitting Options, and Logic Synthesis Options forms, then review the results of the compilation process.

#### Reducing Non-Forced Global Clocks

(MACH215 and MACH 3xx/4xx Devices Only)

The Reduce Non-forced Global Clocks? option on the MACH Fitting Options form, when set to "Y," allows you to restrict the Fitter's freedom to place such signals at global clock pins.  The quantity of single-literal, floating, non-block-restricted clock signals placed at global clock pins can be no greater than the total number of global clock pins minus the value assigned to the Reduce Non-forced Global Clocks? option.

#### Gate SplittingXE "gate splitting"§

Gate splitting is a technique by which equations that are too large to fit on the product terms available to a single macrocell are split among more than one macrocell.  The feedback from the additional macrocells is used to complete the required equation.  The advantage of gate splitting is that it allows you to implement equations that otherwise would be too large.  The disadvantage is the propagation delay that is introduced by each successive pass of a signal through the AND/OR array.

The Use automatic gate splitting option on the Logic Synthesis Options form controls the splitting of equations into smaller ones with fewer product terms.

Reducing the gate-splitting threshold is most useful if most equations are well below the maximum number of product terms the device can place at a macrocell and one or two equations exceed the maximum size.  In this case, the Minimizer's default method of operation is to place as many product terms as possible at each macrocell and split the remainder as required.

Example
A MACH211 design consists of six equations having 12 product terms each and two equations having 17 product terms each.  (A MACH211 macrocell can implement up to 16 product terms without gate-splitting.)  The Fitter can implement each of the six smaller equations as single-macrocell equations, but the two larger equations must be implemented using two macrocells each.  In its default mode, the Minimizer will split each of the 17-product-term equations into one equation of 16 product terms and one equation of 2 product terms (the extra product term is required to accept feedback from the second macrocell).

Reducing the gate-splitting threshold to 12 will result in less of an imbalance in the number of product terms placed at each macrocell.  Each of the original, 12-product-term equations remains at a single macrocell, while the two larger equations are again split into two macrocells each: one with 12 product terms and one with six product terms.  Thus, none of the equations is at the maximum capacity of its macrocell, which improves the odds of fitting the design and makes it easier to add logic to the design later.

| ã | Note: Do not reduce the gate-splitting threshold if doing so will cause many equations to be split. If, for instance, the preceding example's six smaller equations had contained 15 product terms each, setting the gate-splitting threshold to 12 would have caused all eight equations to be split, resulting in 16 under-utilized macrocells. |
| --- | --- |

The gate-splitting threshold option operates as follows:

r	If the option is set to "N" (the default setting), your equations will not be changed.  The Fitter will fail to fit any equation having more product terms than can be accommodated in a single macrocell using the macrocell's product terms and product terms steered to it from adjacent macrocells.

r	If the option is set to "Y," the Minimizer will perform gate splitting.   Every equation that contains more than the threshold number of product terms will be divided into multiple equations with fewer product terms each.  The gate-splitting threshold is defined as the lesser of the following:

u	The value you set using the Gate split max # pterms per eqn field of the Logic Synthesis Options form

u	MACH 1xx devices: 12 product terms.
MACH 2xx devices: 16 product terms
MACH 3xx/4xx devices: 20 product terms for combinatorial signals and signals that are unambiguously synchronous, 18 product terms for all registered signals other than unambiguously synchronous ones.  (Refer to "Synchronous vs. Asynchronous Operation" in Chapter 10 for details.)

The number of equations that result from gate splitting depends on a) the number of product terms in the original equation and b) the setting of the Gate split max # pterms per eqn field of the Logic Synthesis Options form.

##### All MACH Devices

The automatic gate-splitting feature eliminates the need for manual gate-splitting and iterative fitting attempts due to signals that have more product terms than the macrocells to which they are mapped.

##### MACH 3xx/4xx Devices

The Minimizer treats as asynchronous (split at 18 product terms) all registered signals that are not forced to be synchronous.  If you have an equation with 19 or 20 product terms that you want implemented without gate-splitting, you must provide conditions that are unambiguously synchronous.  (See "Forcing the Synchronous Mode" in Chapter 10 for details.)

Example
Consider the following:

r	An unambiguously synchronous registered equation with 22 product terms

r	Use automatic gate splitting field set to "Y"

r	Threshold set to 20

r	Gate split max # pterms per eqn field set to 20

The equation will be split into two equations, one with 20 product terms and one with 3 product terms, as shown below.

µ §

It will take two passes through the array to implement the new equations.

Using product-term steering for the MACH 3xx/4xx device, you can implement equations of the following sizes without gate-splitting:

r	Registered equations in asynchronous macrocells with up to 18 product terms

r	Registered equations in synchronous macrocells with up to 20 product terms

r	Combinatorial equations with up to 20 product terms

If you have a MACH 3xx/4xx design that contains equations with more than the number of product terms allowed by the corresponding macrocells, you must set the gate-splitting option to "Y" and set the threshold and Gate split max # pterms per eqn fields to appropriate values.

For maximum device speed, you should set the Use automatic gate splitting option to "N" and let the Fitter implement asynchronous equations with 18 or fewer product terms and synchronous or combinatorial equations with 20 or fewer product terms using product-term steering.  Equations implemented using this method require only one pass through the array.  Product-term steering also decreases the total demand for signal routing resources because no feedback signals are required.

After partitioning, the MACHXL placer looks for valid placements that satisfy product term requirements, MACH_SEG_x restrictions, and any preassignments.  After a placement is found, the router attempts to connect all the placed signals to the logic blocks requiring these signals.

If a route is not found, the router checks alternate routes through the input muxes.  If these alternate routes still do not succeed, the placer tries a new placement.

The MACHXL Fitter continues trying to fit the design until one of the following occurs:

r	The design fits

r	All possible placements are exhausted

r	The time limit you specified is reached

## Failure to Fit on Second Pass

In the unlikely event that a design that fit previously fails to fit after back-annotation or using a PLC file, follow these steps:

1.	Open the Compilation Options form (File:Set up:Compilation Options).

2.	Highlight the Handling of Preplacements field in the MACH Fitting Options form and press the F2 key to display the list of available options.

3.	Select the "Float non-input nodes, groups" option and press the Enter key.

4.	Press the F10 key to confirm your choice.

5.	Recompile the design.

Floating non-input nodes and groups does not affect device pinout but does allow the Fitter greater flexibility than does a fully-specified preplacement.

# Understanding Global Clock Signals

(MACH215 and MACH 3xx/4xx Devices Only)

A global clock is a signal that has two essential qualities:

r	It is selected for placement at a global clock pin

r	It is routable through the block clock mechanism

If you specify more than four clock signals in an all-synchronous MACH 3xx/4xx design, the fitting process will fail.  Synchronous macrocells must receive their clock signals through the block clock mechanism that is only available for clocks signals originating at one of the device's four global clock pins.

## Balancing Clock Resources and Requirements

Each MACH 3xx/4xx design can use up to four global clock signals.  Whether a given clock signal is global or product-term driven depends on the following rules.

If more than four clock signals that qualify for global implementation are defined, the four most-used clock signals are implemented as global clocks, if possible.  If clock signals are used an equal number of times, they are selected for global implementation in the order in which they appear in the .PLA file (see the "Initialization" section at the beginning of this chapter for information on the .PLA file).

The Fitter then attempts to reduce the number of clock signals that must be placed at global clock pins by removing forcing conditions.  For example, if a pin is input-paired and defined as registered, the input pairing forces the corresponding clock signal to be global.  If this is the only condition forcing that clock signal to be global, the Fitter removes the input-pairing and prints an appropriate warning message in the log file.

After the available global clocks are defined, all remaining clocks are product-term driven rather than global.

## Global Clock RulesXE "MACH 3XX/4XX:Global Clock Rules"§

MACH 3xx and 4xx devices permit inputs to the global clock pins to be routed two ways:

r	Signals from global clock pins 0, 1, 2, and 3 are available through the block clock mechanism to clock synchronous macrocells.  Signals from global clock pins 0 and 1 are available through the block clock mechanism to clock asynchronous macrocells.

r	Signals from all four global clock pins can be routed through the central switch matrix for use as logic inputs or product-term clocks.

The same inputs to clock/input pins (all devices except MACH465) can be routed both through the block clock mechanism and through the central switch matrix, simultaneously.  (Refer to "Global Clock Acquisition" in Chapter 6 for more information on the routing of global clocks.)

If a clock signal is defined as anything other than a single, combinatorial input pin, the clock must be product term-driven rather than global.  (A clock driven by feedback or a registered input is thus a product-term driven clock.)

If a clock signal is defined as a single input pin but the pin is placed manually at a pin other than one of the four global clock pins, the clock must be product-term driven rather than global.

### Conditions Forcing Placement at a Global Clock Pin

If one or more of the following conditions exists, a clock signal must be placed at a global clock pin:

r	It is the default clock.  (That is, it is neither declared nor used in any equations, but is implicitly necessary to clock registers or latches used in the design.)  The "MACH 3xx/4xx Design Considerations" section in Chapter 10, "Device Reference," lists the default clock pin for each device.

r	It is declared a pin and clocks or latch-enables either a) a preplaced, input-paired node or b) any other node that satisfies none of the forcing conditions for non-globality.  (Refer to the next section for forcing conditions for non-globality.)

r	It is declared a pin and controls a signal having both non-ground Set and non-ground Reset conditions.  (That is, it clocks or latch-enables a signal that must be placed in a synchronous macrocell because of Set/Rest requirements.)

r	It is declared a pin and controls a signal having more product terms than can be accommodated by an asynchronous macrocell.  (That is, it clocks or enables a signal that must be placed in a synchronous macrocell because of product-term requirements.)

r	It is preplaced at a global clock pin and the Global Clocks routable as  Pterm Clocks? option of the MACH Fitting Options form is set to "N."

### Manually Forcing a Clock Signal to be Global

According to the global clock rules described above, some clocks are forced to be implemented as global clocks, some are forced to be non-global, and others are not forced to be either global or non-global.  The portion of the log file (Design.LOG) that is generated by the Fitter's Normalization process reports the status of each clock signal.  If the log file shows that a clock you intended to be global was implemented as non-global, you can force a clock signal to be global by following these steps:

1.	Preplace the signal at a global clock pin.

2.	Set the Global Clocks routable as  Pterm Clocks? option of the MACH Fitting Options form to "N."

3.	Recompile the design.

| ã | Note: Preplacing clock signals at global clock pins can be useful in some situations, but can also inhibit fitting by reducing partitioning flexibility. |
| --- | --- |

### Conditions Forcing Non-Global Clocks

A non-global clock is one that is not global.  If a signal's clock is non-global, that signal must be implemented using an asynchronous macrocell.

If one or more of the following forcing conditions exists, a clock signal must be non-global:

r	It is declared a node.  This is true because feedback from nodes cannot be routed through the block clock mechanism, and all global clocks must be routable through the block clock mechanism.

r	It is an input-paired pin.  This is true because the global clock pins are physically incapable of being input-paired.

r	It appears on the left side of an equation.  This is true because global clock pins can be used for input only; the appearance of a signal on the left side of an equation implies output.

r	It is block-restricted.  (That is, it appears in a MACH_SEG_x statement.)  This is true because all global clock signals must be available to all blocks.

r	It is preplaced at a pin other than a global clock pin.

r	In MACH465 designs only, a clock must be non-global if one of the following conditions exists:

u	The clock name appears on the right side of any equation other than a .CLKF equation.

u	The clock name appears as a term in a multi-literal clock equation.

This is because the MACH465 device does not allow signals from the global clock pin to be routed through the central switch matrix, as they would have to be in order to be available as logic inputs.

### Resolving Contradictions

If a clock signal satisfies one or more of the conditions that would force it to be global and simultaneously satisfies one or more of the conditions that would prevent it from being global, the clock cannot be implemented and will cause a failure to fit.  Contradictions of this type are most likely to occur when adapting a design from a different MACH device to the MACH465, because the MACH465 is more restrictive than other MACH devices in the use of its global clock pins.  Such contradictions result in an error message from the Fitter.

To resolve the failure, you must modify the design to remove the contradiction.  If the error occurred because the signal was used in the design to clock both synchronous and non-synchronous macrocells, you can resolve the problem without affecting functionality by following these steps:

1.	Add a non-global clock input pin to the design.

2.	On the circuit board, route the external clock signal to both the non-global clock pin and global clock pin.

3.	Substitute the name of the non-global clock pin for the global clock pin in all equations other than the .CLKF equations for synchronous macrocells.

All MACH3xx/4xx devices except MACH465 have clock/input pins.

However, macrocells in the asynchronous mode can receive only block clock 0 and block clock 1 through the block clock mechanism.

If the design has many routing possibilities, you can often speed the fitting process by setting the Reduce Routes Per Placement? option (MACH 3xx/4xx designs only) to "Y."  With the option set to "Y," the Fitter tries only a few routes per placement.  If a fit is not found, the Fitter tries a different placement.

This assumes that each product-term cluster involved can contribute all of its original product terms.  However, available clusters frequently have fewer product terms available.  Refer to "Cluster Size" in this chapter for more information.

For the purpose of this discussion, the true and complement of a signal count as one signal because signal inversion can be done on a macrocell-by-macrocell basis through the flexible clock generator.

This will occur if there are conditions that force all of the registered signals to be implemented in synchronous macrocells.  If such forcing conditions do not exist, the excess clocks will be implemented as product-term clocks instead of global clocks; hence, the affected equations will be implemented in asynchronous macrocells.

Except on the MACH465 device.  When processing a MACH465 design, the Global clocks routable as PT clocks? option of the MACH Fitter Options form is suppressed.

In the MACH465 device, placement at a global clock pin is sufficient, because the Global Clocks routable as  Pterm Clocks? option is unavailable when processing a MACH465 design.  In other MACH devices, if the Global Clocks routable as  Pterm Clocks? option is set to "Y," the global clock signal will be placed at a global clock pin but the same signal will be available to other macrocells as a product-term clock through the central switch matrix.

This option appears when you process any MACH3xx/4xx devices except the MACH465.  In MACH465 designs, preplacing a signal at a global clock pin is sufficient to force the signal to be implemented as a global clock.