| A | State Segment In Depth |
| --- | --- |

Contents

µOverview	468

Defining Moore and Mealy Machines	469

Creating State-Machine Equations	470

Condition Equations	471

Transition Equations	471

Output Equations	472

State-Machine Example	472

Default Branches	474

Global Defaults	474

Local Defaults	474

Assigning State Bits	475

Automatic State-Bit Assignment	475

Manual State-Bit Assignment	476

Choosing State-Bit Assignments	477

Example Using Manual State-Bit Assignment	479

Using State Bits as Outputs	480

Initializing a State Machine	481

MACH 1xx/2xx Devices	481

MACH 3xx/4xx Devices	481

Illegal State Recovery	482

Clocking a State Machine	484

Example Using State Bits As Outputs, Start-Up, 
and CLKF	485

Multiple State Machines	486

# OverviewXE "STATE:overview"§

State syntax is supported for backward compatibility with PALASM 4 designs only.  AMD recommends that you implement new state machine designs using CASE statements, as described in Chapter 5.

The state-machine design file must include a program segment identified with the keyword STATE. This is called the state segment.

| ã | Note: It is possible to modify state equations with Boolean equations by including both equation and state segments, in any order. |
| --- | --- |

The state segment consists of the following syntax elements.

| Syntax | Definition |
| --- | --- |
| State | This identifies the state machine segment of the PDS file. |
| Machine-type | This identifies the state-machine type as either Moore or Mealy. |
| Start Up | This defines the state of the machine at power-up. |
| Global Defaults | This defines the default transitions if none of the specified conditions for a state are satisfied. |
| Transition Equations | This section defines the transitions from one state to the next. |
| Output Equations | This section defines the outputs for each possible state. |

Continued...

...Continued

| State Assignments | This optional section defines each state as a unique pattern of state bits. |
| --- | --- |
| Condition Equations | This section defines the set of inputs that represents each condition. |

# Defining Moore and Mealy MachinesXE "Moore machine, defined"§XE "Mealy machine, defined"§XE "STATE:Moore and Mealy machines"§

State-machine designs are divided into two basic types: Moore and Mealy.

r	Outputs in a Moore machine are dependent only on the present state.

r	Outputs in a Mealy machine are dependent on the present state and the present inputs.

You begin the state segment with the keyword STATE on a new line.  Then you define the state-machine type using one of the state-machine-type keywords.

MOORE_MACHINE

or

MEALY_MACHINE

The default is Mealy.

A state-machine design must be either all Moore or all Mealy.  The MACHXL software does not allow you to mix types in the same state machine. If even one state uses outputs that are input-dependent, you must convert the entire design to a Mealy machine.

| ã | Note: You can add Mealy features to a Moore Machine by writing a Boolean equation segment that further decodes the state machine's inputs and outputs. |
| --- | --- |

Another reason to convert a Moore design to Mealy is to reduce the total number of states in a design.  If you are running short of flip-flops in which to store state bits, you may be able to reduce the number of states, and thus the number of state bit flip-flops required, by implementing the design in Mealy form.  To reduce the number of states, the application must include cases in which multiple states can be collapsed down to a single state that produces different outputs depending on the inputs.

Do not convert a Mealy design to the Moore model unless Mealy-specific features are deleted.  If the Mealy design includes multiple transitions to the same state, each having different outputs, the equivalent Moore design will require additional states.  In some cases, a Moore design will not fit on a given device, while the same design implemented in Mealy form will fit.

## Creating State-Machine EquationsXE "state-machine equations, creating"§XE "STATE:creating state machine equations"§

There are four types of state-machine equations.  They have the following functions.

| Transition equations (required) | For each state, the equations specify what the next state will be under various conditions. See Condition Equations below. |
| --- | --- |
| Output Equations (optional) | These equations specify the outputs of the state machine. No output cases are required when the state bits themselves are the outputs. |
| Condition Equations (normally required) | These equations specify a condition name for each set of input values used to determine a transition. You can use input names directly only if a single input controls the transition; otherwise, you must use condition names. |
| State-Assignment Equations (optional) | These equations specify the bit code to be assigned to each state name used in the design. If these equations are omitted, the software will assign the bit codes automatically. |

### Condition EquationsXE "condition equations"§

You must replace each set of inputs that controls a transition with a logical name, called a condition.

The condition equations, preceded by the keyword CONDITIONS, must appear either before the keyword STATE or after all state-segment statements. CONDITIONS are written as simple Boolean equations.

CONDITIONS
Condition 1 = Boolean Expression
Condition 2 = Boolean Expression
...
Condition n = Boolean Expression

If a condition consists of a single input, you can use the input name instead of a condition equation.

If two conditions evaluate true at the same time, the software issues an overlapping condition error message.

>> ERROR Overlapping state transition conditions

To remove the overlapping conditions, you must write the equations so that no more than one equation can be evaluated as true at any time.

### Transition EquationsXE "transition equations"§

You must write at least one transition equation for each state. Within each state's transition equation, you must also write one expression to define each possible transition to a next state.

Use default branches to define the next state if the inputs fail to match any of the transition conditions defined for the present state.  Global defaults specify the default procedure for the entire state-machine design.  Local defaults specify the default procedure for one state only.

Present_state	:=	Condition_name 	-> Next_state
	+	Condition_name 	-> Next_state
	...
	+->	State_name   ;(this is the default branch)

### Output EquationsXE "output equations"§

To specify outputs for a Moore machine, you need to specify only the present state and the desired outputs, since the outputs are not affected by input conditions. The syntax for a Moore machine output equation follows.

State_name.OUTF = Output_expression

To specify outputs for a Mealy machine you must specify the input condition along with the present state.  The syntax for Mealy machine output equations is as follows.

State_name.OUTF 	=	Condition_1 ->	Output_expression_1
	+ 	Condition_2 -> Output_expression_2
	...

+-> 	Output_expression_n ;default output

The software allows you to specify the desired output pin values for each state or transition, without regard to the polarity of the device. The output equations are adjusted automatically to produce the requested behavior.

If you define the output pins as active low by using complemented pin names in the pin statements, the output pin will have the opposite value of the equation.

### State-Machine ExampleXE "state-machine, example"§

The following example shows a 3-bit up/down counter described in state-machine language.  The declaration segment is shown below.

;------------ Declaration Segment ------------------------
TITLE    COUNTER STATE MACHINE
...
CHIP  _CTR  MACH435
;------------ PIN Declarations ---------------------------
PIN     ?     CLOCK                  ;CLOCK
PIN     ?     ENABLE                 ;ENABLE
PIN     ?     UP_DWN                 ;INPUT
PIN     ?     CNT0        COMB       ;OUTPUT
PIN     ?     CNT1        COMB       ;OUTPUT
PIN     ?     CNT2        COMB       ;OUTPUT

Continued...

...Continued

;------------ State Segment ------------------------------

STATE
MOORE_MACHINE

ZERO	:= UP  -> ONE
	+ DOWN -> SEVEN
	+ STOP -> ZERO

ONE	:= UP  -> TWO
	+ DOWN -> ZERO
	+ STOP -> ONE

TWO	:= UP  -> THREE
	+ DOWN -> ONE
	+ STOP -> TWO

THREE	:= UP  -> FOUR
	+ DOWN -> TWO
	+ STOP -> THREE

FOUR	:= UP  -> FIVE
	+ DOWN -> THREE
	+ STOP -> FOUR
FIVE	:= UP  -> SIX
	+ DOWN -> FOUR
	+ STOP -> FIVE

SIX	:= UP  -> SEVEN
	+ DOWN -> FIVE
	+ STOP -> SIX

SEVEN	:= UP  -> ZERO
	+ DOWN -> SIX
	+ STOP -> SEVEN

ZERO.OUTF     = /CNT2 * /CNT1 * /CNT0
ONE.OUTF      = /CNT2 * /CNT1 *  CNT0
TWO.OUTF      = /CNT2 *  CNT1 * /CNT0
THREE.OUTF    = /CNT2 *  CNT1 *  CNT0
FOUR.OUTF     =  CNT2 * /CNT1 * /CNT0
FIVE.OUTF     =  CNT2 * /CNT1 *  CNT0
SIX.OUTF      =  CNT2 *  CNT1 * /CNT0
SEVEN.OUTF    =  CNT2 *  CNT1 *  CNT0

CONDITIONS
UP 	= ENABLE * UP_DWN
DOWN 	= ENABLE * /UP_DWN
STOP  	= /ENABLE

## Default BranchesXE "Default Branches"§

You use default branches to define the next state should the inputs fail to match any of the transition conditions defined for the present state.

The software supports two types of defaults.

r	Global defaults specify the default branch for all states except those for which local defaults are defined.

r	Local defaults specify the default branch for one state only.

You can include both local and global defaults in your design.  Local defaults will override global defaults.

### Global DefaultsXE "Global Defaults"§

Global defaults are defined after the machine-type definition. The global default statement can specify the default branch in one of three ways.  The statement below causes the state machine to remain in the same state if the inputs do not match any of the defined transition conditions for that state.

DEFAULT_BRANCH  HOLD_STATEXE "DEFAULT_BRANCH  HOLD_STATE"§

The following statement causes the state machine to branch to the specified state if the inputs do not match any of the defined transition conditions for that state.

DEFAULT_BRANCH  State nameXE "DEFAULT_BRANCH  State name"§

The next statement causes the state machine to branch to the next state if the inputs do not match any of the defined transition conditions for that state. The next state is defined as the state whose transition equation follows the transition equation for the present state in the PDS file. There is no next-state branch possible from the state whose transition equations appear last.

DEFAULT_BRANCH  NEXT_STATEXE "DEFAULT_BRANCH  NEXT_STATE"§

### Local DefaultsXE "Local Defaults"§

Unlike global defaults, local defaults always specify a branch to a specific state. Local defaults can be used alone or in combination with global defaults.

In combination with global defaults, local defaults provide a mechanism for defining default branches that differ from the norm.

Used alone, local defaults offer a way to specify each default branch explicitly. Local defaults allow you to see all possible branches from a given state at one glance.

Local defaults appear as the last line in a transition equation, using the special symbol +->, which is formed by typing the characters +, -, and >.

Present_state 	:= 	Condition_name -> Next_state
	+  	Condition_name -> Next_state
	.
	.
	.
	+->	State_name   ;default  branch

## Assigning State BitsXE "State Bits, assigning"§XE "Assigning State Bits"§

In some applications, you must control the assignment of the state-bit code.  However, most of the time the state-bit code is not important as long as it allows the device to differentiate between states.

### Automatic State-Bit AssignmentXE "State-Bit Assignment:automatic"§XE "Automatic State-Bit Assignment"§

You can allow the software to assign state bit-codes to state registers automatically. To do this, simply omit the state assignment equations. When the file is compiled, the software displays the following type of message to the screen and writes it to the log file.

|> WARNING E1351  Automatically assigning state bit 
                  _ST0 to ? NODE.

|> WARNING E1351  Automatically assigning state bit 
                  _ST1 to ? NODE.\

|> WARNING E1351  Automatically assigning state bit 
                  _ST2 to ? NODE.

STATE REGISTERS USED

Continued...

...Continued

PIN NUMBER:     PIN NAME:
? NODE_           _ST0
? NODE            _ST1
? NODE            _ST2

STATE BIT ASSIGNMENT USED

STATE NAME:  STATE REGISTERS VALUES:
             _ST2       _ST1     _ST0
ZERO           0         0        0
ONE            0         0        1
TWO            0         1        0
THREE          0         1        1
FOUR           1         0        0
FIVE           1         0        1
SIX            1         1        0
SEVEN          1         1        1

The warning message lists the pins to which state bits were assigned and the state-bit code for each state.  In the 3-bit counter example, three-state registers are used to allow for 8 possible states.  These are defined as nodes and named _ST0, _ST1 and _ST2.

State ZERO is assigned the bit code 0,0,0 which means all the state registers are low.  State ONE is assigned bit code 0,0,1.  A bit code for each state is listed with the message.

The first state defined in the transition equations is the first to be assigned a state code.  If there is no start-up statement, the software assigns the first state all zeros when the device specifies power-up reset, and all ones when the device specifies power-up preset.

### Manual State-Bit AssignmentXE "State-Bit Assignment:manual"§XE "Manual State-Bit Assignment"§

You can control state-bit assignment manually using state assignment equations.  To do this, you must define a pin or node for each of the state bits.  You do this in the declaration segment of the PDS file just as you would define any pin or node.

Then, in the STATE segment, you write an equation for each state specifying the value of the state bits in Boolean format.

State_name = Boolean expression

If you don't need to use the state bits as outputs and the device you are using contains buried flip-flops, you can assign state bits to them.  This will save output pins that can be used for other purposes.

### Choosing State-Bit AssignmentsXE "State-Bit Assignment:choosing"§

The state-bit assignments you choose have a large impact on the number of product terms that will be required to implement your design.  If you choose assignments so that the state-register bits change by only one bit at a time, as the state machine goes from state to state, the number of product terms will often be reduced.

For example, consider a design consisting of four states, A, B, C and D, where the transition between states is alphabetical.  One possible assignment is to use a simple binary count as follows.

| State | Bit Assignment |
| --- | --- |
| A | 00 |
| B | 01 |
| C | 10 |
| D | 11 |

Notice that this assignment causes two bits to change as the machine moves from state B to state C.

The following is a better assignment for product-term reduction.

| State | Bit Assignment |
| --- | --- |
| A | 00 |
| B | 01 |
| C | 11 |
| D | 10 |

Notice that this assignment causes only one bit to change as the machine moves from B to C.

If you need to use the state bits as outputs to identify when the machine is in a particular state, you can minimize the number of required outputs by choosing state bits appropriately.

For example, consider a design that has six states, A through F, where you need to identify states C, D and E.  The following assignment allows you to identify these states using only one output pin.

| State | BIT2 | BIT1 | BIT0 |
| --- | --- | --- | --- |
| A | 0 | 0 | 0 |
| B | 0 | 0 | 1 |
| C | 1 | 0 | 1 |
| D | 1 | 1 | 1 |
| E | 1 | 1 | 0 |
| F | 0 | 1 | 0 |

This assignment lets you use BIT2 as an output to identify when the machine is in any of the three states of interest.  BIT2 can be assigned to an output pin and BIT1 and BIT0 can be assigned to buried nodes, freeing output pin resources.

### Example Using Manual State-Bit AssignmentXE "State-Bit Assignment:manual, example"§

The following example uses state-assignment equations to manually assign the state bits to nodes named BIT0, BIT1 and BIT2.

;------------ Declaration Segment ------------------------
TITLE  COUNTER STATE MACHINE WITH STATE BIT ASSIGNMENT
...
CHIP  _CTR  MACH435
;------------ PIN Declaration-----------------------------
PIN     ?     CLOCK                   ;CLOCK

PIN     ?     ENABLE                  ;ENABLE
PIN     ?     UP_DWN                  ;INPUT
PIN     ?     CNT0     COMB           ;OUTPUT
PIN     ?     CNT1     COMB           ;OUTPUT
PIN     ?     CNT2     COMB           ;OUTPUT
NODE    ?     BIT0     REGISTERED     ;OUTPUT
NODE    ?     BIT1     REGISTERED     ;OUTPUT
NODE    ?     BIT2     REGISTERED     ;OUTPUT
;------------ State Segment ------------------------------
STATE
MOORE_MACHINE
DEFAULT_BRANCH ZERO

ZERO      := UP  -> ONE
          + DOWN -> SEVEN

ONE       := UP  -> TWO
          + DOWN -> ZERO

TWO       := UP  -> THREE
          + DOWN -> ONE

THREE     := UP  -> FOUR
          + DOWN -> TWO

FOUR      := UP  -> FIVE
          + DOWN -> THREE

FIVE      := UP  -> SIX
          + DOWN -> FOUR

SIX       := UP  -> SEVEN
          + DOWN -> FIVE

SEVEN     := UP  -> ZERO
          + DOWN -> SIX

ZERO         =  /BIT2 * /BIT1 * /BIT0
ONE          =  /BIT2 * /BIT1 *  BIT0
TWO          =  /BIT2 *  BIT1 * /BIT0
THREE        =  /BIT2 *  BIT1 *  BIT0
FOUR         =   BIT2 * /BIT1 * /BIT0

Continued...

Continued...

FIVE         =   BIT2 * /BIT1 *  BIT0
SIX          =   BIT2 *  BIT1 * /BIT0
SEVEN        =   BIT2 *  BIT1 *  BIT0

ZERO.OUTF    =  /CNT2 * /CNT1 * /CNT0
ONE.OUTF     =  /CNT2 * /CNT1 *  CNT0
TWO.OUTF     =  /CNT2 *  CNT1 * /CNT0
THREE.OUTF   =  /CNT2 *  CNT1 *  CNT0
FOUR.OUTF    =   CNT2 * /CNT1 * /CNT0
FIVE.OUTF    =   CNT2 * /CNT1 *  CNT0
SIX.OUTF     =   CNT2 *  CNT1 * /CNT0
SEVEN.OUTF   =   CNT2 *  CNT1 *  CNT0

CONDITIONS
UP   = ENABLE *  UP_DWN
DOWN = ENABLE * /UP_DWN

### Using State Bits as OutputsXE "State Bits as Outputs"§XE "Using State Bits as Outputs"§

Combining the state and output functions allows you to use less resources than if you use separate state bits and output bits. This can sometimes allow you to implement a design in a device that could not otherwise accommodate it.

Due to practical considerations, you can occasionally create a state-machine design where all of the outputs are also used as state bits. To do this, your design must meet three conditions.

r	All state bits must be stored in flip-flops that are associated with output or I/O pins.

r	The desired output in each state must be different from the desired output in every other state.

r	The outputs in the design that combine state bits and outputs cannot be combinatorial, since the state bits must be registered.

To use state bits as outputs, you write state-assignment equations.  Make sure the state bits are assigned to registered pins in the declaration segment of the PDS file.  Then you simply omit the output equations from the design.

## Initializing a State MachineXE "state machines:initializing"§XE "Initializing a State Machine"§

You use initialization routines to ensure the state machine powers up in a known state or branches to a known state whenever the initialization condition occurs.

### MACH 1xx/2xx DevicesXE "state machines:initializing::MACH 1xx/2xx devices"§

The START_UP command allows you to specify the starting state for devices that always power up with all bits high or all bits low, or that can be programmed to power up in any configuration.

The following is the syntax for Moore and Mealy machines:

START_UP := POWER_UP	->	State_name

The following is the syntax for Mealy machine output initialization:

START_UP.OUTF := POWER_UP	-> Outputs

The power-up parameter has the following effects.

r	In devices that initialize with all flip-flops high or all flip-flops low, the START_UP command assigns the appropriate all-high or all-low state-bit code to the specified state.

r	In devices with programmable power up, the START_UP command programs the device to power up in the specified state. If you specify a particular state-bit code using the manual state-bit assignment syntax, the software programs the flip-flops to initialize with the specified values.

If you do not include a start-up statement, the device will power up in the state that appears in the first transition equation in the PDS file.

### MACH 3xx/4xx DevicesXE "state machines:initializing::MACH 1xx/2xx devices"§

The START_UP and POWER_UP keywords (described in the previous section) are not supported for the MACH 3xx/4xx devices.

In certain modes of operation, the MACH 3xx/4xx devices pose challenges to the designer who wants to initialize a state machine to a known state of state registers, output pins, or both.  These challenges arise from the following causes:

r	Unpredictable power-up state for individual registers

r	The possibility, related to the problem of unpredictable power-up, that the state machine can power up in an undefined state

If the 22V10/MACH1XX/2XX S/R Compatibility? option in the MACH Fitting Options form is set to "Y," the Fitter is free to swap Set and Reset lines for macrocells to which active-low equations are mapped.  In addition, the Fitter is free to swap Set and Reset lines in order to get a successful fit, even if there is no active-low equation.  Power-up initialization is done by asserting the Reset line upon orderly power-up.  If the Set and Reset lines were swapped for some macrocells, those macrocells will power up in a high state instead of the expected low state.

It is possible, but tedious, to refer to the Block Partitioning Summary of the Fitter Report (see Chapter 9 for details on the Fitter Report) to determine if the Fitter swapped the Set and Reset lines for any of the macrocells in which you have an interest.  If swapping has occurred, you can rewrite the design so that all relevant equations are written with the same polarity (if all are active high, the state machine will power up all zeroes; if all are active low, the state machine will power up all ones).

Sometimes it is simpler to accept the possibility that the state machine may start up in a undefined state, and use an illegal state recovery scheme to drive the machine to a known state, as described in the next section.

# Illegal State RecoveryXE "STATE:Illegal State Recovery"§XE "Illegal State Recovery"§

An illegal, or undefined, state condition occurs whenever the state machine's state bits assume a state for which no transition equation exists.  If the power-up state is unknown, the state machine can start up in an unknown state from which no transition conditions are defined.  Unless your design uses all possible state-bit combinations, it is possible that the device will power up in an unknown state.  In this case, you must provide additional Boolean logic to force the state machine to a known starting state if for any reason it reaches an unknown state.  If the number of possible states is only slightly greater than the number of defined states, the easiest way to do this is simply to define the remaining possible states and write, for each one, an equation specifying an unconditional transition to the desired starting state.  If the number of undefined states is large, the following remedy may be easier than writing a large number of state definition and state transition equations:

Write a simple Boolean equation in the EQUATIONS segment of the PDS file to detect an illegal state condition and force the state machine into a known state.  You must write one equation for each state bit.  The general form of the equation is:

Desired_state-bit_value = /(ORed list_of_all_valid_state-bit_patterns)

Consider the case of a state machine with two states:

STATE1 = /BIT1 * /BIT2
STATE2 = /BIT1 *  BIT2

There are two possible conditions under which the state machine will be in an illegal state:

BIT1 * /BIT2
BIT1 *  BIT2

To write a set of equations that force the state machine to STATE1 whenever an illegal state condition occurs, write two equations, one for /BIT1 (BIT1 = 0 in STATE1) and /BIT2 (BIT2 = 0 in STATE1):

/BIT1 = /(  (/BIT1 * /BIT2) + (/BIT1 * BIT2)  )
/BIT2 = /(  (/BIT1 * /BIT2) + (/BIT1 * BIT2)  )

For the sake of simplicity, and in large designs, it helps to define a STRING statement for each state's state-bit pattern, and use these in the illegal state recovery equations.  The following code fragment shows how this is done.

...
STRING  STATE_1 '(/BIT1 * /BIT2)'
STRING STATE_2 '(/BIT1 * BIT2)'
...
EQUATIONS
/BIT1 = /(STATE_1 + STATE_2)
/BIT2 = /(STATE_1 + STATE_2)
...
STATE
...

| ã | Caution: You must add logic to control or suppress unwanted outputs until the state machine enters a known state. |
| --- | --- |
| ã | Note: You cannot use the automatic state bit assignment feature if you want to implement illegal state recovery, because you must reference the state bits by name in the EQUATIONS segment of the design file, and thus must define them explicitly in the STATE segment. |

## Clocking a State MachineXE "State Machine, clocking"§XE "Clocking a State Machine"§

The clock input to the state registers is normally connected to the default clock.  For devices with multiple clock sources or clocks formed by product terms, there are two ways to use a clock other than the default.

The clock source equation is placed in the state segment of a PDS file and is used to specify a clock signal for all flip-flops in the state machine.

The following is syntax for clock source equations.

CLKF = Clock Signal

The .CLKF function equation is placed in the equation segment of the PDS file.  To use this method, you must declare the state registers, manually assign the state bits, and write a .CLKF equation for each register in the state machine.

### Example Using State Bits As OutputsXE "State Bits As Outputs, example"§, Start-UpXE "Start-Up, example"§, and CLKFXE "CLKF, example"§

The following example modifies the 3-bit counter design to add a power-up routine, use the state bits as outputs, and specify a clock signal other than the default.

Notice that the state bits have been defined as pins instead of nodes and the output equations have been removed.

;------------ Declaration Segment ------------------------
TITLE COUNTER STATE MACHINE USING STATE BITS AS OUTPUTS, INITIALIZATION AND NON-DEFAULT CLOCKING
...
CHIP  _CTR  MACH111

;------------ PIN Declarations ---------------------------
PIN     ?     CLOCK                   ;CLOCK
PIN     ?     ENABLE                  ;ENABLE
PIN     ?     UP_DWN                  ;INPUT
PIN     ?     BIT0     REGISTERED     ;OUTPUT

PIN     ?     BIT1     REGISTERED     ;OUTPUT
PIN     ?     BIT2     REGISTERED     ;OUTPUT
;------------ State Segment -----------------------------

STATE
MOORE_MACHINE

START_UP := POWER_UP -> ZERO
CLKF = CLOCK
DEFAULT_BRANCH ZERO

ZERO      := UP  -> ONE
          + DOWN -> SEVEN

ONE       := UP  -> TWO
          + DOWN -> ZERO

TWO       := UP  -> THREE
          + DOWN -> ONE

THREE     := UP  -> FOUR
          + DOWN -> TWO

Continued...

...Continued

FOUR      := UP  -> FIVE
          + DOWN -> THREE

FIVE      := UP  -> SIX
          + DOWN -> FOUR

SIX       := UP  -> SEVEN
          + DOWN -> FIVE

SEVEN     := UP  -> ZERO
          + DOWN -> SIX

ZERO     =  /BIT2 * /BIT1 * /BIT0
ONE      =  /BIT2 * /BIT1 *  BIT0
TWO      =  /BIT2 *  BIT1 * /BIT0
THREE    =  /BIT2 *  BIT1 *  BIT0
FOUR     =   BIT2 * /BIT1 * /BIT0
FIVE     =   BIT2 * /BIT1 *  BIT0

SIX      =   BIT2 *  BIT1 * /BIT0
SEVEN    =   BIT2 *  BIT1 *  BIT0

CONDITIONS
UP = ENABLE * UP_DWN
DOWN = ENABLE * /UP_DWN

# Multiple State MachinesXE "State Machines, multiple"§XE "Multiple State Machines"§

The MACHXL state machine language extensions do not support multiple state machines.  Refer to "Building State Machines with CASE Statements" in Chapter 5.