| 5 | Language Reference |
| --- | --- |

Contents

Symbols and Operators	129

Keywords	131

Symbols and Operators

| Operator | Definition | Example |
| --- | --- | --- |
| ,XE ","§XE "comma"§ | Literal separator | IN[1,2,3] IN[1..4,6..9] |
| ^XE "^"§XE "caret"§ | Checks that a pin's three-state output buffer is disabled | CHECK ^OUT2 ^OUT4 |
| #bXE "#b"§ | Binary radix XE "binary radix "§(defines the following number as base 2) | #b101000 |
| #dXE "#d"§ | Decimal radix XE "decimal radix "§(defines the following number as base 10) | #d20 |
| #hXE "#h"§ | Hexadecimal radix XE "hexadecimal radix "§(defines the following number as base 16) | #h28C |
| #oXE "#o"§ | Octal radix XE "octal radix "§(defines the following number as base 8) | #o50 |
| %XE "%"§ | Don't careXE "Don't care"§ | CHECK %OUT1 |
| ' 'XE "' '"§ | String delimitersXE "string delimiters"§ | STRING RESET 'A1 * /A2' |

Continued...

...Continued

| Operator | Definition | Example |
| --- | --- | --- |
| ( )XE "( )"§ | Expression groupingXE "expression grouping"§ | OUT3 = (A*B) + (C*D*F) |
| *XE "*"§ | ANDXE "AND"§ | OUT4 = A*B |
| *=XE "*="§ | Latched equationXE "latched equation"§ | OUT5 *= A*B |
| +XE "+"§ | ORXE "OR"§ | OUT6 = A+B |
| +->XE "+->"§ | Local default XE "local default "§(state transition) | START := C1 -> S2 + C2 -> S3 +-> S4 |
| ->XE "->"§ | State transitionXE "state transition"§ | START := C1 -> S2 + C2 -> S1 |
| x..XE ".."§yXE "x..y"§ | Range XE "range "§of signals in a vector, where x and y are positive integers | INPUT[0..9] |
| /XE "/"§ | NOTXE "NOT"§ | /A |
| :XE "\\:"§ | Case constant valueXE "case constant value"§ | 1: BEGIN OUT1 = IN1 END |
| :+:XE "\\:+\\:"§ | XORXE "XOR"§ | OUT7 = IN1 :+: IN2 |
| :*:XE "\\:="§ | XNOR1XE "registered equation"§ | OUT8 := IN1 :*: IN2 |
| :=XE "\\:="§ | Registered equationXE "registered equation"§ | OUT9 := A*B |
| ;XE ";"§ | CommentXE "comment"§ | ;set low before clocking |
| <XE "<"§ | Less thanXE "less than"§ | WHILE A < 2 DO... |
| <=XE "<="§ | Less than or equalXE "less than or equal"§ | WHILE B <= 2 DO... |
| <>XE "<>"§ | Not equalXE "not equal"§ | IF A <> 2 THEN... |

Continued...

...Continued

| Operator | Definition | Example |
| --- | --- | --- |
| =XE "="§ | Combinatorial XE "combinatorial "§(if not otherwise specified in pin declaration) | OUT = IN1 * /IN2 |
| >XE ">"§ | Greater thanXE "greater than"§ | WHILE A > 2 DO... |
| >=XE ">="§ | Greater than or equalXE "greater than or equal"§ | WHILE B >= 2 DO... |
| ?XE "?"§ | Floating pin or node XE "floating pin or node "§ or Discrepancy XE "discrepancy "§("clashXE "clash"§") in simulation | PIN ? OUT3 REG HHHLLL?LL |
| [ ]XE "[ ]"§ | Term bracketsXE "term brackets"§ | INPUT[1..9] |
| { }XE "{ }"§ | SubstituteXE "substitute"§ | OUT2=A*B*C OUT3={OUT2}*D ;this is converted to ;OUT=A*B*C*D |
| SpaceXE "Space"§, tabXE "tab"§ | SeparatorXE "separator"§ | PIN 2 IN1 REG |

KeywordsXE "keywords"§

This section describes the keywords reserved by the MACHXL software for special uses.  You can only use keywords in your designs in the following ways:

r	For the purpose associated with each keyword, as described in the keyword descriptions that follow

r	As part of a special string that the MACHXL software treats as a literal and does not parse, such as the string that defines the author of a design file

The keywords reserved by the MACHXL software for special use are listed below and described in detail in the sections that follow.

| ã | Note: Using a reserved word, symbol or operator as a pin or node name will result in errors |
| --- | --- |
| ã | Note: Using an illegal character (umlauts, @, etc.) in a design file or in signal or node names may cause general protection (GP) errors, if not caught by the Parser program. |
| ã | Note: Signal names with more than fourteen characters are truncated by the Parser program with a unique identifier. A cross-reference file is used to restore the original name during back-annotation. |

ASYNC_LIST
AUTHOR

CASE

CHECK

CHECKQ

CHIP

.CLKF

CLKF

CLOCKF

COMBINATORIAL

COMPANY

CONDITIONS

DATE

DEFAULT_BRANCH

DEFAULT_OUTPUT

EQUATIONS

FOR-TO-DO

GND

GROUP

IF-THEN-ELSE

.J

.K

LATCHED

LOW_POWER_LIST
MACH_SEG_x

MEALY_MACHINE

MINIMIZE_OFF

MINIMIZE_ON

MOORE_MACHINE

NC

NCLKF

NODE

.OUTF

PAIR

PATTERN

PIN

PRELOAD

.R

REGISTERED

REVISION

.RSTF

.S

.SETF

SETF

SIGNATURE

SIMULATION

START_UP

START_UP.OUTF

STATE

STRING

SYNC_LIST
.T

TITLE

TRACE_OFF

TRACE_ON

.TRST

VCC

WHILE_DO

| ASYNC_LIST XE "ASYNC_LIST"§ |  |
| --- | --- |
| Purpose | Defines signals in the list as asynchronous so that the corresponding macrocells will be configured properly. |
| Syntax | GROUP ASYNC_LIST List_of_asynchronous_signals |
| Where Used | DECLARATION segment of MACH215 and MACH 3xx/4xx designs |
| Remarks | Use signal names that have been defined correctly using PIN and/or NODE statements. If a signal name appears in both a GROUP SYNC_LIST and a GROUP ASYNC_LIST statement, a warning is generated and the signal is treated as if it appeared in neither statement. This keyword affects the Fitter only if the device supports both asynchronous and synchronous macrocells. |
| Example | ;DECLARATION SEGMENT ... CHIP TEST2 MACH435 ... NODE ? BR1 REG NODE ? BR2 REG NODE ? BR3 REG ... GROUP ASYNC_LIST BR1 BR2 BR3 |
| See Also | r GROUP r SYNC_LIST |

| AUTHOR XE "AUTHOR "§ |  |
| --- | --- |
| Purpose | Begins a header statement that identifies the author of a MACHXL design file. |
| Syntax | AUTHOR Author_name |
| Where Used | DECLARATION segment |
| Remarks | Use any combination of up to 59 alphanumeric characters in the author's name. The author's name can include the characters a-z, the numerals 0-9, the period, and the underscore character. The software generates a warning if you omit this header statement, but continues processing. Arrange header statements in the order shown below. |
| Example | ;DECLARATION SEGMENT TITLE PART 123 DECODER PATTERN 12 REVISION 2 AUTHOR KAREN OLSEN COMPANY ANYCO LTD DATE 1 JANUARY 1993 |
| See Also | r TITLE r PATTERN r REVISION r COMPANY r DATE r CHIP |

| CASE XE "CASE "§ |  |
| --- | --- |
| Purpose | A high-level control-of-flow construct that specifies different actions for different condition values. |
| Syntax | CASE Condition_signals BEGIN Value: BEGIN Action_statement END Value: BEGIN Action_statement END OTHERWISE: BEGIN Action_statement END END |
| Where Used | EQUATIONS segment |
| Remarks | The CASE statement r Specifies actions to be performed when the value of the condition signals matches various constant values r Optionally allows a default action to be performed if the value of the condition signals does not match any of the constant values r Is useful for implementing state machine designs (including multiple state machines) and for address range decoding. You can use groups, vector notation, and strings to specify the condition signals. The set of condition signals is converted into a single, binary value before being compared with the constant values. |
|  | ã Note: Signals for which you fail to specify values in all conditions are treated as don't cares by default whenever their state is undefined. You can force the software to set such signals to logical zero instead. Refer to the discussion on the CASE statement in Chapter 6, "Equations Segment In Depth," for details. |
|  | ã Note: The polarity of a signal referenced in a CASE statement can be affected by the position of the reference in the CASE statement as well as by the position of the CASE statement in the design file. Refer to the "Controlling Polarity from CASE Statements" section in Chapter 6, "Equations Segment In Depth," for details. Observe the following rules: r Follow the CASE keyword with the string name, vector, or list of signals that form the complete set of conditions signals. r Enclose signal names or vector notation in parentheses separated by commas. For example: (A,B,C[0..4]). r Use the appropriate radix operator to specify the radix of each constant value. The default radix is decimal (N10). You can nest CASE statements within other CASE or IF-THEN-ELSE statements and you can nest IF-THEN-ELSE statements within CASE statements. |
|  | You need not use the OTHERWISE statement if you define actions for every possible condition value. However, if a condition value occurs for which you do not specify an action, the outputs are considered to be don't care. Signals for which default conditions are not specified are eliminated from the design during the logic-reduction process. You can force the CASE statement to treat unspecified signals as false instead of as don't cares. Refer to the section on the Case statement in Chapter 6, "Equations Segment In Depth," for additional details. |
| Example | CASE (X,Y) BEGIN 0: BEGIN ;if X,Y=0 (decimal) C = /A * /B ; (X=0 and Y=0) END 3: BEGIN ;if X,Y=3 (decimal) C = /A * B ; (X=1 and Y=1) END OTHERWISE: BEGIN C = A * B ;if X,Y=1 or X,Y=2 END ;(X=0 and Y=1) or ;(X=1 and Y=0) END |
| See Also | r CASE in Chapter 6, "Equations Segment In Depth" r IF-THEN-ELSE r RADIX in Chapter 6, "Equations Segment In Depth" |

| CHECK XE "CHECK "§ |  |
| --- | --- |
| Purpose | Use CHECK to specify what you expect a pin's logical state to be at any point in the simulation. |
| Syntax | CHECK List_of_expected_values |
| Where Used | SIMULATION segment or auxiliary simulation file |
| Remarks | If the tested pin does not have the value you specify in the CHECK statement, a question mark appears in the simulation history, trace, and waveform output where the discrepancy occurred. You specify the logical state you expect for each pin by preceding the pin name with one of the following symbols: Symbol Function Null Indicates that a pin declared as active high should test as a logical 1. The letter H in the simulation file represents the logical high (1) state. / Indicates that a pin declared as active high should test as a logical 0. The letter L in the simulation file represents the logical low (0) state. ^ Indicates that the pin should be in the high-impedance state because its three-state I/O buffer is disabled. The letter Z in the simulation file represents the high-impedence state. % Indicates that the pin should be in the undefined state. The letter X in the simulation file represents the undefined state. |
|  | The list of expected values consists of pin, node, or state, and string names, as well as vectors, to be verified. Each signal name can be up to 14 characters in length. Include up to 76 characters per line of the CHECK statement and use as many lines as you need. Use a blank space to separate the CHECK keyword from the list of signal names, and to separate individual items in the list of signal names. If the signal has the same polarity in the CHECK statement as in the pin/node declaration, the signal is checked to verify that it is a logical 1. If the polarity is reversed, the signal is checked to verify that it is a logical 0. |
|  | A discrepancy, or "clash," occurs when the value of the signal is different from the value expressed for that signal in the CHECK statement. The location of each simulation discrepancy is marked in the simulation output with a question mark and a warning is issued in the execution-log file. |
| Example | SIMULATION CHECK P1 /P2 ^P3 %P4 |
| See Also | r CHECKQ |

| CHECKQXE "CHECKQ"§ |  |
| --- | --- |
| Purpose | Use CHECKQ to specify what you expect the logical state of a register's Q output to be at any point in the simulation procedure. |
| Syntax | CHECKQ List_of_expected_values |
| Where Used | SIMULATION segment or auxiliary simulation file |
| Remarks | CHECKQ verifies signal values at the Q output. This makes CHECKQ especially useful for verifying internal signals. In all other regards, CHECKQ functions just like CHECK. |

| Example | SIMULATION CHECKQ P1 /P2 |
| --- | --- |
| See Also | r CHECK |

| CHIPXE "CHIP"§ |  |
| --- | --- |
| Purpose | Introduces the statement in which you assign a name to the chip and specify the MACH device used in the design. |
| Syntax | CHIP Chip_name Device_name |
| Where Used | DECLARATION segment |
| Remarks | Every MACHXL design must include a CHIP statement that follows the DATE statement and precedes the PIN and NODE statements. When assigning a name to the chip, observe the following rules: r Use up to 14 alphanumeric characters (the first character cannot be numeric) r Do not use operators, keywords, carriage returns, tabs, or blanks ã Note: The CHIP statement is automatically constructed by the Create a New Design form. The form's Device = field allows you to choose from a list of all supported devices. |
| Example | CHIP SYNCHRO MACH435 |
| See Also | r TITLE r PATTERN r REVISION r AUTHOR r COMPANY r DATE |

| .CLKFXE ".CLKF"§ |  |
| --- | --- |
| Purpose | Defines the conditions under which the the edge-selectable clock signal occurs. |
| Syntax | Pin_name.CLKF = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | r If no clock pin is defined using the .CLKF keyword, registered outputs default to the default clock pin for the device. r An active-high .CLKF equation defines a rising-edge clock signal. An active-low .CLKF equation defines a falling-edge clock signal. ã Note: Multiple functional equations are ORed, then de Morgan's theorem is applied. On devices that do not support product-term clocks, the Fitter will subsequently generate an error message. The Fitter will also generate an error if the resulting equation exceeds a single product term. |
| Example 1 | P2.CLKF = I2 * /I3 P[2..3].CLKF = CLK1 ;this equation clocks ;both I/Os from pin CLK1 P[2..3].CLKF = CLK[0..1] ;this equation clocks ;the following equations: ;P[2].CLKF = CLK[0] ;P[3].CLKF = CLK[1] |
| Example 2 | ;the following code segment clocks a group of I/Os ;DECLARATION SEGMENT GROUP CLOCKS A B C ... ;EQUATIONS SEGMENT CLOCKS.CLKF = D * E |
| See Also | r NCLKF r CLKF r CLOCKF |

| CLKFXE "CLKF"§ |  |
| --- | --- |
| Purpose | Defines the rising-edge clock signal to be used to synchronize a state machine. |
| Syntax | CLKF = Clock_pin |
| Where Used | STATE segment |
| Remarks | If you do not use a CLKF statement, the default clock pin for the device is used. If you want to clock the state machine with a clock other than the default clock, add a CLKF statement to the STATE segment. In the PIN statements portion of the design file, you must declare a logical name for the clock pin before you can use it on the right side of the CLKF statement. |
| Example | STATE ... CLKF = CLK2 ... STATE1 := CONDIT_1 -> STATE2 +-> STATE1 ... |
| See Also | r NCLKF |

| CLOCKFXE "CLOCKF"§ |  |
| --- | --- |
| Purpose | Generates a clock pulse on dedicated clock pins during simulation. |
| Syntax | CLOCKF List_of clock_pin_names |
| Where Used | SIMULATION segment |
| Remarks | Where no clock pin names are specified, the default clock pin for the device is used. CLOCKF generates three test vectors: r For rising-edge clocks: raise clock, propagate output, lower clock. r For falling-edge clocks: lower clock, propagate output, raise clock. |
|  | In the simulation trace and waveform files, the letter "c" appears in the header over the last vector of each CLOCKF pulse. |
| Example | SIMULATION ;begin SIMULATION segment SETF INIT CLOCKF CLK1 ... |
| See Also | r CLKF r NCLKF |

| COMBINATORIALXE "COMBINATORIAL"§ |  |
| --- | --- |
| Purpose | In the case of an I/O pin used for output or a buried node, defines the logic macrocell associated with the pin or node as combinatorial. In the case of an I/O pin use for input, causes the pin signal to bypass the input register. |
| Syntax | PIN Pin_number Pin_name COMBINATORIAL |
| Where Used | DECLARATION segment |
| Remarks | When a pin or node is defined as combinatorial, the corresponding signal bypasses the macrocell register. May be abbreviated COMB. ã Note: If you do not specify a pin or node as COMBINATORIAL, REGISTERED, or LATCHED, it is COMBINATORIAL by default. |
| Example | ;declaration segment ... PIN 11 IN5 COMBINATORIAL ;INPUT PIN 12 IN6 COMBINATORIAL ;INPUT PIN 13 OUT1 REGISTERED ;OUTPUT PIN 14 OUT2 COMBINATORIAL ;OUTPUT ... |
| See Also | r LATCHED r REGISTERED |

| COMPANYXE "COMPANY"§ |  |
| --- | --- |
| Purpose | Begins a header statement that identifies the design author's company. |
| Syntax | COMPANY Company_name |
| Where Used | DECLARATION segment |
| Remarks | Use any combination of up to 59 alphanumeric characters in the company name. The company name can include the characters a-z, the numerals 0-9, the period, and the underscore character. The software generates a warning if you omit this header statement, but continues processing. Arrange header statements in the order shown below. |
| Example | ; DECLARATION SEGMENT TITLE Part 123 Decoder PATTERN 12 REVISION 2 AUTHOR Karen Olsen COMPANY Advanced Micro Devices DATE 1 January 1993 |
| See Also | r AUTHOR r CHIP r DATE r PATTERN r REVISION r TITLE |

| CONDITIONSXE "CONDITIONS"§ |  |
| --- | --- |
| Purpose | Identifies the beginning of the condition equations portion of the state machine design. |
| Syntax | CONDITIONS Condition_equation_name = Boolean_expression |
| Where Used | STATE segment |
| Remarks | Use the condition equation names defined after the CONDITIONS keyword in state transition and state output equations, to define state branching conditions. Each condition equation must have a unique name consisting of up to 14 alphanumeric characters. Condition equation names cannot contain operators or keywords. Parentheses and multiple product terms are allowed. Specify unconditional-signals using VCC (logical 1) and GND (logical 0). The software generates error messages if you specify conflicting or overlapping conditions in the state transition or state output equations. ã Note: If the condition consists of a single signal, you can use the signal name in the state transition and state output equations without defining a condition equation, as shown in Example 2, below. |
| Example 1 | STATE ... 1GREEN := CAR_WAITING -> 1YELLOW ;the condition +-> 1GREEN ;CAR_WAITING is ... ;defined below CONDITIONS CAR_WAITING = SENSOR * /INIT ... |
| Example 2 | ;declaration segment ... PIN 2 I2 COMBINATORIAL ;I2 is an input pin ... STATE ... 1GREEN := I2 -> 1YELLOW ;uses I2 pin as condition +-> 1GREEN ... |
| See Also | r STATE r DEFAULT_BRANCH r DEFAULT_OUTPUT |

| DATEXE "DATE"§ |  |
| --- | --- |
| Purpose | Identifies the date of the design. |
| Syntax | DATE Date_information |
| Where Used | DECLARATION segment |
| Remarks | The DATE statement follows the COMPANY statement. Use any combination of up to 59 alphanumeric characters in the date. The date can include the characters a-z, the numerals 0-9, the period, and the underscore character. The software generates a warning if you omit the DATE statement, but continues processing. |
| Example | ; DECLARATION SEGMENT TITLE Part 123 Decoder PATTERN 12 REVISION 2 AUTHOR Karen Olsen COMPANY Advanced Micro Devices DATE 1 January 1993 |
| See Also | r AUTHOR r CHIP r COMPANY r PATTERN r REVISION r TITLE |

| DEFAULT_BRANCHXE "DEFAULT_BRANCH"§ |  |
| --- | --- |
| Purpose | Defines the global default branch for a state machine. |
| Syntax | DEFAULT_BRANCH State_name DEFAULT_BRANCH HOLD_STATE DEFAULT_BRANCH NEXT_STATE |
| Where Used | STATE segment |
| Remarks | Include the optional DEFAULT_BRANCH keyword once, anywhere in the STATE segment of a design file. There are three choices for the global default branch: r DEFAULT_BRANCH State_name Branch to a specific state. The state name must be properly declared in the STATE segment of the design file. r DEFAULT_BRANCH HOLD_STATE Remain in the current state. HOLD_STATE is the default action if a) you do not specify a DEFAULT_BRANCH and b) you do not specify a local default using the +-> operator. r DEFAULT_BRANCH NEXT_STATE Branch to the next state. The "next state" in this context is the state defined immediately after the current state in the state transition equations. ã Note: DEFAULT_BRANCH does not work for undefined states. |
| Example | STATE ;begin STATE segment DEFAULT_BRANCH STATE_1 or STATE DEFAULT_BRANCH HOLD_STATE or STATE DEFAULT_BRANCH NEXT_STATE |
| See Also | r Appendix A, "State Segment In Depth" r DEFAULT_OUTPUT r OUTPUT_HOLD |

| DEFAULT_OUTPUTXE "DEFAULT_OUTPUT"§ |  |
| --- | --- |
| Purpose | Defines the next output-pin value of a state machine when the value cannot be determined from the design. |
| Syntax | DEFAULT_OUTPUT List_of_output_pin_values |
| Where Used | STATE segment |
| Remarks | Specify the output pin values as you want them to appear at the output pins, regardless of whether the pins are defined as active-high or active-low. Polarities are adjusted automatically to generate the specified values at the pins. r List the pin name with no prefix to generate a logical 1 at the pin. r Complement the pin name with a forward slash to generate a logical 0 at the pin. ã Note: If the output pin and state bit are the same, there is no need to write output equations. |
| Example | STATE ;state setup and defaults ... DEFAULT_OUTPUT /OUT1 OUT2 /OUT3 |
| See Also | r Appendix A, "State Segment In Depth" r DEFAULT_BRANCH |

| EQUATIONSXE "EQUATIONS"§ |  |
| --- | --- |
| Purpose | Begins the EQUATIONS segment of the design file. |
| Syntax | EQUATIONS Boolean_equations ... |
| Where Used | EQUATIONS segment |
| Remarks | Include the EQUATIONS keyword immediately following the DECLARATION segment and before any Boolean equations, functional equations, CASE, or IF-THEN-ELSE statements. |
| Example | EQUATIONS OUT1 = IN1 * /IN2 OUT1.CLKF = CLK1 OUT1.TRST = /OE |
| See Also | r Boolean Equations and Functional Equations in Chapter 6, "Equations Segment In Depth" r CASE r IF-THEN-ELSE |

| FOR-TO-DOXE "FOR-TO-DO"§ |  |
| --- | --- |
| Purpose | Repeats a set of instructions a predefined number of times. |
| Syntax | FOR Variable_name := Start_value TO End_value DO BEGIN Action_statement(s) END |
| Where Used | SIMULATION segment |
| Remarks | The variable name must be a unique name consisting of 1-14 alphanumeric characters. The variable name cannot be used elsewhere in the design except in another FOR--TO--DO loop that is not nested with the first one. The start value must be an integer greater than or equal to zero and less than or equal to the end value. The end value must be an integer greater than or equal to zero. ã Note: If the start and end values are the same, the action statement is executed once. You can nest FOR-TO-DO constructs within other FOR-TO-DO constructs or within IF-THEN-ELSE or WHILE-DO constructs. ã Note: You must use the := assignment operator in the FOR-TO-DO statement or the software will generate an error message. |
| Example | SIMULATION ... FOR X := 1 TO 20 DO BEGIN CLOCKF CLK1 END ... |
| See Also | r IF-THEN-ELSE (simulation) r WHILE-DO |

| GNDXE "GND"§ |  |
| --- | --- |
| Purpose | Specifies an unconditional-low signal (logical 0) in a Boolean or state machine equation. |
| Syntax | Pin_or_node_name<.function> = GND |
| Where Used | DECLARATION, EQUATIONS, and STATE segments |
| Remarks | Assign a value of GND to the pin or node to set the pin or node permanently to logical 0. Assign a value of GND to a functional equation to hold the corresponding function low. (In the case of the .TRST function, for example, use GND to disable the three-state output buffer unconditionally.) |
| Example | OUT4 = GND ;sets OUT4 to logical 0 OUT5.TRST = GND ;disables output from OUT5 ;so the OUT5 I/O pin can be ;used for input |
| See Also | r VCC |

| GROUPXE "GROUP"§ |  |
| --- | --- |
| Purpose | Assigns a group name to several pins and/or nodes so that a single equation can control the whole group. |
| Syntax | GROUP Group_name List_of_pin_and__node_names |
| Where Used | DECLARATION segment |
| Remarks | The group name can contain 1-14 characters including the letters a-z, the numerals 0-9, and the underscore character. The GROUP keyword follows the PIN and NODE statements. |
|  | Separate the group name and signal names in the list of pins and nodes from the GROUP keyword and from each other with blank spaces. The range operator can be used to include vectors of pins or nodes in a group. Use the group name as follows. r On the left side of equations in the EQUATIONS segment r To represent all members of the group in the SIMULATION segment ã Note: You can use GROUP to control banks of registers that use common controls, such as reset or output enable lines. |
| Example | ;DECLARATION SEGMENT ... PIN 39..43 OUT[28..32] REG GROUP OUTPUT_BANK IN2 OUT3 OUT[28..31] ... |
|  | EQUATIONS ... OUTPUT_BANK.CLKF = CLK1 OUTPUT_BANK.TRST = /A * /B O1 = C + D * /E ... SIMULATION ... SETF OUT[28] /OUT[31] CLOCKF CLK1 CHECK OUTPUT_BANK = #b10111... |
| See Also | r MACH_SEG_x r ASYNC_LIST r LOW_POWER_LIST r SYNC_LIST |

| IF-THEN-ELSEXE "IF-THEN-ELSE"§ |  |
| --- | --- |
| Purpose | Specifies one set of actions if a condition is true and (optionally) a second set of actions if the condition is false. |
| Syntax | IF ( Condition_expression ) THEN BEGIN Actions expressed as Boolean equations or other constructs END ELSE BEGIN Actions expressed as Boolean equations or other constructs END |
| Where Used | EQUATIONS segment |
| Remarks | The condition expression can be either r A single pin or node or a Boolean expression. The condition evaluates as true if the pin or node is logical 1 or if the Boolean expression is true. The condition expression evaluates as false if the pin or node is logical 0 or if the Boolean expression is false. r A vector consisting of a) a range of pins or b) a comma-separated list of pin names, the = assignment operator, and test value defined as a binary, octal, decimal, or hexadecimal radix number (the default radix is decimal). |
|  | IF-THEN-ELSE statements can be nested inside other IF-THEN-ELSE and CASE statements. (Do not add an extra END or ENDIF statement or an error message will be generated during compilation.) Parentheses surrounding the condition expression are optional, but recommended. You can nest parentheses. |
| Example | IF (I[0..7] = #B10001001) THEN BEGIN A = B * /C END ELSE BEGIN A = GND END |
| See Also | r CASE r IF-THEN-ELSE (SIMULATION) r IF-THEN-ELSE in Chapter 6, "Equations Segment In Depth" |

| IF-THEN-ELSE, SIMULATIONXE "IF-THEN-ELSE, simulation"§ |  |
| --- | --- |
| Purpose | Specifies one set of actions if a condition is true and (optionally) a second set of actions if the condition is false. |
| Syntax | IF ( Condition_expression ) THEN BEGIN Simulation_actions END ELSE BEGIN Simulation_actions END |
| Where Used | SIMULATION segment |
| Remarks | The condition expression can be r The index variable used in a FOR-TO-DO statement, an assignment operator, and a test value expressed as a positive integer or zero (but only if the IF-THEN-ELSE statement is nested inside the FOR-TO-DO statement). |
|  | r A single pin or node or a Boolean expression. The condition evaluates as true if the pin or node is logical 1 or if the Boolean expression is true. The condition expression evaluates as false if the pin or node is logical 0 or if the Boolean expression is false. r A vector consisting of a) a range of pins or b) a comma-separated list of pin names, an assignment operator, and test value defined as a binary, octal, decimal, or hexadecimal radix number (the default radix is decimal). Valid assignment operators include =, <, >, <=, >=, and <>. IF-THEN-ELSE statements can be nested inside other IF-THEN-ELSE or FOR-TO-DO and WHILE-DO statements. (Do not add an extra END or ENDIF statement or an error message will be generated during compilation.) Parentheses surrounding the condition expression are optional, but recommended. You can nest parentheses. |
| Example | SIMULATION ... FOR K := 1 TO 5 DO BEGIN IF K = 2 THEN BEGIN SETF INIT /B CLOCKF CLK1 END ELSE BEGIN SETF /INIT CLOCKF CLK1 END END |
| See Also | r IF-THEN-ELSE r FOR-TO-DO r WHILE-DO |

| IPAIRXE "IPAIR"§ |  |
| --- | --- |
| Purpose | Alternate form of the keyword PAIR, used for input pairing. (See the section on the PAIR keyword in this chapter.) |

| .JXE ".J"§ |  |
| --- | --- |
| Purpose | Defines the J-input logic for a J-K flip-flop and simultaneously specifies J-K behavior for the corresponding macrocell. |
| Syntax | Pin_name.J = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | You can place the .J equation anywhere in the EQUATIONS segment. Observe the following rules. r You must specify output logic for a J-K flip-flop using one .J and one .K equation. r You can append the .J suffix to group, string, or vector names to assign a J-type equation to several pins or nodes at one time. |
| Example | OUT1.J = C * /D |
| See Also | r COMBINATORIAL r .K r .R r .S r .T |

| .KXE ".K"§ |  |
| --- | --- |
| Purpose | Defines the K-input logic for a J-K flip-flop and simultaneously specifies J-K behavior for the corresponding macrocell. |
| Syntax | Pin_name.K = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | You can place the .K equation anywhere in the EQUATIONS segment. Observe the following rules. r You must specify output logic for a J-K flip-flop using one .J and one .K equation. r You can append the .K suffix to group, string, or vector names to assign a J-type equation to several pins or nodes at one time. |
| Example | OUT1.K = C * /D |
| See Also | r COMBINATORIAL r .J r .R r .S r .T |

| LATCHEDXE "LATCHED"§ |  |
| --- | --- |
| Purpose | Defines a pin or node as a latched output type. |
| Syntax | PIN Pin_number PinName LATCHED |
| Where Used | DECLARATION segment |
| Remarks | When a pin or node is defined as latch, the result of the sum-of-products logic is latched by the macrocell associated with the pin. May be abbreviated LAT. |
| Example | PIN 23 ACK LAT |
| See Also | r COMBINATORIAL r REGISTERED |

| LOW_POWER_LIST XE "LOW_POWER_LIST"§ |  |
| --- | --- |
| Purpose | Defines signals that will be configured in the power-down mode. The power-down mode, available on the MACH111, MACH131, MACH211, and MACH231 devices, trades off speed for reduced power consumptions. Refer to the device data sheet for details. |
| Syntax | GROUP LOW_POWER_LIST List_of_power-down_signals |
| Where Used | DECLARATION segment of MACH111, MACH131, MACH211, and MACH231 designs |
| Remarks | Use signal names that have been defined correctly using PIN and/or NODE statements. Unused macrocells and input pins are automatically configured in the power-down mode; do not include them in the GROUP LOW_POWER_LIST statement. |
| Example | ;DECLARATION SEGMENT ... CHIP TEST2 MACH111 ... NODE ? BR1 NODE ? BR2 NODE ? BR3 ... GROUP LOW_POWER_LIST BR1 BR2 BR3 |
| See Also | r GROUP |

| MACH_SEG_xXE "MACH_SEG_x"§ |  |
| --- | --- |
| Purpose | Predefines group names that automatically place all signals defined as group members in the same block of a MACH device. |
| Syntax | GROUP MACH_SEG_I List_of_pins_and_nodes |
| Where Used | Used as a group name in a GROUP statement. The MACH_SEG_x group name can appear in the DECLARATION, EQUATIONS, and SIMULATION segments of the design file: r Define MACH_SEG_x groups in the DECLARATION segment. r Use group names on the left side of equations in the EQUATIONS segment. |
| Remarks | Replace the x in the predefined group MACH_SEG_x group names with the letter that represents the PAL block into which you want all group signals to be placed. Use the letters A-H for 8-block devices, the letters A-P for 16-block devices. For example, MACH_SEG_A, MACH_SEG_B, MACH_SEG_C, and MACH_SEG_D, correspond to the first four PAL blocks of the MACH435 device: Block A, Block B, Block C, and Block D. Define MACH_SEG_x groups only in the GROUP statement. All of the rules that normally apply to the GROUP keyword apply to MACH_SEG_x groups. |
| Example | ;DECLARATION SEGMENT ... GROUP MACH_SEG_A R[1..4] P[1..8] ... EQUATIONS MACH_SEG_A.TRST = IN2 * /IN3 ... SIMULATION ... SETF MACH_SEG_A ... |
| See Also | r GROUP |

| MEALY_MACHINEXE "MEALY_MACHINE"§ |  |
| --- | --- |
| Purpose | Identifies a state machine as a Mealy type. |
| Syntax | MEALY_MACHINE |
| Where Used | STATE segment |
| Remarks | Place the MEALY_MACHINE keyword immediately after the STATE keyword. ã Note: If you do not define the state machine as Mealy or Moore, the software uses the MEALY_MACHINE default. |
|  | Do not use both MEALY_MACHINE and MOORE_MACHINE keywords in the same design file. If you need to implement both types of state machines on the same MACH device, you must define one of them in the EQUATIONS segment using Boolean logic, as described in the "Building State Machines with CASE Statements" section in Chapter 6, "Equations Segment In Depth." |
| Example | ... STATE ;begin the STATE segment MEALY_MACHINE ;define design as a Mealy machine ... |
| See Also | r Appendix A, "State Segment In Depth" r MOORE_MACHINE r STATE |

| MINIMIZE_OFFXE "MINIMIZE_OFF"§ |  |
| --- | --- |
| Purpose | Switches off most logic reduction until a) the keyword MINIMIZE_ON occurs or b) the EQUATIONS segment ends. |
| Syntax | MINIMIZE_OFF |
| Where Used | EQUATIONS segment |
| Remarks | If you place a pair of MINIMIZE_OFF and MINIMIZE_ON statements around equation(s) for some pin or node, but other equations for the same pin or node remain outside the pair of MINIMIZE_OFF and MINIMIZE_ON statements, the MINIMIZE_OFF statement will be ignored for that pin or node. |
| Example | EQUATIONS ... X = A * /B MINIMIZE_OFF Y = A * C + A * B + A Y.TRST = ENABLE Y.CLKF = CLK1 MINIMIZE_ON Z = /(X+Y) ... |
| See Also | r "MINIMIZE_ON and MINIMIZE_OFF" in Chapter 6, "Equations Segment In Depth" r MINIMIZE_ON |

| MINIMIZE_ONXE "MINIMIZE_ON"§ |  |
| --- | --- |
| Purpose | Used after the keyword MINIMIZE_OFF to restore normal logic reduction for the remainder of the design file or until the next occurrence of MINIMIZE_OFF. |
| Syntax | MINIMIZE_ON |
| Where Used | EQUATIONS segment |
| Remarks | If you place a pair of MINIMIZE_OFF and MINIMIZE_ON statements around equation(s) for some pin or node, but other equations for the same pin or node remain outside the pair of MINIMIZE_OFF and MINIMIZE_ON statements, the MINIMIZE_OFF statement will be ignored for that pin or node. |
| Example | EQUATIONS ... X = A * /B MINIMIZE_OFF Y = A * C + A * B + A Y.TRST = ENABLE Y.CLKF = CLK1 MINIMIZE_ON Z = /(X+Y) ... |
| See Also | r "MINIMIZE_ON and MINIMIZE_OFF" in Chapter 6, "Equations Segment In Depth" r MINIMIZE_OFF |

| MOORE_MACHINEXE "MOORE_MACHINE"§ |  |
| --- | --- |
| Purpose | Identifies a state machine as a Moore type. |
| Syntax | MOORE_MACHINE |
| Where Used | STATE segment |
| Remarks | Place the MOORE_MACHINE keyword immediately after the STATE keyword. ã Note: If you do not define the state machine as Mealy or Moore, the software uses the MEALY_MACHINE default. Do not use both MEALY_MACHINE and MOORE_MACHINE keywords in the same design file. If you need to implement both types of state machines on the same MACH device, you must define one of them in the EQUATIONS segment using Boolean logic. Appendix A, "State Segment In Depth," contains information on multiple state machines. |
| Example | ... STATE ;begin the STATE segment MOORE_MACHINE ;define design as a Moore machine ... |
| See Also | r Appendix A, "State Segment In Depth" r MEALY_MACHINE r STATE |

| NCLKFXE "NCLKF"§ |  |
| --- | --- |
| Purpose | Defines the falling-edge clock used to synchronize a state machine. |
| Syntax | NCLKF = Clock_pin or product term |
| Where Used | STATE segment |
| Remarks | Where no clock pin names are specified, the software selects the default clock pin for the device and the default clock mode (rising-edge clock). NCLKF automatically creates appropriate falling-edge clock assignments for all state bit registers and the state output registers used in your design. |
| Example | NCLKF = T1_CKBf |
| See Also | r CLOCKF r CLKF |

| NODEXE "NODE"§ |  |
| --- | --- |
| Purpose | Declares a signal other than a pin (typically, a buried or input register) and defines the signal's name, polarity, and storage type. |
| Syntax | NODE Node_number_or_? Node_name Storage_type [Output_pair_info] |
| Where Used | DECLARATION segment |
| Remarks | Find the node number for the node you are declaring in the appropriate MACH device node list in Chapter 10, "Device Reference." Substitute the symbol ? for the node number to "float" the node (have the software assign it to a specific node during the fitting procedure). |
|  | Assign a logical name to the node, observing the following rules. r Each name must be unique. r Node names can consist of 1-14 alphanumeric characters including the characters a-z, the numerals 0-9, and the underscore character. |
|  | To define the node as active low: r Complement the logical name you assign to the node in the NODE declaration statement. (Preferred method) |

|  | Example NODE ? /BR1 REG ... BR1 = IN2 * /IN5 r Precede the node name with the forward slash character / (for example, /BR2) when you write the Boolean equation that defines the node's behavior. Do not leave a space between the forward slash and the node name. (Alternate method) Example NODE ? BR1 REG ... /BR1 = IN2 * /IN5 You can use the range operator to define a vector of nodes with a single NODE statement. The vector of node numbers must contain the same number of elements as the vector of node names. All nodes in the vector share the same polarity and storage type. The storage type can be any one of the following. r COMBINATORIAL r LATCHED r REGISTERED NODE statements must follow the CHIP statement. |
| --- | --- |

| Example | NODE 1 BR1 REGISTERED ;active high NODE 2 /BR2 REGISTERED ;active low NODE 3..8 STATEBIT[1..6] REGISTERED ;the last NODE statement defines six nodes, all of ;which are active high and registered |
| --- | --- |
| See Also | r [..] (RANGE OPERATOR) in the "Symbols and Operators" section of this chapter r COMBINATORIAL r LATCHED r PAIR r PIN r REGISTERED r "Vectors" in Chapter 6, "Equations Segment In Depth" |

| OPAIRXE "OPAIR"§ |  |
| --- | --- |
| Purpose | Alternate form of the keyword PAIR, used for output pairing. (See the section on the PAIR keyword in this chapter.) |

| .OUTFXE ".OUTF"§ |  |
| --- | --- |
| Purpose | Defines the state outputs for Mealy and Moore state machines. |
| Syntax | ;Moore Machine State_name.OUTF = Output_expression ;Mealy Machines State_name.OUTF = Condition_1 -> Output_1 + Condition_2 -> Output_2 ... + Condition_n -> Output_n +-> Local_default_output |
| Where Used | STATE segment |
| Remarks | Specify the actual output you want to appear at the pin. ã Note: The software adjusts automatically for pin polarity if you have set the Ensure polarity after minimization is parameter (in the Logic Synthesis Options menu) to "As specified in design file." |
| Example 1 | ... STATE MOORE_MACHINE ... STATE1.OUTF = /CNT2 * CNT1 * /CNT0 ... |
| Example 2 | ... STATE MEALY_MACHINE ... STATE1.OUTF = RUN_UP -> /CNT2 * CNT1 * /CNT0 ... TEST2 -> CNT2 * CNT1 * CNT0 +-> /CNT2 * /CNT1 * /CNT0 ... |
| See Also | r +-> (LOCAL DEFAULT) in the "Symbols and Operators" section of this chapter r DEFAULT_OUTPUT r MEALY_MACHINE r MOORE_MACHINE r OUTPUT EQUATIONS in Appendix A, "State Segment In Depth" r STATE |

| PAIRXE "PAIR"§ |  |
| --- | --- |
| Purpose | An optional attribute used r In the PIN statement to direct input pairing between the pin and a node (interchangeable in this context with the alternate keyword form IPAIR) r In the NODE statement to direct output pairing between the node and a pin (interchangeable in this context with the alternate keyword form OPAIR) |
| Syntax | ã For Input Pairing: PIN ? Pin_name PAIR Node_name ã For Output Pairing: NODE ? Node_name Storage_type PAIR Pin_name |
| Where Used | DECLARATION segment |
| Remarks | The default storage type for pins and nodes is COMBINATORIAL. REGISTERED and LATCHED are the only valid storage types for the NODE statement when specifying input pairing. You can specify a specific pin and a specific node or float the pin, the node, or both the pin and the node (by using the ? symbol in place of a pin or node number). If you pair a specific pin with a specific node, both must correspond to the same macrocell. |
| Example 1 | ;input pairing with both signals floating PIN ? I1 COMBINATORIAL PAIR R1 NODE ? R1 REGISTERED ... EQUATIONS ... R1 = I1 ; node passes through value of input pin |
| Example 2 | ;output pairing with fixed pin and floating node NODE ? L2 COMBINATORIAL PAIR OUTPUT1 PIN 3 OUT1 COMBINATORIAL ... EQUATIONS ... L2 = A * /C ; define logic of one member of pair OUT1 = {L2} ; copy right side of L2 equation using {}, or ; write it out if you prefer: OUT1 = A * /C ; BOTH members of pair must have ; identical equations or an error will occur |
| See Also | r NODE r "Pairing" in Chapter 6, "Equations Segment In Depth" r PIN |

| PATTERNXE "PATTERN"§ |  |
| --- | --- |
| Purpose | Identifies the pattern (if any) of a MACHXL design file. |
| Syntax | PATTERN Pattern_name |
| Where Used | DECLARATION segment |
| Remarks | Place the PATTERN keyword after TITLE and before REVISION. Use any combination of up to 59 alphanumeric characters in the pattern name. The pattern name can include the characters a-z, the numerals 0-9, the period, and the underscore character. |
| Example | ;DECLARATION SEGMENT TITLE PART 123 DECODER PATTERN 12 REVISION 2 AUTHOR KAREN OLSEN COMPANY ANYCO LTD DATE 1 JANUARY 1993 |
| See Also | r TITLE r REVISION r AUTHOR r COMPANY r DATE r CHIP |

| PINXE "PIN"§ |  |
| --- | --- |
| Purpose | Declares a pin and defines the pin's name, position, polarity, and storage type. |
| Syntax | PIN Pin_number Pin_name Storage_type [Pairing_info] |
| Where Used | DECLARATION segment |

| Remarks | Find pin number in the appropriate MACH device pin diagram in Chapter 10, "Device Reference." Substitute the symbol ? for the pin number to "float" the pin (have the software assign it to a specific pin during the fitting procedure). Assign a logical name to the pin, observing the following rules. r Each name must be unique. r Pin names can consist of 1-14 alphanumeric characters including the characters a-z, the numerals 0-9, and the underscore character. |
| --- | --- |
|  | To define the pin as active low: r Either do this: In the PIN or NODE declaration statement, complement the logical name you assign to the pin. (Preferred method) |
|  | Example NODE ? /OUT2 REG ;declared active low ... OUT2 = IN2 * /IN5 |
|  | r Or this: In the EQUATIONS segment, complement the pin's (node's) logical name when it appears on the left side of the equation. (Alternate method) Example PIN ? OUT2 REG ;declared active high ... /OUT2 = IN2 * /IN5 ;but converted to active low here |
|  | You can use the range operator to define a vector of pins with a single PIN statement. The vector of pin numbers must contain the same number of elements as the vector of pin names. All pins in the vector share the same polarity and storage type. The storage type can be any one of the following. r COMBINATORIAL r LATCHED r REGISTERED PIN statements must follow the CHIP statement. |

| Example | PIN 3 IN1 COMB ;active high PIN 4 /IN2 COMB ;active low PIN 5..10 STATE_OUT[1..6] REGISTERED ;the last PIN statement defines six pins, all of ;which are active high and registered |
| --- | --- |
| See Also | r [..] (RANGE OPERATOR) in the "Symbols and Operators" section of this chapter r COMBINATORIAL r LATCHED r NODE r PAIR r REGISTERED r VECTORS in Chapter 6, "Equations Segment In Depth" |

| PRELOADXE "PRELOAD"§ |  |
| --- | --- |
| Purpose | Allows you to load specified values into registers during simulation. |
| Syntax | PRELOAD List_of_desired_signals |
| Where Used | SIMULATION segment |
| Remarks | ã Note: The PRELOAD command is supported in software simulation but not in the hardware. Test vectors are turned off at the first occurrence of the PRELOAD command. The list of desired signals can include pin names, node names, state names, condition names, groups, and strings. r You can load a logical 1 into a pin or node by specifying the uncomplemented pin or node name. r You can load a logical 0 into a pin or node by specifying the complement of the pin or node name. r You can load a specified state by specifying the state name. |
| Example | SIMULATION PRELOAD Q1 /Q2 |

| .RXE ".R"§ |  |
| --- | --- |
| Purpose | Defines the R-input logic for an S-R flip-flop and simultaneously specifies S-R behavior for the corresponding macrocell. |
| Syntax | Pin_name.R = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | You can place the .R equation anywhere in the EQUATIONS segment. Observe the following rules. r You must specify output logic for an S-R flip-flop using one .S and one .R equation. r You can append the .R suffix to group, string, or vector names to assign an R-type equation to several pins or nodes at one time. |
| Example | OUT1.R = C * /D |
| See Also | r COMBINATORIAL r .J r .K r .S r .T |

| REGISTEREDXE "REGISTERED"§ |  |
| --- | --- |
| Purpose | Defines a pin or node as a registered output type. |
| Syntax | PIN Pin_number_or_? Pin_name Storage_type [Input_pairing_info] |
| Where Used | DECLARATION segment |
| Remarks | When a pin or node is defined as REGISTERED, the macrocell associated with the pin is configured as a flip-flop. r The flip-flop is a D-type flip-flop by default. r Write .J and .K equations to configure the macrocell as a J-K-type flip-flop. r Write .S and .R equations to configure the macrocell as an S-R-type flip-flop. r Write a .T equations to configure the macrocell as a T-type flip-flop. The REGISTERED keyword can be abbreviated REG. |
| Example | PIN 12 OUT4 REG PIN 13 OUT5 REGISTERED |
| See Also | r COMBINATORIAL r .J r .K r .R r .S r .T |

| REVISIONXE "REVISION"§ |  |
| --- | --- |
| Purpose | Identifies the revision (if any) of a MACHXL design file. |
| Syntax | REVISION Revision_name |
| Where Used | DECLARATION segment |
| Remarks | Place the REVISION keyword after PATTERN and before AUTHOR. Use any combination of up to 59 alphanumeric characters in the revision name. The revision name can include the characters a-z, the numerals 0-9, the period, and the underscore character. |
| Example | ;DECLARATION SEGMENT TITLE PART 123 DECODER PATTERN 12 REVISION 2 AUTHOR KAREN OLSEN COMPANY ANYCO LTD DATE 1 JANUARY 1993 |
| See Also | r TITLE r PATTERN r AUTHOR r COMPANY r DATE r CHIP |

| .RSTFXE ".RSTF"§ |  |
| --- | --- |
| Purpose | Defines the condition under which a flip-flop's programmable reset input is asserted. |
| Syntax | Pin_or_node name.RSTF = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | To assign the same .RSTF equation to all registers in a device, write the .RSTF equation for the global node (node 1). ã Note: Multiple functional equations are ORed, then de Morgan's theorem is applied to produce a single product term. The Fitter generates an error if the resulting equation exceeds a single product term. The pin name on the left side of the .RSTF equation cannot be complemented using the / symbol. |
|  | You can append the .RSTF suffix to group, string, or vector names to control the programmable reset of several pins or nodes at one time. ã Note: The .RSTF function appears as an L in the simulation history file. |
| Example | OUT2.RSTF = IN1 * /IN2 |
| See Also | r "Functional Equations" in Chapter 6, "Equations Segment In Depth" r .SETF |

| .SXE ".S"§ |  |
| --- | --- |
| Purpose | Defines the S-input logic for an S-R flip-flop and simultaneously specifies S-R behavior for the corresponding macrocell. |
| Syntax | Pin_name.S = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | You can place the .S equation anywhere in the EQUATIONS segment. Observe the following rules. r You must specify output logic for an S-R flip-flop using one .S and one .R equation. r You can append the .S suffix to group, string, or vector names to assign an S-type equation to several pins or nodes at one time. |
| Example | OUT1.S = C * /D |
| See Also | r COMBINATORIAL r .J r .K r .R r .T |

| .SETFXE ".SETF"§ |  |
| --- | --- |
| Purpose | Defines the condition under which a flip-flop's programmable preset input is asserted. |
| Syntax | Pin_or_node_name.SETF = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | To assign the same .RSTF equation to all registers in a device, write the .RSTF equation for the global node (node 1). ã Note: Multiple functional equations are ORed, then de Morgan's theorem is applied to produce a single product term. The Fitter generates an error if the resulting equation exceeds a single product term. The pin name on the left side of the .SETF equation cannot be complemented using the / symbol. You can append the .SETF suffix to group, string, or vector names to control the programmable reset of several pins or nodes at one time. ã Note: The .SETF function appears as an H in the simulation history file. |
| Example | OUT2.SETF = IN1 * /IN2 |
| See Also | r "Functional Equations" in Chapter 6, "Equations Segment In Depth" r .RSTF |

| SETFXE "SETF"§ |  |
| --- | --- |
| Purpose | Used during simulation to set inputs to specified values. |
| Syntax | SETF List_of_desired_signals |
| Where Used | SIMULATION segment |
| Remarks | The list of desired signals can include pin and node names. r You can load a logical 1 into a pin or node by specifying the uncomplemented pin or node name. r You can load a logical 0 into a pin or node specifying the complemented pin or node name. You must generate a clock signal using the CLOCKF keyword in order to see the effects of a SETF statement on registered outputs. |
| Example | SETF IN1 /IN2 |
| See Also | r CLOCKF r PRELOAD |

| SIGNATUREXE "SIMULATION"§ |  |
| --- | --- |
| Purpose | In MACH devices which support JTAG, SIGNATURE allows you to specify the JTAG USERCODE fuses. |
| Syntax | SIGNATURE signature |
| Where Used | DECLARATION segment |
| Remarks | The signature is passed through the MACHXL Fitter programs to the JEDEC file where it is used to program the 32 U fuses. The signature can be a number signature, a character signature, or a null signature (the default). In a number signature the form is <radix><number> where the number can contain N digits. The value of N is determined by the radix as follows: N = 32 for binary representation N = 10 for octal representation N = 9 for decimal representation N = 8 for hexadecimal representation |
|  | SIGNATURE supports binary, octal, decimal and hexadecimal number radices; the default is decimal. (See the discussion on Radix Operators in Chapter 6, "Equations Segment In Depth.") All characters following the Nth character are truncated, and any untruncated portion of <number> is converted to its 32 bit binary equivalent, with 0 fill on the left. A number signature is represented in JEDEC file by the UH field. Character signatures are truncated after the fourth character, or if less than four characters, expanded to four characters by space filling on the right. The character signature is represented in the JEDEC file by the UA field. The default signature is all zeros. ã Note: If a selected MACH device is not a JTAG part, the MACHXL software ignores the signature. ã Note: The SIGNATURE syntax is optional and can be ommited. |
| Example | SIGNATURE #hf ;hex result is UH ;0000000F SIGNATURE #hz5 ;character signature disguised as hex ;result is UA #hz5 SIGNATURE #b101010101010 ;hex result is UH 00000AAA |
| See Also | r 5 Volt In-Circuit Programming Development Kit |

| SIMULATIONXE "SIMULATION"§ |  |
| --- | --- |
| Purpose | Begins the SIMULATION segment of the design file. |
| Syntax | SIMULATION Simulation_control_statements ... |
| Where Used | SIMULATION segment |
| Remarks | Include the SIMULATION keyword after the EQUATIONS and STATE segments, if any. ã Note: The SIMULATION segment is optional and can be omitted from design files that do not contain simulation control statements. |
| Example | SIMULATION SETF INIT CLOCKF CLK1 SETF /INIT IN1 IN2 /IN3 CLOCKF CLK1 CHECK OUT1 /OUT2 ;check external pins CHECKQ BR6 /BR8 /BR10 ;checkq buried registers ... |
| See Also | r Chapter 7, "Simulation Segment In Depth" r CHECK r CHECKQ r CLOCKF r FOR-TO-DO r IF-THEN-ELSE (SIMULATION) r PRELOAD r SETF r WHILE-DO |

| START_UPXE "START_UP"§ |  |
| --- | --- |
| Purpose | Specifies the starting state for a state machine. |
| Syntax | START_UP := POWER_UP -> State1_name |
| Where Used | STATE segment |
| Remarks | The START_UP command is supported for MACH 1xx/2xx designs but not for MACH 3xx/4xx designs. The device goes to a known state on power-up or initialization. r If you have programmed the device to power up with all Q outputs high, State1 is automatically programmed to 11...1. r If you have programmed the device to power up with all Q outputs low, State1 is automatically programmed to 00...0. Use this initialization routine to ensure that the state machine powers up in a known state and branches to a known state whenever initialization occurs. |
|  | If you do not specify a START_UP state and you have not assigned state bits manually, the software assigns the default startup state bit configuration to the state that corresponds to the first transition equation defined in the design file. ã Note: If you assign state bits manually and none of your states corresponds to the default power-up configuration, the state machine will power up in an undefined state. Refer to the section on Illegal State Recovery in Appendix A, "State Segment In Depth." |
| Example | STATE MOORE_MACHINE START_UP := POWER_UP -> STATE1 + INIT -> STATE1 ... |
| See Also | r START_UP.OUTF |

| START_UP.OUTFXE "START_UP.OUTF"§ |  |
| --- | --- |
| Purpose | Specifies the starting output for a state machine. |
| Syntax | START_UP.OUTF := POWER_UP -> List_ of_outputs or START_UP.OUTF := List_ of_outputs |
| Where Used | STATE segment |
| Remarks | The START_UP command is supported for MACH 1xx/2xx designs but not for MACH 3xx/4xx designs. When power is applied to the device, the device outputs assume the values specified in the list of outputs. Use this initialization routine to ensure that the state machine powers up and initializes with its outputs under control. ã Note: The START_UP.OUTF statement is especially useful in registered Mealy machines, in which the outputs trail the state transitions by one clock cycle. |
| Example 1 | STATE ... START_UP.OUTF := POWER_UP -> /OUT1 /OUT2 /OUT + INIT -> /OUT1 /OUT2 /OUT3 ... |
| Example 2 | STATE ... START_UP.OUTF := /OUT1 /OUT2 /OUT ... |
| See Also | r START_UP |

| STATEXE "STATE"§ |  |
| --- | --- |
| Purpose | Begins the STATE segment of the design file. |
| Syntax | STATE State_machine_equations ... |
| Where Used | STATE segment |

| Remarks | Include the STATE keyword immediately following the EQUATIONS segment, if any, and before any state equations or the CONDITIONS keyword. ã Note: A design file can contain both Boolean equations (in the EQUATIONS segment) and state machine equations (in the STATE segment). Make sure the equations from the Boolean portion of the design do not overlap the equations derived from the state machine portion of the design. ã Note: The STATE segment is optional and can be omitted from design files that contain only Boolean equations or only Boolean and simulation equations. |
| --- | --- |
| Example | STATE MOORE_MACHINE START_UP := POWER_UP -> STATE1 + INIT -> STATE1 STATE1 := COND1 -> STATE2 + COND2 -> STATE3 +-> STATE1 ... STATE1.OUTF = /OUT1 * /OUT2 * /OUT3 ... CONDITIONS COND1 = IN1 * IN2 COND2 = IN1 * /IN2 ... |
| See Also | r Appendix A, "State Segment In Depth" r MEALY_MACHINE r MOORE_MACHINE r START_UP r START_UP.OUTF |

| STRINGXE "STRING"§ |  |
| --- | --- |
| Purpose | Allows you to assign a string name to a character string and use the string name anywhere in the remainder of the design file instead of repeating the character string. The software substitutes the character string for the string name during processing. |
| Syntax | STRING String_name 'Character_string_to_be_substituted' |
| Where Used | DECLARATION segment |
| Remarks | The string name can consist of 1-14 alphanumeric characters. The string name cannot contain keywords or operators. Include the STRING keyword after the PIN and NODE statements and before the EQUATIONS and STATE keywords. Separate the STRING keyword from the character string with a blank space. Surround the character string with single quotes. |
|  | The software replaces each occurrence of the string name with the specified character string during the early stages of processing. ã Note: If the string contains an expression, the effect of complementing the string differs based on the contents of the string. If the string is enclosed in parentheses, the expression is complemented according to DeMorgan's theorem. If not, just the first element is complemented. |
| Example | STRING STR1 'A1 * /A2 * A3' STRING STR2 '/(/A1 * A2 * /A3)' ... EQUATIONS ... OUT2 = STR1 ; A1 * /A2 * A3 substituted for STR1 + STR2 ; /(/A1 * A2 * /A3) substituted for STR2 ... |

| SYNC_LIST XE "SYNC_LIST"§ |  |
| --- | --- |
| Purpose | Defines signals in the list as synchronous so that the corresponding macrocells will be configured properly. |
| Syntax | GROUP SYNC_LIST List_of_synchronous_signals |
| Where Used | DECLARATION segment of MACH215 and MACH 3xx/4xx designs |
| Remarks | Use signal names that have been defined correctly using PIN and/or NODE statements. If a signal name appears in both a GROUP SYNC_LIST and a GROUP ASYNC_LIST statement, a warning is generated and the signal is treated as if it appeared in neither statement. This keyword affects the Fitter only if the device supports both asynchronous and synchronous macrocells. |
| Example | ;DECLARATION SEGMENT ... CHIP TEST2 MACH435 ... NODE ? BR1 REG NODE ? BR2 REG NODE ? BR3 REG ... GROUP SYNC_LIST BR1 BR2 BR3 |
| See Also | r GROUP r ASYNC_LIST |

| .TXE ".T"§ |  |
| --- | --- |
| Purpose | Defines the T-input logic for a T flip-flop and simultaneously specifies T behavior for the corresponding macrocell. |
| Syntax | Pin_name.T = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | Signal names must be properly declared in the PIN and NODE statements. You can place the .T equation anywhere in the EQUATIONS segment. Observe the following rules. r You can specify output logic for an T flip-flop using just one .T equation (but additional functional equations such as .CLKF and .TRST are allowed). r You can append the .T suffix to group, string, or vector names to assign a T-type equation to several pins or nodes at one time. |
| Example | OUT1.T = C * /D |
| See Also | r COMBINATORIAL r .J r .K r .R r .S |

| TITLEXE "TITLE"§ |  |
| --- | --- |
| Purpose | Identifies the title of the design. |
| Syntax | TITLE Title_name |
| Where Used | DECLARATION segment |
| Remarks | The TITLE statement is the first non-comment statement in the design file. Use any combination of up to 59 alphanumeric characters in the title. The title can include the characters a-z, the numerals 0-9, the period, and the underscore character. The software generates a warning if you omit the TITLE statement, but continues processing. |
| Example | ; DECLARATION SEGMENT TITLE Part 123 Decoder PATTERN 12 REVISION 2 AUTHOR Karen Olsen COMPANY Advanced Micro Devices DATE 1 January 1993 |
| See Also | r AUTHOR r CHIP r COMPANY r DATE r PATTERN r REVISION |

| TRACE_OFFXE "TRACE_OFF"§ |  |
| --- | --- |
| Purpose | Turns off the simulation trace function. |
| Syntax | TRACE_OFF |
| Where Used | SIMULATION segment |

| Remarks | You must include the TRACE_ON keyword in the SIMULATION segment before you include the TRACE_OFF keyword. If you include multiple TRACE_ON keywords and omit the TRACE_OFF keyword, the software inserts TRACE_OFF immediately before each TRACE_ON keyword (after the first one). Therefore, you cannot nest TRACE_ON statements. You can use any number of TRACE_ON, TRACE_OFF pairs in your design file. The polarity of the signal names listed in the TRACE_ON statement determines how the resulting trace will appear: |
| --- | --- |
|  | r If the signal name in the TRACE_ON statement has the same polarity as the same signal name in the PIN or NODE statement, the trace will go high when the signal goes high. r If the signal name in the TRACE_ON statement does not have the same polarity as the same signal name in the PIN or NODE statement, the trace will go low when the signal goes high. The software generates a warning if it has to generate a TRACE_OFF keyword for you, but continues processing. |
| Example | SIMULATION TRACE_ON A1 A2 A3 OUT1 OUT2 SETF A1 A2 A3 CLOCKF CLK1 ... TRACE_OFF |
| See Also | r CHECK r CHECKQ r TRACE_ON |

| TRACE_ONXE "TRACE_ON"§ |  |
| --- | --- |
| Purpose | Turns on the simulation trace function. |
| Syntax | TRACE_ON |
| Where Used | SIMULATION segment |
| Remarks | The trace feature allows you to generate a separate simulation file that reports the simulated behavior of only those pins and nodes you specify in the TRACE_ON statement, and in the order you specify. You must include the TRACE_ON keyword in the SIMULATION segment before you include the TRACE_OFF keyword. You can use any number of TRACE_ON, TRACE_OFF pairs in your design file. If you include multiple TRACE_ON keywords and omit the TRACE_OFF keyword, the software inserts TRACE_OFF immediately before each TRACE_ON keyword (after the first one). Therefore, you cannot nest TRACE_ON statements. The software generates a warning if it has to generate a TRACE_OFF keyword for you, but continues processing. |
| Example | SIMULATION TRACE_ON A1 A2 A3 OUT1 OUT2 SETF A1 A2 A3 CLOCKF CLK1 ... TRACE_OFF |
| See Also | r CHECK r CHECKQ r TRACE_ON |

| .TRSTXE ".TRST"§ |  |
| --- | --- |
| Purpose | Defines the conditions under which an I/O pin's programmable three-state buffer is enabled for output. |
| Syntax | Pin_name.TRST = Boolean_expression |
| Where Used | EQUATIONS segment |
| Remarks | The Boolean expression that defines output enable must fit on one product term. You cannot complement the pin name on the left side of the equation. For example, /OUT1.TRST = A * /B is not a valid .TRST equation. ã Note: Multiple functional equations are ORed, then de Morgan's theorem is applied to produce a single product term. The Fitter generates an error if the resulting equation exceeds a single product term. Use VCC on the right side of the equation to enable the pin's output permanently. Use GND on the right side of the equation to disable the pin's output permanently. |
| Example | OUT2.TRST = A * /B OUT3.TRST = VCC OUT4.TRST = GND |
| See Also | r GND r VCC |

| VCCXE "VCC"§ |  |
| --- | --- |
| Purpose | Specifies an unconditional-high signal (logical 1) in a Boolean or state machine equation. |
| Syntax | Pin_or_node_name<.function> = VCC |
| Where Used | DECLARATION, EQUATIONS, and STATE segments |
| Remarks | Use the pin or node name, and assignment operator, and VCC to set the pin or node permanently to logical 1. Use the pin or node name, a functional suffix such as .TRST, an assignment operator, and VCC, to hold the corresponding function high. (In the case of the .TRST function, for example, use VCC to enable the three-state output buffer unconditionally.) |
| Example | OUT4 = VCC ;sets OUT4 to logical 1 OUT5.TRST = VCC ;enables output from OUT5 |
| See Also | r GND |

| WHILE-DOXE "WHILE-DO"§ |  |
| --- | --- |
| Purpose | Repeats a set of instructions as long as a specified condition exists. |
| Syntax | WHILE Condition DO BEGIN Action_statement(s) END |
| Where Used | SIMULATION segment |
| Remarks | The condition can be any Boolean expression. Valid assignment operators include =, <, >, <=, >=, and <>. Parentheses surrounding the condition expression are optional, but recommended. You can nest parentheses. You can nest WHILE-DO constructs within other WHILE-DO constructs or within IF-THEN-ELSE and FOR-TO-DO constructs. |

| Example | SIMULATION ... WHILE ( /IN1 * /IN2 ) DO BEGIN CLOCKF CLK1 END ... |
| --- | --- |
| See Also | r IF-THEN-ELSE (Simulation) r FOR-TO-DO |

If device does not support XOR in hardware, this is converted to sum-of-products logic.