Upgrading Sprint Expert to use C/99 software

Program a new 93C46 eeprom (it is easier to use a DIL version) with the C/99 code as shown below, turn off programmer, and remove module.
Cut off top of module with a sharp knife, carefully scrape out the filler to expose all the pins of the eeprom and replace module in programmer board to check that it still works correctly.  Remove the original SOIC eeprom from the module by carefully cutting its legs, and then attach six thin wires to the pads 2 – 7. (Pins 1 and 8 are unused).  Connect these wires to the pins of either a similar SOIC ic or preferably, a 8 – pin DIL socket.  Refer to manufacturer’s data sheet for details of the different pin connections.
Original  eeprom is 8-pin SOIC (Rotated version) type 93C46. (Catalyst 93C46J or equivalent) Any standard 5volt DIL 93C46 should work ok.
C/99 93C46 EEPROM Code (corrected)
00

42
D0
00
B2
88
9D
00
00

C0
00
8B
88
00
00
00
00
10

00
00
42
D0
00
B2
80
00

00
00
80
00
E0
00
00
00

20

00
00
E0
B2
00
00
00
00

00
00
00
00
00
00
00
00

30

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

40

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

50

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

60

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

70

00
00
00
00
00
00
00
00

00
00
00
00
00
00
5F
80

Original C/96 93C46 EEPROM Code
00

3C
C4
40
86
4A
ED
00
00

C0
00
70
3C
86
ED
18
00

10

00
00
00
00
00
00
80
00

00
00
80
00
E0
00
00
00

20

00
00
C3
8C
83
00
2D
00

00
00
00
00
00
00
00
00

30

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

40

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

50

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

60

00
00
00
00
00
00
00
00

00
00
00
00
00
00
00
00

70

00
00
00
00
00
00
00
00

00
00
00
00
00
00
1C
56

