1 - ADDENDUM ON NEW FEATURES OF ST6 SIMULATOR

1.1 - Modifications in "ST6 FAMILY SOFTWARE DEV. TOOLS"

List and contents of the modifications in the user documentation of SIMST6 in the book "st6 family

software tools - user manual"

release of June 1991.

Page 5/97 : - Suppress the Note.

- Replace by : The simulator handles the common peripherals of the ST6 family.

It allows to define events on simulated pins and it displays execution results.

Page 6/97 : in "Meaning of the flags" add :

- -ST6XXX :specifies the type of device to be used.

Page 7/97 : in Invocation examples add :

- SIMST6 -ST6210      The simulator loads the

configuration file of the ST6210 in order to simulate its pins.

If this option is not set, then a configuration menu runs which asks what type of device to be used.

Page 40/97 : in 2.8.1

Purpose : Display the current status of the simulated pins.

Dialog box :

Hit <Space>

Type Pin group name                   Time begin time to display

The Type field is used to select one of the four possible groups available (PORT A, PORT

B,PORT C,OTHERS), while the Time field is used to select the display time. When the cursor is

positioned on this field, each press on <space> make it increment of 1 and so the graphic display

is update shifted of 1 microsecond. A press on <Ctrl r> make it decrement of 1 with the

correspondent display update. It's possible too to enter a value in this field.

Display : This command displays on graphic form the status of the pins and the

form of the signals on the different pins.

The screen displays the status of the ports :

-       field A "Type" displays the name of the pin group displayed.

each press on <space bar> or <CTRL r> changes the group.

-       field B "Time" displays the time at the first position on the graph.

-       field C displays the time at the beginning.

-       field D displays the time at the end of the display.

-       field E displays the current status of the pin at the start time.

Page 41/97 : in 2.8.2

2.8.2 - WATCHDOG STATUS

Purpose : Set the status of the watchdog

Dialog box :

Hit <Space>

WATCHDOG

Mode

The mode field is used to select the running mode of the watchdog, either  HARDWARE or SOFTWARE.

Page 43/97 : between NEXT and PM, insert :

PIN    [<name>] -d [<time>]                  -> displays pin values

<name>  -i <0|1> [<time>]            -> defines a value on pin

<name>  -c <period> [<time>]         -> defines a cyclic value

<name>  -a <value> [<time>]          -> defines an analogic value

[<name>] -s [-a] <filename>           -> save pin contents in file

[<name>] -g <filename>                -> get pin content from file

Page 75/97 : shift the 3.20 in 3.21 and insert

3.20 PIN : define/read pin values.

Syntax :

PIN [<name>] -d [<time>] : displays the value of a pin.

<name>                 requested pin, if omitted, display values for all

pins

<time>                 if requested, display the value at the requested

instant ; if not requested, display all known

values

PIN <name> -i <0|1> [<time>] : enter value of an input pin

<name>                 requested pin

<0|1>                  value of the pin

<time>                 if requested, value is valid at the requested

instant ; if not, value is valid immediately

PIN <name> -c <period> [<time>] : enter a cyclic evolution of the

pin

<name>                 requested pin

<period>               period for each level, values are 0 and 1

<time>                 if requested, periodic value is valid at the

requested instant ; if not, periodic value is

valid immediately

PIN <name> -a <value> [<time>] : enter an analog value on the pin

<name>                 requested pin

<value>                decimal analog value on the pin

<time>                 if requested, analog value is valid at the

requested instant ; if not, analog value is valid

immediately

PIN [<name>] -s [-a] <file> : save values of a pin in a file

<name>                 name of the pin ; if omitted, save values for all

pins

-a                     if not mentioned, flush the file ; if mentioned,

append at the end of the file

<file>                 name of the file

PIN [<name>] -g <file> : get values of a pin from a file

<name>                 name of the pin ; if omitted, get values for all

pins found in the file

<file>                 name of the file ; recorded values are flushed

Note that it is similar than using command "PIN [<name>] -d" with

the log feature on, and the "use" command from the same file.

examples :

x: PIN TIMER -d

PIN TIMER -U

x: rem no value defined on TIMER pin

x: PIN TIMER -C 20

x: rem defines a cyclic value on the pin timer with

x: rem a period of 40 fs because it is 20 fs for each level

x: PIN TIMER -d

PIN TIMER -C 20

x: PIN PA0 -i 0 10

x: rem defines PA0 in input for a level 0 10 fs after beginning

x: PIN PA0 -i 1 20

x: rem this value should happen 20 fs after the previous change

x: rem therefore 30 fseconds after the time 0

x: PIN PA0 -i 0 40

x: PIN PA0 -i 1 70

x: PIN PA0 -i 0 80

x: PIN PA0 -i 1 100

x: rem user program switches PA1 each time PA0 becomes 1

x: GO

x: rem program stops on a breakpoint

x: PIN PA0 -d

PIN PA0 -I 0 10

PIN PA0 -I 1 20

PIN PA0 -I 0 40

PIN PA0 -I 1 70

PIN PA0 -I 0 B0

PIN PA0 -I 1 100

x: PIN PA1 -d

PIN PA1 -O 0 0

PIN PA1 -O 1 20

PIN PA1 -O 0 70

PIN PA1 -O 1 100

x: pin PA2 -a 34 10

x: rem means the decimal value 34 is defined 10 fs after time 0

x: rem on pin PA2.

x: pin -g cyclic.in

reading from file CYCLIC.IN

reading complete

x: rem get values from the file cyclic.in

x: pin pa0 -s pa0val.in

saving complete for file PA0VAL.IN

x: rem save values of pin pa0 in file pa0val.in

1.2 - About I/O simulation

1.2.1 - Simulated ST6 peripheral devices

Note :

x: is the prompt of the line mode.

>  is the prompt of assembler.

rem means comment.

- Ports

The ports of the ST6 can now be simulated, either in input or

output and over and above in analogic mode. The simulator is

configurate like the ST6 device you asked to use. So the number

of ports and pin available is the same as the real device.

Input mode :

In order to use a port in input mode, you should firstly, in

line mode define thanks to the PIN command the differents value

taken by the port during the time.

Example :

x: pin PA0 -i 0 0

x: rem set the port at low level on begining

x: pin pa0 -i 1 20

x: rem set the bit 0 of port A at high level at 20 fs.

x: pin pa0 -i 0 10

x: rem set the bit 0 of port A low 10 fs later.

Output mode :

In order to use a port (or some pins of a port) in output mode,

you should write a value into the port data register. If the Port

Data Direction Register and the Port Data Option Register are

well configured. You should find the value you wrote on the port

displayed either in graphic or in line mode, it's a way to check

the timings and to ensure the correct working order of the

application simulate.

Example :

> ldi DDRA,FF

> ldi ORA,00

> ldi DRA,A5

This sample example show how to output a value on a port by

writing a value to the Data Register of the Port.

Analogic mode :

To use a bit port in analogic mode, the port should be

configured in the correct mode for analogic input. The Analog to

Digital Converter Control Register must be configured too for a

conversion, and so the result of the conversion is put in the A/D

Converter Data Register.

Example :

x: pin pa0 -a 45 20

x: rem the decimal value 45 is put on the bit 0 of port A.

- Timer

The timer can be used in the same way as the real one, the

timer pin can be configured in input or output. The timer can

also be drived by the internal clock as a normal mode.

- Watchdog

The watchdog functioning mode can be select by the watchdog

menu in graphic mode either in the software (default) mode or in

hardware mode.

- Analog to Digital Converter

The analogic to digital converter can be drived by the port

pins well configured and the result of a conversion can be read

in the A/D Converter Data Register.

WARNING : The conversion time of the real ST6 is not take in

account in simulation, so ensure to got it in your mind when you

use it.

- Interruptions

The whole peripherals can if they are well configured activate

the interrupt associated to the device.

The reset pin is also available and is able to reset the ST6

device, only in a high to low transition.

WARNING : The reset is not set while the reset pin is low as the

real ST6.

Don't forget to begin all of yours applications using the

interrupts by the instruction RETI, because you should not be

able to use it in  the case arises.