MOD 3/94

Changes to the TOP44PLC

These changes are required for the INTEL iFX740 device.

The Intel iFX740 has one additional Vcc pin which is not connected on the TOP44PLC. A fast clock is not connected, too. To read, program or test these devices some additional connections are required.

A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

The changes are :

| - | Remove the TOP44PLC from the programmer |
| --- | --- |
| - | Turn the TOP44PLC so that the 44pin socket is on the bottom |
| - | Make the 3 connections (A , B and C) shown below of this MOD by soldering an isolated wire on the marked pads |
| - | Place the TOP44PLC onto the programmer |

Check your work by

| - | Running the SPRINT selftest utility |
| --- | --- |
| - | Running the PAL.EXE and select the device INTEL iFX740 |