MOD 6/95

Changes on SPRINT EXPERT related with TOP-QT

When you receive the TOP-QT you must perform a modification first in the OPTIMA / EXPERT programmer. This is due to the very high required VDD voltage of the QUIXTAL. The OPTIMA / EXPERT programmers are supplied by +12 V and +5 V. We have included a current limiter in the 12 V power supply that should prevent the programmer from any serious damage if a short occurs. This current limiter has a voltage drop of about 2 V and as an effect the maximum VCC is limited. This limiter can be shorted without any concern.

A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

NOTE : You can leave the modification in the programmers without it effecting  programming of all other devices.

a) SPRINT EXPERT

Please remove the EXPERT PC-card out of your PC. You will find two ICs LM317  which are placed at position IE1 and ID1. Please short all 3 pins of the LM317 (see schematic) at the position IE1. Reassemble the programmer, install the TOP-QT and connetc it to your PC. Then start the selftest software. Check the calibration (test 2 and 3). If you encounter any problems please contact Data I/O.