Subject:	National SC14408 microcontroller programming with SPRINT

Date:		12/10/2001

Author:	SK

Pages:	1

### SC14408 microcontroller:

### Flash Programming issues

The User's RAM (in the programmer) does not correspond directly to the SC14408 memory map, because the FLASH modules overlap the Information blocks. This file should assist you in creating your datafile accordingly.

Data in Programmer's User RAM is partitioned as follows:

| User's RAM | Device memory | Description |
| --- | --- | --- |
| 0 0000h – 0 ABFFh | 0 0000h – 0 ABFFh | 43 kbyte main array |
| 1 0000h – 6 53FFh | 1 0000h – 6 53FFh | 341 kybte main array |
| 7 0000h – 7 00FFh | 0 8000h – 0 80FFh | INFO Block1 (256bytes) |
| 7 0100h – 7 01FFh | 2 0000h – 2 00FFh | INFO Block2 (256bytes) |
| 7 0200h – 7 02FFh | 4 0000h – 4 00FFh | INFO Block3 (256bytes) |

A part of INFO block 1 determines, how the SC14408 bootprogram behaves after RESET . The remaining part of this block and INFO blocks 2 and 3 can be freely used for data storage.

##### INFO Block 1

| Address in User’s RAM | PARAMETER | VALUE |
| --- | --- | --- |
| 7 0000h – 7 0001h | WPROT | Program FLASH is write protected if: 70000h = 77h (‘w’), 70001h = 57h (‘W’) |
| 7 0002h – 7 0003h | FPROG | Program FLASH is programmed if: 70002h = 70h (‘p’), 70003h = 50h (‘P’) |
| 7 0004h – 7 0005h | OFFSET | 16 bit offset to 8 words in on-chip FLASH. If FFFFh no read protection. |
| 7 0006h – 7 0015h | INFOCODE[0-7] | If these 8 words are equal to 8 on-chip words starting at OFFSET, then no read protection |