MOD 3/95

Changes on the TOP44PLCC adapter

This modification is required for the AMD MACH111 device.

The AMD MACH 111 needs one additional fast clock which is not connected on the TOP44PLC. To read, program or test these devices one additional connection is required.

A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

The changes are :

| - | Remove the TOP44PLC from the programmer |
| --- | --- |
| - | Turn the TOP44PLC so that the 44pin socket is on the bottom |
| - | Make the connection (A) shown below of this MOD by soldering an isolated wire on the marked pads |
| - | Place the TOP44PLC onto the programmer |

Check your work by

| - | Running the SPRINT selftest utility |
| --- | --- |
| - | Running the PAL.EXE and select the device AMD MACH111 |