Subject:	Freescale MC9S12X microcontroller programming with SPRINT

Date:		11/15/2007

Author:	SK

Pages:	3

### MC9S12X (S12X) microcontroller:

### Flash Programming issues

Since S12X microcontroller can have up to 4 MByte flash memory, but the native address range is restricted to 64k, the modules will be mapped through an address window of 16 kByte size. The additional address bits are located in the PPAGE register.

The allowed PPAGE register values are $00..$FF. The corresponding linear flash address space is $400000-$7FFFFF.

The SPRINT programmer and SW use this linear address space to address the up to 4 MByte Flash of the S12X microcontroller.  For mapping details refer to Table 2.

##### Flash Protection/Option Fields

Flash block 0 also holds a field of 16-bytes containing Security, Protection and "backdoor" comparison bytes.

Table 1: Flash 0 Protection/Security Field

| Programmer Address | MCU Address | Description |
| --- | --- | --- |
| $7FFF00-$7FFF07 | $FF00-$FF07 | Backdoor comparison key |
| $7FFF08-$7FFF09 | $FF08-$FF09 | Reserved (set to FF) |
| $7FFF0A | $FF0A | ###### Protection byte for Flash block 3 |
| $7FFF0B | $FF0B | Protection byte for Flash block 2 |
| $7FFF0C | $FF0C | Protection byte for Flash block 1 |
| $7FFF0D | $FF0D | Protection byte for Flash block 0 |
| $7FFF0E | $FF0E | COP Configuration (bits 3..0) |
| $7FFF0F | $FF0F | Security Byte SEC[7:0] |

Security Byte:

The bits 1 and 0 of this byte defines the secure state of the microcontroller. Only the combination SEC[1:0] = 10 define the UNSECURED state, all other states will prevent any access to the S12X memory.

NOTE:       Microcontroler are normally factory preprogrammed with the UNSECURED state. The error “ramload operation failed” can also mean, you have a microcontroller in the SECURED state. In this case please try to reprogram the device with erasing ON.

Table 2: Memory Map for MC9S12X C

|  |  | Programmer Address Range | MCU Address Range |  |
| --- | --- | --- | --- | --- |
|  | # EPAGE |  |  |  |
| 1k: | $00 | 100000 – 1003FF | 800 – BFF |  EEPROM  |
| 1k: | $01 | 100400 – 1007FF | 800 – BFF |  |
| 1k: | $FC | 13F000 – 13F3FF | 800 – BFF |  |
| 1k: | $FD | 13F400 – 13F7FF | 800 – BFF |  |
| 1k: | $FE | 13F800 – 13FBFF | 800 – BFF |  |
| 1k: | $FF | 13FC00 – 13FFFF | C00 – FFF |  |
|  | # PPAGE |  |  |  |
| 16k: | $00 | 400000 - 403FFF | 8000 – BFFF |  FLASH  |
| 16k: | $01 | 404000 - 407FFF | 8000 – BFFF |  |
| 16k: | $FC | 7F0000 – 7F3FFF | 8000 – BFFF |  |
| 16k: | $FD | 7F4000 – 7F7FFF | 4000 – 7FFF |  |
| 16k: | $FE | 7F8000 – 7FBFFF | 8000 – BFFF |  |
| 16k: | $FF | 7FC000 – 7FFFFF | C000 – FFFF |  |

For HCS12 and S12X family compatibility please refer to the application note AN2615 from Freescale.

MASK SETS

The flash programming procedure is usually the same for all mask sets. However, feature sets, functionality can vary on the life cycle of the product. For that it can be useful if the programmer checks also the mask code beside the part ID.

Following mask sets are supported and with ID Check enabled it can be verified (status: C/2007).

If your mask set is not listed, you can disable the ID check.

Please send us a message (ccs-europe@data-io.de) including the device name, mask set and if possible the mask code.

Table 3 Supported mask sets

| Device | Mask set | Menu selection | part ID, mask code (HEX) |
| --- | --- | --- | --- |
| MC9S12XA256 | 0L15Y | MC9S12XA256 .. 0L | C410 |
| MC9S12XB128 | 2M42E | MC9S12XB128 .. 2M | C102 |
| MC9S12XB256 | 2M42E | MC9S12XB256 .. 2M | C402 |
| MC9S12XDx128 | 0M42E | MC9S12XDx128 .. 0M | C100 |
| MC9S12XDx128 | 1M42E | MC9S12XDx128 .. 1M | C101 |
| MC9S12XDx256 | 0L15Y | MC9S12XDx256 .. 0 | C410 |
| MC9S12XDT384 | 1L15Y | MC9S12XDT384 .. 1L | C411 |
| MC9S12XDx512 | 0L15Y | MC9S12XDx512 .. 0 | C410 |
| MC9S12XDx512 | 1L15Y | MC9S12XDx512 .. 1 | C411 |

MARGIN VERIFY OF THE FLASH MEMORY

Instead of standard verify the programmer performs a 16 bit signature check in Margin 0 (high sensitive for program margin ) and Margin 1 (high sensitive for erase margin) verify mode at the maximum operation frequency.