Changes on TOP44PLC,

Dear SPRINT user,

When you receive the screen message "Please ask your distributor for MODxxx..., a hardware change is necessary for the device you are attempting to program. A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

# Required for device TI TMS 77C82

The TMS 77C82 has two ground pins. These pins are not connected on the TOP44PLC. To read, program or test this device additional connections are required.

The changes are :

| - | Remove the TOP44PLC from the programmer |
| --- | --- |
| - | Turn the TOP44PLC so that the 44pin socket is on the bottom and you see the solder side of the socket. |
| - | Make the 2 connections ( A to A and B to B ) shown below, by soldering an isolated wire on the marked pads |
| - | Place the TOP44PLC onto the programmer |

Check your work by

| - | Running the SPRINT selftest utility |
| --- | --- |
| - | Running the PROM.EXE and select the device TI TMS 77C82 |