# ECN 2/95

Modification for adapter S434/S435/S497/S498

Dear SPRINT user,

When you receive the screen message "Please ask your distributor for ECNxx/yy " a hardware change is necessary for the device you want to program.  A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.  The Microchip PIC 16C5x devices require that addition of 2 resistors to the used adapter. Please follow the instructions below to add them to the adapter.

1.) 	Add a 1K Ohm resistor between pin 27 and pin 26 of the 2*14 pin connector

2.)	Cut the track between pin 1 and the via connection in the middle 
		of the two connectors	and add a 100 Ohm resistor

Start the programming software (should be B/95 or newer) and select the device, the message should not longer appear on the screen.