Document Version: 2001.001

Dear SPRINT user,

when you get the screen message MOD..., a hardware change is necessary for that special device you want to program. The required change is easy to do, so if you want to save time and money you can do that yourself, but we can also rework your unit at our facility. If you choose to make this change yourself, please exercise appropriate care not to make any unintentional contacts. Should you encounter any difficulties, please contact us immediately.

Remove the TOP3PLC from your programmer and look to the following schematic to find the required solder points. There are two reserve relais which can beused for special functions as required here.

WARNING : It is not posible to have two different modifications in the same TOP, that are using the same signals (e.g. L1, L2 etc). Only modifications that are using different signals can be installed together in one TOP31PLC.

| MOD | Device | what to do |
| --- | --- | --- |
| 9/92 | TI TMS 87C110 TMS 87C510 | 1.) Connect solder pad L1 with pin 1 of the 32-pin programming socket, using a piece of isolated wire. 2.) Connect solder pad L2 with pad L3. |
| 1/93 | AMD 29F0X0 INTEL28F0X0 | 1.) Connect the kathode of diode D46 with pin 31 of the 32-pin programming socket, using a piece of isolated wire. |
| 3/93 | Lattice GAL16VP8 | 1.) Connect solder pad L1 with pin 5 of the 20-pin programming socket, using a piece of isolated wire. 2.) Connect solder pad L5 with pin 15 of the 20-pin programming socket, using a piece of isolated wire. 3.) Connect solder pad L2 with pad L4. 4.) Connect solder pad L6 with pad L7. |

Reinstall the TOP and select the device you want to program. The software can check if the MOD is performed and the screen message must not appear any more. For programming all the other devices it is not necessary to remove this MODs.