Modification for TOP48

Dear SPRINT user,

When you receive the screen message "Please ask your distributor for MODxxx..., a hardware change is nrequired for the device you are attempting to program. A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

This modification concerns devices that require fast pull-ups on a special pins or that have the supply-pins at non-standard locations. Look to the schematic on the page 2 of this document to locate the positions of the pins and the requested solder pads.

1)	Remove the TOP48DIP of your programmer.

2.)	Connect the requested solder pad to the requested pin by an isolated wire

Note :	The solder pads are splitt into two different parts, one circle and one square - please

use always the square part to connect the isolated wire with the requested pin.

3.)	Connect the TOP to the programmer and reselect the device

The message "Please ask your distributor...." should no longer appears on the screen.

| MOD | Device | Signal | Pin-number |
| --- | --- | --- | --- |
| 15/94 | MC68HC711D3 | GND : JP5 | 5 |
| 16/94 | Z86E02/04/07/08 | VCC : JP7 | 20 |
| 20/94 | MC68HC11A1/A8 | GND : JP5 | 23 |
| 21/94 | SDA 2506 | VCC : JP7 | 23 |
| 13/95 | Cypress PALCE16V8 | S7 S8 | 19 20 |
| 20/95 | AMD Am27S27 | VCC : JP7 | 35 |
| 1/96 | SGS M92256 | GND : JP5 | 23 |
| 5/96 | Philips ABT22V10A LVT22V10 | solder a 10KW resistor between JP7 and pin 33 and between JP8 and pin 34 |  |
| 12/96 | COP87L22/42 | VCC : JP7 | 20 |
| 13/96 | COP87L20/40/84 COP8SAx7-28p | VCC : JP7 | 16 |
| 14/96 | COP87L88 | GND : JP5 | 37 |
| 19/96 | PZx032 | GND : JP5 | 32 |
| 20/96 | MC68HC705J1A | VCC : JP7 | 23 |
| 4/97 | PZx064 | GND : JP5 | 44 |

| MOD | Device | Signal | Pin-number |
| --- | --- | --- | --- |
| 13/97 not with new ASICs | Atmel ATF1500/A | S7 or S8 | 6 |
| 14/97 | COP SAx7-16 | VCC : JP7 GND : JP5 | 22 27 |
| 16/97 | Xicor 88C75 | VCC :JP7 | 48 |
| 22/97 | Intel 87C42 (DIL) | CLOCK : JP2 | 7 |
| 18/98 | MC68HC805PV8 | CLOCK : JP2 | 30 |
| 19/98 | Motorola 68HC705CJ4 | JP1 or JP2 or JP3 or JP4 | 43 |
| 2/99 | Samsung KM29N040T | VCC : JP7 GND : JP5 | 44 5 |
| 4/99 | Atmel AT45dxxSOIC 28pin | GND : JP5 | 11 |
| 5/99 | Atmel AT45dxxTSOP 28 pin | VCC : JP7 GND : JP5 | 16 17 |
| 6/99 | Mitsubishi M37510EFFP 176 pin | VCC : JP7 | 39 |
| 7/99 | Xilinx 1704LVC | GND : JP5 GND : JP6 | 4 22 |
| 8/99 | AT45DB081/D081-TSOP(32pin) | VCC : JP7 GND : JP5 | 39 40 |
| 9/99 | M29W102BB –TSOP(40 pin) | VCC : JP7 GND : JP5 | 24 42 |
| 10/99 | AS29F080 –SO(44 pin) | VCC : JP7 GND : JP5 | 3 11 |
| 12/99 | TI MSP430P112 | VCC: JP7 | 16 |
| 13/99 | ST72T101xx | GND: JP5 CLK: JP2 | 37 12 |