Subject:	Motorola MC9S12 microcontroller programming with SPRINT

Date:		10/24/2008

Author:	SK

Pages:	4

### MC9S12 (STAR12) microcontroller:

### Flash Programming issues

Since STAR12 microcontroller can have up to 512 kByte flash memory, but the native address range is restricted to 64k, the modules will be mapped through an address window of 16 kByte size. The additional address bits are located in the PPAGE register.

Since the allowed PPAGE register values are $20..$3F (currently) and S-records use physical addresses, the corresponding linear address space is $80000-$FFFFF.

The SPRINT programmer and SW use this linear address space to address the up to 512 kByte Flash of the MC9S12 microcontroller.  For mapping details refer to Table 2.

##### Flash Protection/Option Fields

Flash block 0 also holds a field of 16-bytes containing Security, Protection and "backdoor" comparison bytes.

Table 1: Flash 0 Protection/Security Field

| Programmer Address | MCU Address | Description |
| --- | --- | --- |
| $FFF00-$FFF07 | $FF00-$FF07 | Backdoor comparison key |
| $FFF08-$FFF09 | $FF08-$FF09 | Reserved (set to FF) |
| $FFF0A | $FF0A | ###### Protection byte for Flash block 3 |
| $FFF0B | $FF0B | Protection byte for Flash block 2 |
| $FFF0C | $FF0C | Protection byte for Flash block 1 |
| $FFF0D | $FF0D | Protection byte for Flash block 0 |
| $FFF0E | $FF0E | Reserved (set to FF) |
| $FFF0F | $FF0F | Security Byte SEC[7:0] |

Security Byte:

The bits 1 and 0 of this byte defines the secure state of the microcontroller. Only the combination SEC[1:0] = 10 define the UNSECURED state, all other will prevent any access to the MC9S12 memory.

CAUTION ! Several mask sets (0K36N) can not be erased by the programmer, once
                   SECURED. Set the security state carefully. The error “ramload operation
                   failed” can also mean, you have a microcontroller in the SECURED state.

Table 2: Memory Map for MC9S12 C

|  |  | Programmer Address Range | MCU Address Range |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 00000 00800 00C00 00FFF | xx000 xx800 xxC00 xxFFF | - ¦ ¦ - | - ¦ ¦ ¦ - | - ¦ ¦ ¦ ¦ - |  |  |  | ## EEPROM |
|  | # PPAGE |  |  |  |  |  |  |  |  |  |
| 16k: | $20 | 80000 - 83FFF | 8000 - BFFF |  |  |  |  |  |  | ---------------------------------------------------------------- FLASH ---------------------------------------------------------------- |
| 16k: | $21 | 84000 - 87FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $22 | 88000 – 8BFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $23 | 8C000 - 8FFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $24 | 90000 - 93FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $25 | 94000 - 97FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $26 | 98000 – 9BFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $27 | 9C000 - 9FFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $28 | A0000 - A3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $29 | A4000 - A7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2A | A8000 - ABFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2B | AC000 - AFFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2C | B0000 – B3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2D | B4000 - B7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2E | B8000 - BBFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $2F | BC000 - BFFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $30 | C0000 - C3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $31 | C4000 - C7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $32 | C8000 - CBFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $33 | CC000 - CFFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $34 | D0000 - D3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $35 | D4000 - D7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $36 | D8000 - DBFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $37 | DC000 - DFFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $38 | E0000 - E3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $39 | E4000 - E7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $3A | E8000 - EBFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $3B | EC000 - EFFFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $3C | F0000 - F3FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $3D | F4000 - F7FFF | 8000 - BFFF |  |  |  |  |  |  |  |
| 16k: | $3E | F8000 - FBFFF | 8000 - BFFF or 4000 - 7FFF |  |  |  |  |  |  |  |
| 16k: | $3F | FC000 - FFFFF | 8000 - BFFF or C000 - FFFF |  |  |  |  |  |  |  |

MASK SETS

The flash programming procedure is usually the same for all mask sets. However, feature sets, functionality can vary on the life cycle of the product. For that it can be useful if the programmer checks also the mask code beside the part ID.

Following mask sets are supported and with ID Check enabled it can be verified (status: C/2008).

If your mask set is not listed, you can disable the ID check.

Please send us a message (ccs-europe@data-io.de) including the device name, mask set and if possible the mask code.

Table 3 Supported mask sets

| Device | Mask set | Menu selection | part ID, mask code (HEX) |
| --- | --- | --- | --- |
| MC9S12A128 | 3L40K | MC9S12Ax128 | 0113 |
| MC9S12B128 | 2L80R | MC9S12B128 | 2102 |
| MC9S12C32 | 1L45J | MC9S12C32 | 3300 |
| MC9S12C32 | 1M34C | MC9S12C32 FA1M | 3311 |
| MC9S12C32 | 2L45J | MC9S12C32 FA2L | 3302 |
| MC9S12C64 | 2L09S | MC9S12C64 | 3102 |
| MC9S12C128 | 2L09S | MC9S12C128 | 3102 |
| MC9S12DJ64 | 0L86D | MC9S12Dx64 | 0200 |
| MC9S12DJ64 | 1L86D | MC9S12Dx64 ..1 | 0201 |
| MC9S12DJ64 | 2L86D | MC9S12Dx64 ..2 | 0201 |
| MC9S12DJ64 | 3L86D | MC9S12Dx64 ..3 | 0203 |
| MC9S12DJ64 | 4L86D | MC9S12Dx64 ..4 | 0204 |
| MC9S12DB128 | 0L94R | MC9S12Dx128 .. 0 | 0110 |
| MC9S12DB128 | 3L40K | MC9S12Dx128 .. 3 | 0113 |
| MC9S12DB128 | 4L40K | MC9S12Dx128 .. 4 | 0114 |
| MC9S12DG128 | 1L59W | MC9S12Dx128 .. 5 | 0115 |
| MC9S12DG128 | 5L40K | MC9S12Dx128 .. 5 | 0115 |
| MC9S12DG128 | 2L94R | MC9S12Dx128 .. 5 | 0115 |
| MC9S12DG256 | 0L91N | MC9S12Dx256 .. 0 | 0030 |
| MC9S12DG256 | 0L01Y | MC9S12Dx256 ..0L0 | 0033 |
| MC9S12DG256 | 1K79X | MC9S12Dx256 ..1K7 | 0011 |
| MC9S12DG256 | 1L91N | MC9S12Dx256 .. 1 | 0031 |
| MC9S12DG256 | 2K79X | MC9S12Dx256 .. 2 | 0012 |
| MC9S12DP512 | 1L00M | MC9S12Dx512 | 0401 |
| MC9S12DP512 | 4L00M | MC9S12Dx512 4L | 0404 |
| MC9S12E64 | 2L15P | MC9S12Ex64FU 2L | 5102 |
| MC9S12E128 | 0M06C | MC9S12Ex128FU 0M | 5103 |
| MC9S12GC32 | 2L45J | MC9S12GC32 | 3302 |
| To be continued… | next page |  |  |
| MC9S12GC32 | 1M34C | MC9S12GC32 ..1M | 3311 |
| MC9S12KG128 | 5L74N | MC9S12Kx128 .. 5 | 7105 |
| MC9S12NE64 | 1L19S | MC9S12NE64 ..1L | 8201 |
| MC9S12Q128 | 2L09S | MC9S12Q128 | 3102 |

MARGIN VERIFY OF THE FLASH MEMORY

Additionally to the standard verify the programmer performs also a 16 bit checksum check in Margin 0 (high sensitive for program margin ) and Margin 1 (high sensitive for erase margin) verify mode at the maximum operation frequency.