# Modification 6/97 for S492

Dear SPRINT PLUS48 user,

When you receive the screen message "Please ask your distributor for MODxx/yy " a hardware change is necessary for the device you are attempting to program. Some newer Atmel devices require a capacitor (value 100nF) on the S492 adapter between the Vcc- and the GND-pin.

A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

The major concern is that the capacitor has to be soldered with one side directly/pretty close to Vcc. (see the following schematics for deatails)

S492 - bottom view

Note : This modification can not be detected by the Sprint software. The message "Please ask your distributor for MOD6/97" will not disappear in the Sprint software.  Data I/O recommends customers use at least software release A/97 or any newer version to program these devices.