Motorola MC68HC908
Special Register Programming

The Sprint SW used to program the EENVR (EEPROM Non-Volatile-Register) addresses with factory settings specific for each device. The EEPROM security feature was not available for customer. Also it was not possible to read out these register contents.

Since A6/2002 the Sprint SW programs/reads all programmable bytes and bits without any restrictions (refer also to Table 1).

This change brings a lot of benefits, but also requires, that the user makes sure the correct settings in his file/editor.

Especially older files saved with Sprint for the AZ60 can not be used without running the risk of securing parts of the EEPROM (since the special register contents are 00 in the file).

Please make sure the EENVR addresses contain the correct settings. The security option is also shown in the special device functions menu of the programmer.

|  | ### Flash | Flash Blank State | FLBPR | ### EEPROM | EEPROM Blank State | EENVR (Bit#4 =SEC bit1)) | EENVR Default State= factory setting | EEDIVHNVR (Bit#7 = SEC bit2)) | EEDIVLNVR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # AB32 | 0x8000-0xFDFF 0xFFD0–0xFFFF | 0xFF | 0xFF7E | 0x800 0x9FF | 0xFF | 0xFE1C | ## 0x10 | ## 0xFE10 | ## 0xFE11 |
| # AZ60 | 0x0450–0x04FF 0x0580–0x05FF 0x0E00–0x7FFF 0x8000–0xFDFF 0xFFFC–0xFFFF | 0x00 | 0xFF81 0xFF80 | 0x600 0x7FF 0x800 0x9FF | 0xFF | 0xFE18 0xFE1C | ## 0x70 0x70 | ## n.a. | ## n.a. |
| # AZ60A | 0x0450–0x04FF 0x0580–0x05FF 0x0E00–0x7FFF 0x8000–0xFDFF 0xFFFC–0xFFFF | 0xFF | 0xFF81 0xFF80 | 0x600 0x7FF 0x800 0x9FF | 0xFF | 0xFF7C 0xFE1C | 0xF0 0xF0 | 0xFF70 0xFE10 | 0xFF71 0xFE11 |

Table 1: Memory Map for MC68HC908 microcontroller

Notes:

- Bit #4 in the EENVR register is the EEPROM program/erase protection bit (EEPRTCT). If set to 0, it protects 16 bytes of the EEPROM (in the block 800-9FF the bytes 8F0-8FF, in the block 600-7FF the bytes 6F0-6FF). Due the datasheet, this feature is a write-once feature. Once the protection is enabled, it may not be disabled.

2)	Bit #7 in the EEDIVHNVR register is the EEPROM divider security bit (EEDIVSECD). This bit enables/disables the security feature of the EEDIV registers. When it is set to 0, the state of the registers EEDIVHNVR and EEDIVLNVR are protected from being erased/programmed. Due the datasheet, this feature is a write-once feature. Once this security is enabled, it may not be disabled.